Vivante Display Processing

DRM Driver API Reference

Document Revision 1.12

21 October 2026

This document is compatible with

Vivante display processing DRM drivers v2.x.x and v3.x.x

Legal Notices

COPYRIGHT INFORMATION

This document contains proprietary information of Vivante Corporation and VeriSilicon Holdings Co., Ltd. They reserve the right to make changes to any products herein at any time without notice and do not assume any responsibility or liability arising out of the application or use of any product described herein, except as expressly agreed to in writing by Vivante and/or VeriSilicon; nor does the purchase or use of a product from Vivante or VeriSilicon convey a license under any patent rights, copyrights, trademark rights, or any other of the intellectual property rights of Vivante, VeriSilicon or third parties.

DISCLOSURE/RE-DISTRIBUTION LIMITATIONS

The information contained herein may be directly re-distributed or may be adapted for re-distribution by SoC vendors who have licensed the related Vivante core IP, with the understanding that re-distribution is limited to those customers of the SoC vendor who have active and valid NDA or licenses applicable for the SoC which contains the related Vivante core IP. Otherwise, the information contained herein is not to be used by or disclosed to the third parties without the express written permission of an officer of Vivante Corporation or VeriSilicon Holdings Co., Ltd.

(VeriSilicon Distribution LEVEL B: CONFIDENTIAL – DISTRIBUTION RESTRICTED).

Upon request VeriSilicon provides this document in Word format for adaptive inclusion in documentation intended for the SoC vendor’s customers. VeriSilicon requests a pre-release copy of the adaptation for review and approval. In the adapted document please identify the technology and features described in this document using the Vivante trademark and include a reference to the copyright owner, for example: “This section describes the Vivante® display processing IP and contains copyright material disclosed with permission of VeriSilicon, who has authorized re-distribution of this material restricted to those NDA partners and licensees of [the SoC vendor] who are engineering [SoC vendor’s product] which includes the discussed Vivante IP.”

TRADEMARK ACKNOWLEDGMENT

VeriSilicon® and the VeriSilicon logo design are the trademarks or the registered trademarks of VeriSilicon Holdings Co., Ltd. Vivante® is a registered trademark of Vivante Corporation. All other brand and product names may be trademarks of their respective companies.

For our current distributors, sales offices, design resource centers, and product information, visit our web page located at http://www.verisilicon.com.

Vivante and VeriSilicon Proprietary. Copyright © 2026 by Vivante Corporation and VeriSilicon Holdings Co., Ltd. All rights reserved.

Table of Contents

Legal Notices	2

Table of Contents	3

List of Figures	6

List of Tables	7

1	Introduction	9

1.1	DRM Driver Overview	9

2	Standard Properties	10

3	Universal Enumerations and Structures	12

3.1	Universal Enumerations	12

3.1.1	drm_vs_csc_mode	12

3.1.2	drm_vs_csc_gamut	12

3.1.3	drm_vs_dth_frm_idx	12

3.1.4	drm_vs_dth_frm_mode	13

3.1.5	drm_vs_hist_pos	13

3.1.6	drm_vs_hist_bin_mode	14

3.1.7	drm_vs_data_extend_mode	14

3.1.8	drm_vs_hist_out_mode	15

3.1.9	drm_vs_ds_mode	15

3.1.10	drm_vs_hdr_mode	15

3.2	Universal Structures	16

3.2.1	drm_vs_rect	16

3.2.2	drm_vs_ccm	16

3.2.3	drm_vs_color	17

3.2.4	drm_vs_xstep_lut	17

3.2.5	drm_vs_scale_config	18

3.2.6	drm_vs_dither	19

3.2.7	drm_vs_hist_out	19

3.2.8	drm_vs_ds_config	20

3.2.9	drm_vs_ltm_luma_ave	20

4	Private Properties and IOCTLs	21

4.1	Pre-Processing Pipelines	21

4.1.1	DMA	21

4.1.2	UV Upsampling	25

4.1.3	YUV-to-RGB Conversion	25

4.1.4	Color Keying	26

4.1.5	Gamma	27

4.1.6	De-Gamma Correction	28

4.1.7	High Dynamic Range	29

4.1.8	Super Frame Line Padding	35

4.1.9	Data Extending	36

4.1.10	PVRIC Decompressor	37

4.1.11	DEC400 Decompressor	41

4.1.12	DEC400A Decompressor	44

4.1.13	Custom Format	45

4.1.14	Color Conversion Matrix After De-gamma	45

4.1.15	Cropping	46

4.2	Blending Pipelines	47

4.2.1	Blending	47

4.2.2	Non-Linear Gamut Mapping	51

4.2.3	OETF	52

4.2.4	Blend Dither	52

4.2.5	EOTF	55

4.2.6	Gamut Mapping	55

4.2.7	Background Color	56

4.2.8	Solid Color	56

4.3	Post-Processing Pipelines	57

4.3.1	Scaling	57

4.3.2	Local Tone Mapping	57

4.3.3	Global Tone Mapping	74

4.3.4	Sharpness	75

4.3.5	SR2000	85

4.3.6	Color Conversion Matrix Before De-gamma	88

4.3.7	De-Gamma	89

4.3.8	Brightness Compensation	89

4.3.9	Color Conversion Matrix After De-gamma	91

4.3.10	Color Conversion Matrix Extension	92

4.3.11	Color Calibration Path Mode	93

4.3.12	3D LUT	94

4.3.13	Data Conversion	96

4.3.14	Dither	98

4.3.15	Gamma	101

4.3.16	Blur Compensation	103

4.3.17	Rounded Corner Display	105

4.3.18	On Pixel Sum	107

4.3.19	Burn-in Compensation	110

4.3.20	Letter Box Appendant	114

4.3.21	Sub-Pixel Rendering	115

4.3.22	Histogram/RGB Histogram V2	116

4.3.23	RGB-to-YUV Conversion	120

4.3.24	UV Downsampling	121

4.3.25	VRR Display Mode	121

4.3.26	VRR Compensation	122

4.3.27	Panel Output Cropping	124

4.3.28	Panel Output Splitting	125

4.3.29	RAM-less	126

4.3.30	OFIFO Splicing	126

4.3.31	Free Synchronization	127

4.3.32	DP Synchronization	129

4.3.33	PVRIC Decompressor	130

4.4	Write-Back Paths	133

4.4.1	Write-Back Point	133

4.4.2	Write-Back Splitter	134

4.4.3	Write-Back Frame Difference	135

4.4.4	Write-Back Scaler	137

4.4.5	Write-Back Dither	137

4.4.6	Write-Back RGB-to-YUV Conversion	138

4.4.7	Write-Back Rotation/Flip	139

4.4.8	Write-Back Cropping	139

4.4.9	UV Downsampling	140

4.4.10	Write-back Stripe	141

4.5	Miscellaneous	142

4.5.1	Side-by-Side Buffer	142

4.5.2	Display Stream Compression	143

4.5.3	Display Frame Done Query	146

4.5.4	CRC	146

4.5.5	Test Pattern	152

4.5.6	QoS Management	155

4.5.7	Palette	158

4.5.8	Capability Query	159

4.5.9	Reset	161

4.5.10	Write-Back Status Query	162

4.5.11	Blending Write-Back Information Query	164

A Document Revision History	166

B Conventions	183

Glossary	184

List of Figures

Figure 1. Display Processing DRM Driver Architecture	9

List of Tables

Table 1. Standard Plane Properties	10

Table 2. Standard CRTC Properties	10

Table 3. drm_vs_csc_mode Values	12

Table 4. drm_vs_csc_gamut Values	12

Table 5. drm_vs_dth_frm_idx Values	13

Table 6. drm_vs_dth_frm_mode Values	13

Table 7. drm_vs_hist_pos Values	13

Table 8. drm_vs_hist_bin_mode Values	14

Table 9. drm_vs_data_extend_mode Values	14

Table 10. drm_vs_hist_out_mode Values	15

Table 11. drm_vs_ds_mode Values	15

Table 12.drm_vs_filter_type Values	19

Table 13. drm_vs_dma_mode Values	22

Table 14. drm_vs_line_padding_mode Values	35

Table 15. drm_vs_blend_mode Values	47

Table 16. drm_vs_alpha_mode Values	51

Table 17. drm_vs_galpha_mode Values	51

Table 18. drm_vs_ltm_luma_mode Values	60

Table 19. drm_vs_sharpness_ink_mode Values	72

Table 20. drm_vs_brightness_cal_mode Values	87

Table 21. drm_vs_color_calibration_path_mode Values	89

Table 22. drm_vs_data_trunc_mode Values	93

Table 23. drm_vs_mask_blend_type Values	102

Table 24. drm_vs_spr_out_mode Values	112

Table 25. drm_vs_free_sync_type Values	124

Table 26. drm_vs_wb_point Values	129

Table 27. drm_vs_frm_diff_mode Values	131

Table 28. drm_vs_frm_diff_hit_reset_mode Values	132

Table 29. drm_vs_sbs_mode Values	138

Table 30. DC9x00 CRTC CRC Parameters	143

Table 31. DC9x00 CRTC CRC Position Values	144

Table 32. DC8x00 CRTC CRC Parameters	144

Table 33. DC9x00 Plane CRC Parameters	146

Table 34. DC8x00 Plane CRC Parameters	146

Table 35. position Values	149

Table 36. mode Values	149

Table 37. Plane QoS Level Parameters	152

Table 38. CRTC QoS Level Parameters	153

Table 39. Write-Back Connector QoS Level Parameters	154

Table 40. palette_lut.txt Example	154

Table 41. drm_vs_feature_cap_type Values	155

Table 42. drm_vs_reset_mode Values	158

Introduction

This document provides reference on the Direct Rendering Manager (DRM) properties, including both the Linux standard properties and Vivante private properties. It also provides instructions on how to use the private IOCTLs.

DRM Driver Overview

Vivante Direct Rendering Manager (DRM) driver resides in the Linux kernel space to drive VeriSilicon display processing (DPU) IP. It is integrated into the kernel component, Direct Rendering Manager (DRM), to provide Kernel Mode Setting (KMS) and Graphics Execution Manger (GEM) functions. Both KMS and GEM serve to enable the display processing IP to display content on screen.

Figure 1 shows the architecture and the components of the DRM driver for display processing IPs.

Figure 1. Display Processing DRM Driver Architecture

Standard Properties

The following tables list the supported standard properties. For property details, see the Linux GPU Driver Developer's Guide.

Table 1. Standard Plane Properties

| Property | Type |
| --- | --- |
| type | Enumeration |
| FB_ID | Object |
| IN_FENCE_FD | Signed range |
| CRTC_ID | Object |
| CRTC_X | Signed range |
| CRTC_Y | Signed range |
| CRTC_W | Signed range |
| CRTC_H | Signed range |
| SRC_X | Signed range |
| SRC_Y | Signed range |
| SRC_W | Signed range |
| SRC_H | Signed range |
| IN_FORMATS | Blob |
| COLOR_ENCODING | Enumeration |
| COLOR_RANGE | Enumeration |
| rotation | Bitmask |
| zpos | Range |
| pixel blend mode | Enumeration |
| alpha | Range |

Table 2. Standard CRTC Properties

| Property | Type |
| --- | --- |
| ACTIVE | Range (Boolean) |
| MODE_ID | Blob |
| OUT_FENCE_PTR | Range |
| VRR_ENABLED | Range (Boolean) |
| GAMMA_LUT | Blob |
| GAMMA_LUT_SIZE | Range [0, UINT32_MAX] Note: UINT32_MAX indicates the maximum value of the UInt32 data type. |
| CTM | Blob |

Universal Enumerations and Structures

This section describes the enumerations and structures used throughout the APIs.

Universal Enumerations

drm_vs_csc_mode

Specifies the color space conversion (CSC) mode. Set this member to a value listed in the following table.

Table 3. drm_vs_csc_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_CSC_CM_USR | 0 | Conversion using user-defined coefficients |
| VS_CSC_CM_L2L | 1 | Limited range to limited range conversion |
| VS_CSC_CM_L2F | 2 | Limited range to full range conversion |
| VS_CSC_CM_F2L | 3 | Full range to limited range conversion |
| VS_CSC_CM_F2F | 4 | Full range to full range conversion |

drm_vs_csc_gamut

Specifies the color gamut. Set this member to a value listed in the following table.

Table 4. drm_vs_csc_gamut Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_CSC_CG_601 | 0 | BT.601 |
| VS_CSC_CG_709 | 1 | BT.709 |
| VS_CSC_CG_2020 | 2 | BT.2020 |
| VS_CSC_CG_P3 | 3 | DCI-P3 |
| VS_CSC_CG_ sRGB | 4 | SRGB |

drm_vs_dth_frm_idx

Specifies the frame index type. Set this member to a value listed in the following table.

Table 5. drm_vs_dth_frm_idx Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_DTH_FRM_IDX_NONE | 0 | Disables the frame index. |
| VS_DTH_FRM_IDX_SW | 1 | Uses the frame index configured by the software. |
| VS_DTH_FRM_IDX_HW | 2 | Uses the frame index provided by the hardware. |

drm_vs_dth_frm_mode

Specifies the frame control mode for dithering. Set this member to a value listed in the following table.

Table 6. drm_vs_dth_frm_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_DTH_FRM_4 | 4 | The 4-frame control mode. For DC9400, only this value is supported. |
| VS_DTH_FRM_6 | 6 | The 6-frame control mode. |
| VS_DTH_FRM_8 | 8 | The 8-frame control mode. |
| VS_DTH_FRM_10 | 10 | The 10-frame control mode. |
| VS_DTH_FRM_16 | 16 | The 16-frame control mode. |

drm_vs_hist_pos

Specifies the histogram input position. Set this member to a value listed in the following table.

Table 7. drm_vs_hist_pos Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_HIST_POS_SCALE | 0 | Before the scaler. This value is valid only if the scaler module is supported. |
| VS_HIST_POS_RCD | 1 | Before the RCD module. This value is valid only if the RCD module is supported. |
| VS_HIST_POS_DISPLAY_OUT | 2 | After the DISPLAY module. This value is valid only if the DISPLAY module is supported. |
| VS_HIST_POS_COUNT | 3 | The number of histogram positions. |

drm_vs_hist_bin_mode

Specifies the bin calculation mode. Set this member to a value listed in the following table.

Table 8. drm_vs_hist_bin_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_DATA_BIN_MAX | 0 | Derives from the maximum of the red, green, and blue values. |
| VS_DATA_BIN_WEIGHT | 1 | Derives from the weighted sum of the red, green, and blue values by using the following formula: (R×REG_HIST_BIN_COEF_R +G×REG_HIST_BIN_COEF_G +B×REG_HIST_BIN_COEF_B +512) >> 10 |
| VS_HIST_BIN_RVALUE | 2 | Derives from the red values. |
| VS_HIST_BIN_GVALUE | 3 | Derives from the green values. |
| VS_HIST_BIN_BVALUE | 4 | Derives from the blue values. |
| VS_HIST_BIN_COUNT | 5 | The number of histogram bin modes. |

drm_vs_data_extend_mode

Specifies the bit-depth extension mode for ARGB or YUV color components, excluding the extension mode for the 2-bit alpha channel of the A2R10G10B10 format.

Set this member to a value listed in the following table.

Table 9. drm_vs_data_extend_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_DATA_EXT_STD | 0 | Fills the extended bits with 0. |
| VS_DATA_EXT_MSB | 1 | Fills the extended bits with the highest bits of the pixel component. |
| VS_DATA_EXT_RANDOM | 2 | Fills the extended bits with random values. This value is supported only for certain hardware versions. |

drm_vs_hist_out_mode

Specifies the histogram result output mode. Set this member to a value listed in the following table.

Table 10. drm_vs_hist_out_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_HIST_BY_APB | 0 | The histogram result will be stored in the histogram result registers. |
| VS_HIST_BY_WDMA | 1 | The histgogram result will be written to the specified buffer. |
| VS_HIST_OUT_COUNT | 2 | The number of histogram output modes. |

drm_vs_ds_mode

Specifies the downsampling mode. Set this member to a value listed in the following table.

Table 11. drm_vs_ds_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_DS_DROP | 0 | Drops every other UV value. |
| VS_DS_AVERAGE | 1 | Uses linear interpolation for downsampling. |
| VS_DS_FILTER | 2 | Uses filter for downsampling. |

drm_vs_hdr_mode

Specifies the HDR mode. Set this member to a value listed in the following table.

Table 12. drm_vs_hdr_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_DC_HDR_SDR | 0 | SDR mode |
| VS_DC_HDR_PQ | 1 | PQ mode |
| VS_DC_HDR_HLG | 2 | HLG mode |
| VS_DC_HDR_HDR10 | 3 | HDR10 mode |
| VS_DC_HDR_HDR10plus | 4 | HDR10+ mode |
| VS_DC_HDR_HDR_vivd | 5 | HDR Vivid mode |
| VS_DC_HDR_SDR_IN_HDR | 6 | SDR-to-HDR mode |

Universal Structures

drm_vs_rect

Definition

struct drm_vs_rect {

__u16		x;

__u16		y;

__u16		w;

__u16		h;

};

Members

- x: the X-coordinate of the rectangle's top-left corner.

- y: the Y-coordinate of the rectangle's top-left corner.

- w: the width of the rectangle.

- h: the height of the rectangle.

drm_vs_ccm

Definition

struct drm_vs_ccm {

__s32		coef[9];

__s32		offset[3];

};

Members

- coef: coefficients 00 to 22, in the S2.11 format (1 signed bit, 1 integer bit, and 11 fractional bits).

- offset: offsets 0 to 2, in the S11 format (bit 10 is a signed bit and bits 9:0 are integer bits).

drm_vs_color

Definition

struct drm_vs_color {

__u32		a;

__u32		r;

__u32		g;

__u32		b;

};

Members

- a: the alpha value.

- r: the red value.

- g: the green value.

- b: the blue value.

drm_vs_xstep_lut

Definition

#define VS_MAX_LUT_SEG_CNT		10

#define VS_MAX_LUT_ENTRY_CNT		129

struct drm_vs_xstep_lut {

__u32		seg_cnt;

__u32		seg_point[VS_MAX_LUT_SEG_CNT - 1];

__u32		seg_step[VS_MAX_LUT_SEG_CNT];

__u32		entry_cnt;

__u32		data[VS_MAX_LUT_ENTRY_CNT];

};

Members

- seg_cnt: the number of segments.

- seg_point: the array of segment points except the first and last point.

The first point is fixed to 0 and the last point is fixed at the maximum, which is not configurable.

The difference between two neighboring segment points (seg_point[N+1] – seg_point[N]) must be a power of 2.

- seg_step: the X increment per segment, which must be a power of 2.

- entry_cnt: the number of entries.

The maximum value is VS_MAX_LUT_ENTRY_CNT.

- data: the per-entry LUT value array.

drm_vs_scale_config

Definition

#define VS_SCALE_HORI_COEF_NUM 77

#define VS_SCALE_VERT_COEF_NUM 77

struct drm_vs_scale_config {

__u16				src_w;

__u16				src_h;

__u16				dst_w;

__u16				dst_h;

__u64				factor_x;

__u64				factor_y;

__u32				initial_offsetx;

__u32				initial_offsety;

__u32				coef_h[VS_SCALE_HORI_COEF_NUM];

__u32				coef_v[VS_SCALE_VERT_COEF_NUM];

enum drm_vs_filter_type	filter;

};

Members

- src_w: the image width before scaling.

- src_h: the image height before scaling.

- dst_w: the image width after scaling.

- dst_h: the image height after scaling.

- factor_x: the scaling factor in horizontal direction.

- factor_y: the scaling factor in vertical direction.

- initial_offsetx: the X offset of the center pixel of the horizontal filter.

- initial_offsety: the Y offset of the center pixel of the vertical filter.

- coef_h: the scaling coefficients in horizontal direction.

- coef_v: the scaling coefficients in vertical direction.

- filter: the scaling filter type. Set this member to a value listed in the following table.

Note: DC9000-SR series only support type VS_H9_V5.

Table 12.drm_vs_filter_type Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_H9_V5 | 0 | Horizontal 9-tap filter.Vertical 5-tap filter. |
| VS_H5_V3 | 1 | Horizontal 5-tap filter.Vertical 3-tap filter. |
| VS_H3_V3 | 2 | Horizontal 3-tap filter.Vertical 3-tap filter. |
| VS_H8_V4 | 3 | Horizontal 8-tap filter.Vertical 4-tap filter. |

drm_vs_dither

Definition

struct drm_vs_dither {

enum drm_vs_dth_frm_idx		index_type;

__u8					sw_index;

__u32					table_low[3];

__u32					table_high[3];

enum drm_vs_dth_frm_mode		frm_mode;

};

Members

- index_type: the frame index type. For details, see Section 3.1.3, drm_vs_dth_frm_idx.

- sw_index: the frame index.

This member is valid only if index_type is set to VS_DTH_FRM_IDX_SW.

- table_low[3]: the low dither table values for red, green, and blue channels for gamma dither and panel dither..

- table_high[3]: the high dither table values for red, green, and blue channels for gamma dither and panel dither.

- frm_mode: the frame control mode for dithering. For details, see Section 3.1.4, drm_vs_dth_frm_mode.

drm_vs_hist_out

Definition

struct drm_vs_hist_out {

enum drm_vs_hist_out_mode 		out_mode;

struct drm_vs_buffer_object		bo;

};

Members

- out_mode: the histogram result output mode. For details, see Section  3.1.8, drm_vs_hist_out_mode.

- bo: the buffer configuration for storing histogram results. This member is valid only when out_mode is set to VS_HIST_BY_WDMA.

drm_vs_ds_config

Definition

struct drm_vs_ds_config {

enum drm_vs_ds_mode		h_mode;

enum drm_vs_ds_mode		v_mode;

};

Members

- h_mode: the downsampling mode in horizontal direction. For details, see Section 3.1.9, drm_vs_ds_mode.

- v_mode: the downsampling mode in vertical direction. For details, see Section 3.1.9, drm_vs_ds_mode.

drm_vs_ltm_luma_ave

Definition

struct drm_vs_ltm_luma_ave {

bool 		enable;

__u16		margin_x;

__u16		margin_y;

__u16		pixel_norm;

__u16		ave;

};

Members

- enable: indicates whether the LTM luma average module is enabled.

- margin_x: the horizontal margin size for the luma average module.

- margin_y: the vertical margin size for the luma average module.

- pixel_norm: the pixel normalization value for the luma average module.

- ave: the luma average level.

- Private Properties and IOCTLs

This section describes the private DRM properties and IOCTLs by feature.

Note: Feature support varies among different product versions. For the list of supported features and their descriptions, see the Key Features sections in the provided VeriSilicon display processing IP hardware architecture and features document.

Pre-Processing Pipelines

DMA

DMA_CONFIG

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series and DC8x00 series |
| Data Type | Blob object that contains the DMA mode and ROI configurations provided through the drm_vs_dma structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_dma

Definition

#define VS_MAX_ROI_CNT 2

struct drm_vs_dma {

enum drm_vs_dma_mode   mode;

struct drm_vs_rect     in_rect[VS_MAX_ROI_CNT];

struct drm_vs_rect     out_rect[VS_MAX_ROI_CNT];

};

Members

- mode: the DMA read mode. Set this member to a value listed in Table 13.

Note 1: Scaling is not supported if mode is set to VS_DMA_TWO_ROI, VS_DMA_SKIP_ROI, VS_DMA_EXT_LAYER, or VS_DMA_EXT_LAYER_EX.

Note 2: DC8x00 series only support VS_DMA_NORMAL and VS_DMA_ONE_ROI.

Table 13. drm_vs_dma_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_DMA_NORMAL | 0 | Reads the entire source buffer. |
| VS_DMA_ONE_ROI | 1 | Reads one ROI from the source buffer. |
| VS_DMA_TWO_ROI | 2 | Reads two ROIs from the source buffer. |
| VS_DMA_SKIP_ROI | 3 | Skips the ROI from the source buffer. |
| VS_DMA_EXT_LAYER | 4 | Reads source buffer 0 and source buffer 1. If this value is selected, make sure that the EXT_LAYER_FB property is set. |
| VS_DMA_EXT_LAYER_EX | 5 | Reads one ROI from source buffer 0 and the other from source buffer 1. If this value is selected, make sure that the EXT_LAYER_FB property is set. |

- in_rect: the ROIs for input. For details, see Section 3.2.1, drm_vs_rect.

- If mode is set to VS_DMA_ONE_ROI, set in_rect[0] to the ROI to be read.

- If mode is set to VS_DMA_TWO_ROI, set in_rect[0] and in_rect[1] to the ROIs to be read.

- Make sure that the vertical gap between the ROIs is greater than 0.

- If mode is set to VS_DMA_SKIP_ROI, set in_rect[0] to the ROI to be skipped for input.

- If mode is set to VS_DMA_EXT_LAYER_EX, set in_rect[0] to the ROI to be read from source buffer 0 and in_rect[1] to the ROI to be read from source buffer 1.

- out_rect: the ROIs for output. For details, see Section 3.2.1, drm_vs_rect.

- If mode is set to VS_DMA_ONE_ROI, set out_rect[0] to the ROI to be output.

- If mode is set to VS_DMA_TWO_ROI, set out_rect[0] and out_rect[1] to the ROIs to be output.

- If mode is set to VS_DMA_SKIP_ROI, set out_rect[0] to the ROI to be skipped for output.

- If mode is set to VS_DMA_EXT_LAYER or VS_DMA_EXT_LAYER_EX, set out_rect[0] to the ROI to be output for source buffer 0 and out_rect[1] to the ROI to be output for source buffer 1.

When two output ROIs are specified, make sure:

- The ROIs do not overlap.

- The vertical gap between the ROIs is greater than 0 for DC9400 and 16 for DC9500.

EXT_LAYER_FB

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Type | Frame buffer object |
| Description | The ID of the frame buffer object. This property is valid only if the MODE member of the drm_vs_dma structure for the DMA_CONFIG property is set to VS_DMA_EXT_LAYER or VS_DMA_EXT_LAYER_EX. |

MULTI_EXT_LAYER

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the extended layer configurations provided through the drm_vs_ex_layer structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_ex_layer

Definition

#define VS_MAX_EXT_LAYER_CNT 3

struct drm_vs_ex_layer {

__u8			num;

__u32			fb_id[VS_MAX_EXT_LAYER_CNT];

int			fd;

struct drm_vs_rect 	out_rect[VS_MAX_EXT_LAYER_CNT];

__u32			fourcc;

__u64			modifier;

struct drm_vs_rect 	crop_rect[VS_MAX_EXT_LAYER_CNT];

};

Members

Note: For the constraints of the extended layer feature, see the provided Vivante display processing IP hardware features document.

- num: the number of extended layers.

Valid values: 0 to VS_MAX_EXT_LAYER_CNT, where VS_MAX_EXT_LAYER_CNT specifies the maximum number of extended layers.

- fb_id: the frame buffer object ID of each extended layer.

- fd: the file descriptor of the ongoing process.

- out_rect: the output ROI of each extended layer. For details, see Section 3.2.1, drm_vs_rect.

- fourcc: the resource format of the extended layer.

- modifier: the modifiers for the extended layer resource compression format.

- crop_rect: the rectangular cropping area of the extended layer. For details, see Section 3.2.1, drm_vs_rect.

UV Upsampling

UP_SAMPLE

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series except DC9000Nano |
| Data Type | Range |
| Values | Min = 0 Max = 0xFFFF |
| Description | UP_SAMPLE[7:0]: the horizontal phase, ranging from 0 to 16. UP_SAMPLE[15:8]: the vertical phase, ranging from 0 to 16. Set the phase values according to the previously used downsampling method. |

YUV-to-RGB Conversion

Y2R_CONFIG

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the custom YUV-to-RGB conversion coefficients provided through the drm_vs_y2r_config structure |
| Description | The ID of the blob object set. The default value is 0. Note: If you need to use standard YUV-to-RGB conversion coefficients, call the standard property COLOR_ENCODING. No matter whether standard or custom coefficients are used, make sure that the YUV color range is specified using the COLOR_RANGE property. For property details, see the Linux GPU Driver Developer's Guide. |

drm_vs_y2r_config

Definition

#define VS_MAX_Y2R_COEF_NUM    15

struct drm_vs_y2r_config {

enum drm_vs_csc_mode		mode;

enum drm_vs_csc_gamut	gamut;

__s32				coef[VS_MAX_Y2R_COEF_NUM];

};

Members

- mode: the color space conversion (CSC) mode. For details, see Section 3.1.1, drm_vs_csc_mode.

- gamut: the color gamut. For details, see Section 3.1.2, drm_vs_csc_gamut.

- coef: the YUV-to-RGB conversion coefficients.

  - coef[0] to coef[8]: coefficients 00 to 22, in the S7.12 format (1 signed bit, 6 integer bits, 12 fraction bits).

  - coef[9] to coef[11]: pre-offsets D0 to D2. Each offset has 13 bits (1 signed bit and 12 integer bits).

  - coef[12] to coef[14]: post-offsets D0 to D2. Each offset has 13 bits (1 signed bit and 12 integer bits).

Color Keying

COLORKEY

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC8x00 series |
| Data Type | Blob object that contains the custom color keying value range provided through the drm_vs_colorkey structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_colorkey

Definition

struct drm_vs_colorkey {

struct drm_vs_color	colorkey_low;

struct drm_vs_color	colorkey_high;

struct drm_vs_rect	rect;

bool			transparency;

};

Members

- colorkey_low: the start value of the color keying range.

- colorkey_high: the end value of the color keying range.

- rect: the rectangular area that color keying is applied to. For details, see Section 3.2.1, drm_vs_rect.

- transparency: whether a plane is transparent.

Gamma

PRIOR_GAMMA

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the prior gamma LUT configurations provided through the drm_vs_gamma_lut structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_gamma_lut

For structure details, see Section 4.3.15.1.1, drm_vs_gamma_lut.

De-Gamma Correction

DEGAMMA_CONFIG

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC8x00 series |
| Data Type | Blob object that contains the de-gamma mode and color lookup table (LUT) configurations provided through the drm_vs_degamma_config structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_degamma_config

Definition

#define DEGAMMA_SIZE	260

struct drm_vs_degamma_config {

enum drm_vs_degamma_mode 	mode;

__u16				r[DEGAMMA_SIZE];

__u16				g[DEGAMMA_SIZE];

__u16				b[DEGAMMA_SIZE];

};

Members

- mode: the method of generating a de-gamma lookup table (LUT). Set this member to a value listed in the following table.

Table 14. drm_vs_degamma_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_DEGAMMA_BT709 | 0 | Generates the de-gamma LUT from the BT.709 Gamma 2.4 curve. |
| VS_DEGAMMA_BT2020 | 1 | Generates the de-gamma LUT from the BT.2020 PQ curve. |
| VS_DEGAMMA_USR | 2 | Generates the de-gamma LUT from custom arrays r, g, and b. |

- To generate the de-gamma LUT from custom arrays, set the mode member to VS_DEGAMMA_USR and then set the following members:

- r: the array of red color values in a de-gamma LUT.

- g: the array of green color values in a de-gamma LUT.

- b: the array of blue color values in a de-gamma LUT.

DC9x00 DEGAMMA

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the de-gamma LUT configurations provided through the drm_vs_xstep_lut structure |
| Description | The ID of the blob object set. The default value is 0. |

High Dynamic Range

HDR_UNION

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the high dynamic range (HDR) configurations for pre-processing provided through the drm_vs_hdr structure |
| Description | The ID of the blob object set. The default value is 0. To use a HDR feature, set hdr_enable to 1 in the .json configuration file before configuring other properties. |

drm_vs_hdr

Definition

struct drm_vs_hdr {

bool		demultiply;

bool		eotf_enable;

bool 		gamut_map_enable;

bool		tone_map_enable;

bool 		oetf_enable;

enum 		drm_vs_hdr_mode hdr_mode;

struct		drm_vs_xstep_lut eotf_config;

struct		drm_vs_ccm gamut_map;

struct		drm_vs_tone_map tone_map;

struct		drm_vs_xstep_lut oetf_config;

};

Members

- demultiply: whether to enable the de-multiply module in the HDR pipeline.

To ensure compatibility with different GPU output modes, the positions of the demultiply and Electro-Optical Transfer Function (EOTF) modules will be swapped.

For details, see Section 4.1.7.2, De-Multiply.

- eotf_enable: whether to enable the EOTF module in the HDR pipeline.

- gamut_map_enable: whether to enable the gamut mapping module in the HDR pipeline.

- tone_map_enable: whether to enable the tone mapping module in the HDR pipeline.

- oetf_enable: whether to enable the Optical-Electro Transfer Function (OETF) module in the HDR pipeline.

- hdr_mode: different HDR modes.

Different HDR modes implement distinct processing flows. For example, when HDR V3 hardware is used and hdr_mode is HDR Vivid, the execution order of the Color Correction Matrix (CCM) and Tone Mapping modules are swapped.

For details, see Section 3.1.10, drm_vs_hdr_mode.

- eotf_config: the configurations of the EOTF module in the HDR pipeline.

This member is required when eotf_enable is set to TRUE.

For details, see Section 4.1.7.3, EOTF.

- gamut_map: the configurations of the gamut mapping module in the HDR pipeline.

This member is required when gamut_map_enable is set to TRUE.

For details, see Section 4.1.7.4, Gamut Mapping.

- tone_map: the configurations of the tone mapping module in the HDR pipeline.

This member is required when tone_map_enable is set to TRUE.

For details, see Section 3.2.2, drm_vs_ccm.

- oetf_config: the configurations of the OETF module in the HDR pipeline.

This member is required when oetf_enable is set to TRUE.

For details, see Section 4.1.7.6, OETF.

De-Multiply

de_multiply

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Boolean |
| Description | Enables or disables the de-multiply module. Valid values: 1: enables the de-multiply module. 0: disables the de-multiply module. The default value is 0. |

EOTF

eotf

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the EOTF configurations for pre-processing provided through the drm_vs_xstep_lut structure |
| Description | The ID of the blob object set. The default value is 0. |

Gamut Mapping

gamut

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series and DC8x00 series |
| Data Type | Blob object that contains the gamut mapping configurations for pre-processing, provided through the drm_vs_ccm structure |
| Description | The ID of the blob object set. The default value is 0. |

Tone Mapping

tonemapping

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the tone mapping configurations provided through the drm_vs_tone_map structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_tone_map

Definition

struct drm_vs_tone_map {

struct drm_vs_tone_map_y		pseudo_y;

struct drm_vs_lut			lut;

};

Members

- pseudo_y: the pseudo Y configurations. For details, see Section 4.1.7.5.3, drm_vs_tone_map_y.

- lut: the LUT configurations for tone mapping. For details, see Section 4.1.7.5.4, drm_vs_lut.

drm_vs_tone_map_y

Definition

struct drm_vs_tone_map_y {

enum drm_vs_calcu_mode	y_mode;

__u16				coef0;

__u16				coef1;

__u16				coef2;

__u16				weight;

};

Members

- y_mode: the pseudo Y generation mode. Set this member to a value listed in the following table.

Table 15. drm_vs_calcu_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_CALC_LNR_COMBINE | 0 | Uses a linearly weighted sum of red, green, and blue values as pseudo Y. Ylinear_weight=(coef0 coef1 coef2)RinputGinputBinput (1) |
| VS_CALC_MAX | 1 | Uses the maximum of red, green, and blue values as pseudo Y. Ymax=Max(Rinput, Ginput, Binput) (2) |
| VS_CALC_MIXED | 2 | Derives pseudo Y from a linear combination of the above two values. Y=Ylinear_weight×weight+Ymax×1024-weight+512≫10 (3) |

- coef0, coef1, and coef2: U12 valid at LSB, used in formula 1.

These members are valid only if y_mode is set to VS_CALC_LNR_COMBINE or VS_CALC_MIXED.

- weight: U10 valid at LSB, used in formula 3.

This member is valid only if y_mode is set to VS_CALC_MIXED.

drm_vs_lut

Definition

#define VS_MAX_LUT_ENTRY_CNT	129

struct drm_vs_lut {

__u32		entry_cnt;

__u32		seg_point[VS_MAX_LUT_ENTRY_CNT - 1];

__u32		data[VS_MAX_LUT_ENTRY_CNT];

};

Members

- entry_cnt: the number of entries.

- seg_point: the array of segment points except the first point.

The first point is fixed to 0, and is not configurable.

The last point is U25 valid at LSB. Each of the other points is U24 valid at LSB.

data: the per-entry LUT value array. Each value is U24 valid at LSB.

OETF

oetf

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the OETF configurations for pre-processing, provided through the drm_vs_xstep_lut structure |
| Description | The ID of the blob object set. The default value is 0. |

Super Frame Line Padding

LINE_PADDING

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the super frame line padding configurations through the drm_vs_line_padding structure |
| Description | The ID of the blob object set. The default value is 0, which indicates that super frame line padding is disabled. |

drm_vs_line_padding

Definition

struct drm_vs_line_padding {

enum drm_vs_line_padding_mode	mode;

struct drm_vs_color			color;

};

Members

- mode: the line padding mode. Set this member to a value listed in Table 14.

Table 14. drm_vs_line_padding_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_DMA_LINE_PADDING_1TO1 | 0 | Ratio 1:1 |
| VS_DMA_LINE_PADDING_2TO1 | 1 | Ratio 2:1 |
| VS_DMA_LINE_PADDING_3TO1 | 2 | Ratio 3:1 |
| VS_DMA_LINE_PADDING_3TO2 | 3 | Ratio 3:2 |
| VS_DMA_LINE_PADDING_4TO1 | 4 | Ratio 4:1 |
| VS_DMA_LINE_PADDING_4TO3 | 5 | Ratio 4:3 |
| VS_DMA_LINE_PADDING_5TO2 | 6 | Ratio 5:2 |
| VS_DMA_LINE_PADDING_5TO3 | 7 | Ratio 5:3 |
| VS_DMA_LINE_PADDING_8TO5 | 8 | Ratio 8:5 |

- color: the padded color. For details, see Section 3.2.3, drm_vs_color.

Data Extending

DATA_EXTEND

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the bit depth extension mode of ARGB or YUV components through the drm_vs_data_extend structure The bit depth extension feature extends pixel components by padding extra bits as lowest bits. It can extend alpha components to 10-bit and extend other components of RGB formats and YUV formats to 12-bit. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_data_extend

Definition

struct drm_vs_data_extend {

enum drm_vs_data_extend_mode	data_extend_mode;

struct drm_vs_alpha_data_extend	alpha_data_extend;

};

Members

- data_extend_mode: the bit depth extension mode of ARGB or YUV components except the 2-bit alpha component of the A2R10G10B10 format. For details, see Section 3.1.7, drm_vs_data_extend_mode.

- alpha_data_extend: the bit depth extension mode for the 2-bit alpha component of the A2R10G10B10 format. For details, see Section 4.1.9.1.2, drm_vs_alpha_data_extend.

drm_vs_alpha_data_extend

Definition

struct drm_vs_alpha_data_extend {

bool		enable;

__u32		alpha_extend_value;

};

Members

- enable: whether to enable the bit depth extension feature for alpha components.

Valid values:

true: enables bit depth extension for alpha components.

false: disables bit depth extension for alpha components.

The default value is false.

- alpha_extend_value: the candidate values to be filled in the extended eight bits of the alpha component.

alpha_extend_value[7:0]: candidate value 0, which is used when the original alpha component equals 0.

alpha_extend_value[15:8]: candidate value 1, which is used when the original alpha component equals 1.

alpha_extend_value[23:16]: candidate value 2, which is used when the original alpha component equals 2.

alpha_extend_value[31:24]: candidate value 3, which is used when the original alpha component equals 3.

PVRIC Decompressor

PVRIC Modifier

Compatible Hardware

The PVRIC modifier is available only for DC9x00 series.

Modifier Layout

- Bits [63:56]: the vendor ID

#define DRM_FORMAT_MOD_VENDOR_VS 0x0b

- Bits [55:53]: the category of the modifier

#define DRM_FORMAT_MOD_VS_TYPE_MASK ((__u64)0x07 << 53)

#define DRM_FORMAT_MOD_VS_TYPE_PVRIC 0x02

- Bits [9:8]: the PVRIC compression type, either lossy or lossless

#define DRM_FORMAT_MOD_VS_DEC_LOSSY    (0x01 << 8)

#define DRM_FORMAT_MOD_VS_DEC_LOSSLESS (0x01 << 9)

- Bits [5:0]: the PVRIC tile type

#define DRM_FORMAT_MOD_VS_DEC_TILE_MODE_MASK	0x3F

#define DRM_FORMAT_MOD_VS_DEC_TILE_16X4		0x02

#define DRM_FORMAT_MOD_VS_DEC_TILE_8X8		0x30

#define DRM_FORMAT_MOD_VS_DEC_TILE_32X2		0x31

Modifier Generation

#define fourcc_mod_vs_pvric_code(tile, lossy) \

fourcc_mod_vs_code(DRM_FORMAT_MOD_VS_TYPE_PVRIC, ((tile) | (lossy)))

PVRIC_CLEAR

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the PVRIC clear color configurations provided through the drm_vs_pvric_clear structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_pvric_clear

Definition

struct drm_vs_pvric_clear {

__u64 color[3];

};

Members

color: the clear color value for requesters.

- Bits 31:0 of color[i]: the low 32 bits of the PVRIC clear color value for requester i.

- Bits 63:32 of color[i]: the high 32 bits of the PVRIC clear color value for requester i.

The specified constant color is assigned to the fbdc_clear_colour signal when the compressor tile header indicates that the tile contains only clear-color pixels.

PVRIC_CONST

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the PVRIC constant color value provided through the drm_vs_pvric_const structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_pvric_const

Definition

struct drm_vs_pvric_const {

struct drm_vs_color color[2];

};

Members

color: the constant color values. For structure details, see Section 4.1.10.3.2, drm_vs_color.

- color[0]: constant value 0.

It specifies the constant color to be assigned to the following signals when the compressor tile header indicates that the tile consists of only a constant color:

- Non-YUV format: fbc_fbdc_cr_ch0123_val0

- YUV format: fbc_fbdc_cr_y_val0 and fbc_fbdc_cr_uv_val0

- color[1]: constant value 1.

It specifies the constant color to be assigned to the following signals when the compressor tile header indicates that the tile consists of only a constant color:

- Non-YUV format: fbc_fbdc_cr_ch0123_val1

- YUV format: fbc_fbdc_cr_y_val1 and fbc_fbdc_cr_uv_val1

drm_vs_color

Definition

struct drm_vs_color {

__u32		a;

__u32		r;

__u32		g;

__u32		b;

};

Members

For non-YUV formats:

- a[7:0]: color channel 3 or bits 31:24 of each pixel

- r[7:0]: color channel 2 or bits 23:16 of each pixel

- g[7:0]: color channel 1 or bits 15:8 of each pixel

- b[7:0]: color channel 0 or bits 7:0 of each pixel

For YUV formats:

- a[9:0]: 10-bit Y value

- r[9:0]: 10-bit UV value

- Members g and b are invalid.

IOCTL for Base Address Offset Configuration of PVRIC Data Sections

Compatible Hardware

This IOCTL is available only for DC9x00 series.

Definition

#define DRM_IOCTL_VS_GET_FBC_OFFSET \

DRM_IOWR(DRM_COMMAND_BASE + DRM_VS_GET_FBC_OFFSET, struct drm_vs_pvric_offset)

DRM_IOCTL_DEF_DRV(VS_GET_FBC_OFFSET, vs_get_fbc_offset_ioctl, DRM_MASTER)

For information about the private structure, see Section 4.1.10.4.1, drm_vs_pvric_offset.

drm_vs_pvric_offset

Definition

struct drm_vs_pvric_offset {

__u32 format;

__u32 handles[3];

__u32 header_size[3];

__u64 offsets[3];

};

Members

- format: the four-character code (FourCC) format identifier, in the DRM_FORMAT_* format.

- handles: an array of user-space handles.

- header_size: an array of header section sizes.

- offsets: an array used to receive the base address offsets of PVRIC data sections.

DEC400 Decompressor

DEC400 Modifier

Compatible Hardware

The DEC400 modifier is available for DC8x00 series and DC9x00 series.

Modifier Layout

- Bits [63:56]: the vendor ID

#define DRM_FORMAT_MOD_VENDOR_VS 0x0b

- Bits [55:53]: the category of the modifier

#define DRM_FORMAT_MOD_VS_TYPE_MASK ((__u64)0x07 << 53)

#define DRM_FORMAT_MOD_VS_TYPE_COMPRESSED 0x01

- Bits [7:6]: the data alignment mode for DEC400 compression

#define DRM_FORMAT_MOD_VS_DEC_ALIGN_SHIFT    6

#define DRM_FORMAT_MOD_VS_DEC_ALIGN_MASK    ((__u64)0x03 << 6)

#define DRM_FORMAT_MOD_VS_DEC_ALIGN_32    0x00

#define DRM_FORMAT_MOD_VS_DEC_ALIGN_64    0x01

- Bits [5:0]: the DEC400 tile type

#define DRM_FORMAT_MOD_VS_DEC_TILE_MODE_MASK	0x3F

#define DRM_FORMAT_MOD_VS_DEC_TILE_8X8_XMAJOR	0x00

#define DRM_FORMAT_MOD_VS_DEC_TILE_16X4		0x02

#define DRM_FORMAT_MOD_VS_DEC_TILE_8X4		0x03

#define DRM_FORMAT_MOD_VS_DEC_TILE_4X8		0x04

#define DRM_FORMAT_MOD_VS_DEC_TILE_32X4		0x08

#define DRM_FORMAT_MOD_VS_DEC_RASTER_256X1	0x09

#define DRM_FORMAT_MOD_VS_DEC_RASTER_128X1	0x0A

#define DRM_FORMAT_MOD_VS_DEC_RASTER_64X1	0x0F

#define DRM_FORMAT_MOD_VS_DEC_TILE_16X8		0x10

#define DRM_FORMAT_MOD_VS_DEC_RASTER_32X1	0x16

#define DRM_FORMAT_MOD_VS_DEC_TILE_32X8		0x24

#define DRM_FORMAT_MOD_VS_DEC_LINEAR		0x32

#define DRM_FORMAT_MOD_VS_DEC_TILE_8X8_UNIT2X2		0x34

#define DRM_FORMAT_MOD_VS_DEC_TILE_8X4_UNIT2X2		0x35

#define DRM_FORMAT_MOD_VS_DEC_TILE_8X8_SUPERTILE_X	0x36

#define DRM_FORMAT_MOD_VS_DEC_TILE_32X8_YUVSP8X8	0x37

#define DRM_FORMAT_MOD_VS_DEC_TILE_32X4_YUVSP8X8	0x38

#define DRM_FORMAT_MOD_VS_DEC_TILE_16X8_YUVSP8X8	0x39

#define DRM_FORMAT_MOD_VS_DEC_TILE_16X4_YUVSP8X8	0x3A

Modifier Generation

#define fourcc_mod_vs_dec_code(tile, align) \

fourcc_mod_vs_code(DRM_FORMAT_MOD_VS_TYPE_COMPRESSED, ((tile) | (align << DRM_FORMAT_MOD_VS_DEC_ALIGN_SHIFT)))

Fast Clear

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the fast clear configurations for pre-processing provided through the dc_dec400_fc structure |
| Description | The ID of the blob object set. The default value is 0. |

dc_dec400_fc

Definition

struct dc_dec400_fc {

bool		fc_enable;

__u32		fc_size;

__u32		fc_rgby_value;

__u32		fc_uv_value;

__u32		stride[3];

};

Members

- fc_enable: whether to enable the Vulkan mode of fast clear.

Valid values:

true: enable the Vulkan mode.

- In this mode, DEC400 accesses DDR to obtain fast clear values.

false: disable the Vulkan mode and use the normal mode.

- In this mode, DEC400 uses the fast clear values configured by using the corresponding registers.

The default value is false.

- fc_size: the memory size for DEC400 Vulkan fast clear values.

Valid values: 128 and 256. Unit: bytes. The default value is 128.

- fc_rgby_value: the RGB or Y value in 32 bits for fast clear.

- fc_uv_value: the UV value in 32 bits for fast clear.

- stride[3]: the stride for each of the three memory planes, stored as 32-bit values for fast clear operations.

Fast Clear Modifier

Compatible Hardware

The fast clear modifier is available for DC8x00 series and DC9x00 series.

Modifier Layout

- Bit [14:14]: the fast clear switch

#define DRM_FORMAT_MOD_VS_DEC_FC_MASK (0x01 << 14)

#define DRM_FORMAT_MOD_VS_DEC_FC_DISABLE (0x00 << 14)

#define DRM_FORMAT_MOD_VS_DEC_FC_ENABLE (0x01 << 14)

- Bit [15:15]: the memory size for fast clear values.

#define DRM_FORMAT_MOD_VS_DEC_FC_SIZE_MASK (0x01 << 15)

#define DRM_FORMAT_MOD_VS_DEC_FC_SIZE_128 (0x00 << 15)

#define DRM_FORMAT_MOD_VS_DEC_FC_SIZE_256 (0x01 << 15)

DEC400A Decompressor

DEC400A Modifier

Compatible Hardware

The DEC400A modifier is available for DC9x00 series.

Modifier Layout

- Bits [63:56]: the vendor ID

#define DRM_FORMAT_MOD_VENDOR_VS 0x0b

- Bits [55:53]: the category of the modifier

#define DRM_FORMAT_MOD_VS_TYPE_MASK ((__u64)0x07 << 53)

#define DRM_FORMAT_MOD_VS_TYPE_DEC400A 0x06

- Bits [5:0]: the DEC400A tile type

#define DRM_FORMAT_MOD_VS_DEC_TILE_MODE_MASK	0x3F

#define DRM_FORMAT_MOD_VS_DEC_TILE_8X8_XMAJOR	0x00

#define DRM_FORMAT_MOD_VS_DEC_TILE_16X4		0x02

#define DRM_FORMAT_MOD_VS_DEC_TILE_8X4		0x03

#define DRM_FORMAT_MOD_VS_DEC_TILE_4X8		0x04

#define DRM_FORMAT_MOD_VS_DEC_TILE_32X4		0x08

#define DRM_FORMAT_MOD_VS_DEC_RASTER_256X1	0x09

#define DRM_FORMAT_MOD_VS_DEC_RASTER_128X1	0x0A

#define DRM_FORMAT_MOD_VS_DEC_RASTER_64X1	0x0F

#define DRM_FORMAT_MOD_VS_DEC_TILE_16X8		0x10

#define DRM_FORMAT_MOD_VS_DEC_RASTER_32X1	0x16

#define DRM_FORMAT_MOD_VS_DEC_TILE_32X8		0x24

#define DRM_FORMAT_MOD_VS_DEC_LINEAR		0x32

#define DRM_FORMAT_MOD_VS_DEC_TILE_8X8_UNIT2X2		0x34

#define DRM_FORMAT_MOD_VS_DEC_TILE_8X4_UNIT2X2		0x35

#define DRM_FORMAT_MOD_VS_DEC_TILE_8X8_SUPERTILE_X	0x36

#define DRM_FORMAT_MOD_VS_DEC_TILE_32X8_YUVSP8X8	0x37

#define DRM_FORMAT_MOD_VS_DEC_TILE_32X4_YUVSP8X8	0x38

#define DRM_FORMAT_MOD_VS_DEC_TILE_16X8_YUVSP8X8	0x39

#define DRM_FORMAT_MOD_VS_DEC_TILE_16X4_YUVSP8X8	0x3A

#define DRM_FORMAT_MOD_VS_DEC_TILE_32X8_A		0x3B

- Bit [12:12]: the configurations of the DEC400A RGB-to-YUV conversion

#define DRM_FORMAT_MOD_VS_DEC_YUV_TRANSFORM_SHIFT	12

#define DRM_FORMAT_MOD_VS_DEC_YUV_TRANSFORM_MASK	(0x01 << 12)

#define DRM_FORMAT_MOD_VS_DEC_YUV_TRANSFORM_DISABLE   0x00

#define DRM_FORMAT_MOD_VS_DEC_YUV_TRANSFORM_ENABLE	0x01

Modifier Generation

#define fourcc_mod_vs_dec400a_code(tile, yuv_transform) \

fourcc_mod_vs_code(DRM_FORMAT_MOD_VS_TYPE_DEC400A, \

((tile) | (yuv_transform << DRM_FORMAT_MOD_VS_DEC_YUV_TRANSFORM_SHIFT)))

Custom Format

Custom Format Modifier

Compatible Hardware

The custom format modifier is available for DC8x00 series and DC9x00 series.

Modifier Layout

- Bit [13:13]: whether to use a custom format.

#define DRM_FORMAT_MOD_VS_CUSTOM_FORMAT_MASK (0x01 << 13)

#define DRM_FORMAT_MOD_VS_CUSTOM_FORMAT_DISABLE (0x00 << 13)

#define DRM_FORMAT_MOD_VS_CUSTOM_FORMAT_ENABLE (0x01 << 13)

Color Conversion Matrix After De-gamma

CCM

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the matrix configurations for color conversion after de-gamma, provided through the drm_vs_ccm structure |
| Description | The ID of the blob object set. The default value is 0. |

Cropping

LAYER_CROP

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the cropping configurations provided through the drm_vs_panel_crop structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_panel_crop

Definition

struct drm_vs_panel_crop {

struct drm_vs_rect		crop_rect;

__u16 				panel_src_width;

__u16				panel_src_height;

};

Members

- crop_rect: the rectangular area output after cropping.

For details, see Section 3.2.1, drm_vs_rect.

- panel_src_width: the width of the source image before cropping.

- panel_src_height: the height of the source image before cropping.

Blending Pipelines

Blending

BLEND_MODE

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series and DC8x00 series |
| Data Type | Blob object that contains the blending mode configurations provided through the drm_vs_blend structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_blend

Definition

struct drm_vs_blend {

enum drm_vs_blend_mode		color_mode;

enum drm_vs_blend_mode		alpha_mode;

};

Members

Set each of the following members to a value listed in Table 15.

- color_mode: the color blending mode.

- alpha_mode: the alpha blending mode.

Note 1: For DC9x00 series, the blending modes except VS_BLD_UDEF are treated as fast modes. To customize the alpha configurations, use the BLEND_ALPHA property. For VS_BLD_UDEF, BLEND_ALPHA must be set.

Note 2: For DC8x00 series, the blending modes VS_BLD_PLUS, VS_BLD_BLD, and VS_BLD_UDEF are not supported.

Table 15. drm_vs_blend_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_BLD_CLR | 0 | Clears destination pixels covered by the source to 0. Result alpha value=0 Result color value=0 |
| VS_BLD_SRC | 1 | Replaces the destination pixels with the source pixels. Result alpha value=Sa Result color value=Sc |
| VS_BLD_DST | 2 | Discards the source pixels and leaves the destination unchanged. Result alpha value=Da Result color value=Dc |
| VS_BLD_SRC_OVR | 3 | Draws the source pixels over the destination pixels. Result alpha value=Sa+(1-Sa)×Da Result color value=Sc+(1 - Sa) ×Dc |
| VS_BLD_DST_OVR | 4 | Draws the source pixels behind the destination pixels. Result alpha value=Da+(1-Da)×Sa Result color value=Dc+(1-Da)×Sc |
| VS_BLD_SRC_IN | 5 | Keeps the source pixels that cover the destination pixels, and discards the remaining source and destination pixels. Result alpha value=Sa×Da Result color value=Sc×Da |
| VS_BLD_DST_IN | 6 | Keeps the destination pixels that cover source pixels, and discards the remaining source and destination pixels. Result alpha value=Sa×Da Result color value=Sa×Dc |
| VS_BLD_SRC_OUT | 7 | Keeps the source pixels that do not cover destination pixels, and discards the remaining source and destination pixels. Result alpha value=Sa×(1-Da) Result color value=Sc×(1-Da) |
| VS_BLD_DST_OUT | 8 | Keeps the destination pixels that are not covered by source pixels, and discards the remaining source and destination pixels. Result alpha value=Da×(1-Sa) Result color value=Dc×(1-Sa) |
| VS_BLD_SRC_ATOP | 9 | Discards the source pixels that do not cover destination pixels, and draws the remaining source pixels over destination pixels. Result alpha value=Da Result color value=Sc×Da+(1-Sa)×Dc |
| VS_BLD_DST_ATOP | 10 | Discards the destination pixels that are not covered by source pixels, and draws the remaining destination pixels over source pixels. Result alpha value=Sa Result color value=Sa×Dc+Sc×(1-Da) |
| VS_BLD_XOR | 11 | Discards the source and destination pixels where source pixels cover destination pixels, and draws the remaining source pixels. Result alpha value=Sa+Da-2×Sa×Da Result color value=Sc×(1-Da)+(1-Sa)×Dc |
| VS_BLD_PLUS | 12 | Result alpha value=( Sa+Da)/2 Result color value=( Sc+Dc)/2 |
| VS_BLD_BLD | 13 | Result alpha value=Sa× Sa+Da×(1-Sa) Result color value =Sc× Sa+Dc×(1-Sa) |
| VS_BLD_UDEF | 14 | Result alpha value=Sa×Sa_reg+Da×Da_reg Result color value =Sc×Sc_reg+Dc×Dc_reg If this value is selected, make sure that alpha pre-processing is configured using the BLEND_ALPHA property. |

BLEND_ALPHA

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series and DC8x00 series |
| Data Type | Blob object that contains the alpha configurations provided through the drm_vs_blend_alpha structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_blend_alpha

Definition

struct drm_vs_blend_alpha {

/* src alpha pre progress */

enum drm_vs_alpha_mode		sam;

enum drm_vs_galpha_mode		sgam;

__u32					sga;

__u32					saa;

/* dst alpha pre progress */

enum drm_vs_alpha_mode		dam;

enum drm_vs_galpha_mode		dgam;

__u32					dga;

__u32					daa;

};

Members

- sam: the source alpha mode. Set this member to a value listed in Table 16.

- sgam: the source global alpha mode. Set this member to a value listed in Table 17.

- sga: the source global alpha value.

- saa: the source alpha value used for either color blending or alpha blending.

- dam: the destination alpha mode. Set this member to a value listed in Table 16.

- dgam: the destination global alpha mode. Set this member to a value listed in Table 17.

- dga: the destination global alpha value.

- daa: the destination alpha value used for either color blending or alpha blending.

Table 16. drm_vs_alpha_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_ALPHA_NORMAL | 0 | Uses the input alpha value. αout=αin |
| VS_ALPHA_INVERSE | 1 | Uses the inversed alpha value. αout=1-αin |

Table 17. drm_vs_galpha_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_GALPHA_NORMAL | 0 | Uses the pixel alpha values. |
| VS_GALPHA_GLOBAL | 1 | Uses the global alpha value. |
| VS_GALPHA_MULTIPLE | 2 | Uses the alpha value that is the multiple of the above values. Normalize(Pixel alpha×Global alpha) |

Non-Linear Gamut Mapping

BLD_NONLINEAR_GAMUT_MAP

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the non-linear gamut mapping configurations for blending, provided through the drm_vs_ccm structure |
| Description | The ID of the blob object set. The default value is 0. |

OETF

BLD_OETF

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the OETF configurations for blending, provided through the drm_vs_xstep_lut structure |
| Description | The ID of the blob object set. The default value is 0. |

Blend Dither

BLD_DITHER

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the blend dither configurations provided through the drm_vs_blender_dither structure |
| Description | The ID of the blob object set. The default value is 0, which indicates that the DRM driver uses the default configurations. |

drm_vs_blender_dither

Definition

struct drm_vs_blender_dither {

enum drm_vs_dth_frm_idx			index_type;

__u8						sw_index;

__u8						noise;

__u16						start_x;

__u16						start_y;

__u16						mask;

struct drm_vs_random_dither_seed		seed;

};

Members

- index_type: the frame index type. For details, see Section 3.1.3, drm_vs_dth_frm_idx.

- sw_index: the frame index.

This member is valid only if index_type is set to VS_DTH_FRM_IDX_SW.

- noise: Control the additional bits to add noise besides the bit difference between input and output.

- start_x: the start X-coordinate for crop.

- start_y: the start Y-coordinate for crop.

- mask: the mask used to XOR with the value generated by PRNG.

- seed: the random seed configurations for blend dither. For details, see Section 4.2.4.1.2, drm_vs_random_dither_seed.

drm_vs_random_dither_seed

Definition

#define VS_RANDOM_DITHER_SEED_NUM	8

struct drm_vs_random_dither_seed {

bool		hash_seed_x_enable;

bool		hash_seed_y_enable;

bool		permut_seed1_enable;

bool		permut_seed2_enable;

__u32		hash_seed_x[VS_RANDOM_DITHER_SEED_NUM];

__u32		hash_seed_y[VS_RANDOM_DITHER_SEED_NUM];

__u32		permut_seed1[VS_RANDOM_DITHER_SEED_NUM];

__u32		permut_seed2[VS_RANDOM_DITHER_SEED_NUM];

};

Members

- hash_seed_x_enable: indicates whether the hash seed X-coordinates are valid.

If the hash seed X-coordinates are valid, hash_seed_x is parsed and you need to configure the related registers.

- hash_seed_y_enable: indicates whether the hash seed Y-coordinates are valid.

If the hash seed Y-coordinates are valid, hash_seed_y is parsed and you need to configure the related registers.

- permut_seed1_enable: indicates whether permutation seed array 1 is valid.

If permutation seed array 1 is valid, permut_seed1 is parsed and you need to configure the related registers.

- permut_seed2_enable: indicates whether permutation seed array 2 is valid.

If permutation seed array 2 is valid, permut_seed2 is parsed and you need to configure the related registers.

- hash_seed_x: the array of hash seed X-coordinates.

The array requires eight entries. The default value is [45031, 47417, 37483, 43973, 37517, 45319, 41203, 49261].

This member is valid only if hash_seed_x_en is set to 1.

- hash_seed_y: the array of hash seed Y-coordinates.

The array requires eight entries. The default value is [14463, 56533, 24659, 16843, 20789, 22447, 18371, 32503].

This member is valid only if hash_seed_y_en is set to 1.

- permut_seed1: the permutation seed array 1.

The array requires eight entries. The default value is [30340, 60458, 19992, 32054, 45828, 55594, 31668, 17290].

This member is valid only if permut_seed1_en is set to 1.

- permut_seed2: the permutation seed array 2.

The array requires eight entries. The default value is [27779, 27179, 30013, 29179, 29363, 30529, 27253, 23599].

This member is valid only if permut_seed2_en is set to 1.

EOTF

BLD_EOTF

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the EOTF configurations for blending, provided through the drm_vs_xstep_lut structure |
| Description | The ID of the blob object set. The default value is 0. |

Gamut Mapping

BLD_GAMUT_MAP

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the gamut mapping configurations for blending, provided through the drm_vs_ccm structure |
| Description | The ID of the blob object set. The default value is 0. |

Background Color

BG_COLOR

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series and DC8x00 series |
| Data Type | Blob object that contains the color configurations provided through the drm_vs_color structure |
| Description | The ID of the blob object set. The default value is 0. |

Solid Color

CLEAR

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series and DC8x00 series |
| Data Type | Blob object that contains the clear color value for the plane, provided through the drm_vs_color structure |
| Description | The ID of the blob object set. The default value is 0. |

Post-Processing Pipelines

Scaling

SCALER

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the post-blending scaling configurations provided by the drm_vs_scale_config structure |
| Description | The ID of the blob object set. The default value is 0, which indicates that the DRM driver uses the default configurations. |

Local Tone Mapping

LTM

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the local tone mapping (LTM) configurations provided through the drm_vs_ltm_xgamma structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_ltm

Definition

struct drm_vs_ltm {

struct drm_vs_ltm_xgamma 		ltm_degamma;

struct drm_vs_ltm_xgamma 		ltm_gamma;

struct drm_vs_ltm_luma 		ltm_luma;

struct drm_vs_ltm_freq_decomp 	freq_decomp;

struct drm_vs_1d_lut 		luma_adj;

struct drm_vs_ltm_grid 		grid_size;

struct drm_vs_ltm_af_filter 	af_filter;

struct drm_vs_ltm_af_slice 		af_slice;

struct drm_vs_ltm_af_trans 		af_trans;

struct drm_vs_ltm_tone_adj 		tone_adj;

struct drm_vs_ltm_color 		ltm_color;

struct drm_vs_ltm_dither 		ltm_dither;

struct drm_vs_ltm_luma_ave 		ltm_luma_set;

struct drm_vs_ltm_cd_set 		ltm_cd_set;

struct drm_vs_ltm_hist_set 		ltm_hist_set;

struct drm_vs_ltm_ds 		ltm_ds;

struct drm_vs_ltm_hist_get 		ltm_hist_get;

struct drm_vs_ltm_af_luts 		ltm_af_luts;

struct drm_vs_ltm_sre_wopr 		ltm_sre_wopr;

struct drm_vs_ltm_sd 		ltm_sd;

bool					ad_txt_jud_en;

bool					color_protect_enable;

enum drm_vs_ltm_mode			ltm2_mode;

};

Members

- ltm_degamma: the LTM de-gamma configurations. For details, see Section 4.3.2.1.2, drm_vs_ltm_xgamma.

- ltm_gamma: the LTM gamma configurations. For details, see Section 4.3.2.1.2, drm_vs_ltm_xgamma.

- ltm_luma: the LTM luma calculation configurations. For details, see Section 4.3.2.1.3, drm_vs_ltm_luma.

- freq_decomp: the configurations of LTM luma frequency decomposition. For details, see Section 4.3.2.1.4, drm_vs_ltm_freq_decomp.

- luma_adj: the LTM luma adjustment configurations. For details, see Section 4.3.2.1.5, drm_vs_1d_lut.

- grid_size: the LTM affine grid size configurations. For details, see Section 4.3.2.1.6, drm_vs_ltm_grid.

- af_filter: the LTM affine grid filtering configurations. For details, see Section 4.3.2.1.7, drm_vs_ltm_af_filter.

- af_slice: the LTM affine grid slicing configurations. For details, see Section 4.3.2.1.8, drm_vs_ltm_af_slice.

- af_trans: the LTM local affine transformation configurations. For details, see Section 4.3.2.1.9, drm_vs_ltm_af_trans.

- tone_adj: the LTM global tone adjustment configurations. For details, see Section 4.3.2.1.10, drm_vs_ltm_tone_adj.

- ltm_color: the LTM color processing configurations. For details, see Section 4.3.2.1.11, drm_vs_ltm_color.

- ltm_dither: the LTM dither configurations. For details, see Section 4.3.2.1.12, drm_vs_ltm_dither.

- ltm_luma_set: the LTM luma average configurations. For details, see Section 3.2.9, drm_vs_ltm_luma_ave.

- ltm_cd_set: the LTM content detection configurations. For details, see Section 4.3.2.1.13, drm_vs_ltm_cd_set.

- ltm_hist_set: the LTM local histogram configurations. For details, see Section 4.3.2.1.14, drm_vs_ltm_hist_set.

- ltm_ds: the LTM downscaling configurations. For details, see Section 4.3.2.1.15, drm_vs_ltm_ds.

- ltm_hist_get: the LTM local histogram configurations.

If LTM local histogram is enabled, the histogram results are written back through the WDMA by default. For details, see Section 4.3.2.1.16, drm_vs_ltm_hist_get.

- ltm_af_luts: the LTM affine lookup table configurations. Used in LTM2.0 and GTM CABC. For details, see Section 4.3.2.1.17, drm_vs_ltm_af_lut.

- ltm_sre_wopr:  the LTM wopr configurations. Used in LTM2.0 and GTM CABC. For details, see Section 4.3.2.1.18, drm_vs_ltm_sre_wopr.

- ltm_sd: the LTM scan detection configurations. Used in LTM2.0 and GTM CABC. For details, see Section 4.3.2.1.19, drm_vs_ltm_sd.

- ad_txt_jud_en:  whether to enable the average roi detect module. Only used in LTM2.0.

- color_protect_enable: whether to enable the color protect module. Used in LTM2.0 and GTM CABC.

- ltm2_mode: the algorithm mode of LTM2.0. Only used in LTM2.0.

drm_vs_ltm_xgamma

Definition

#define VS_LTM_XGAMMA_COEF_NUM	65

struct drm_vs_ltm_xgamma {

bool		enable;

__u32		coef[VS_LTM_XGAMMA_COEF_NUM];

};

Members

- enable: whether to enable the gamma or de-gamma module.

- coef: the 65-point fixed interval LUT data for de-gamma.

drm_vs_ltm_luma

Definition

#define VS_LTM_LUMA_COEF_NUM	5

struct drm_vs_ltm_luma {

bool					enable;

bool					luma_blending_enable;

enum drm_vs_ltm_luma_mode		mode;

__u16					coef[VS_LTM_LUMA_COEF_NUM];

};

Members

- enable: whether to enable the LTM luma calculation module.

- luma_blending_enable:  whether to enable the average luma calculation. Used in LTM2.0 and GTM CABC.

- mode: the LTM luma calculation mode. Set this member to a value listed in the following table.

Table 18. drm_vs_ltm_luma_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_LTM_LUMA_GRAY | 0 | Outputs luma from GRAY calculation. |
| VS_LTM_LUMA_LIGHTNESS | 1 | Outputs luma from LIGHTNESS calculation. |
| VS_LTM_LUMA_MIXED | 2 | Outputs luma from GRAP and LIGHTNESS calculation. |
| VS_LTM_LUMA_PROG | 3 | Outputs luma from programmable GRAP and LIGHTNESS coefficient calculation. |

- coef: the coefficients for LTM luma calculation.

drm_vs_ltm_freq_decomp

Definition

#define VS_LTM_FREQ_COEF_NUM	9

struct drm_vs_ltm_freq_decomp {

bool		decomp_enable;

__u16		coef[VS_LTM_FREQ_COEF_NUM];

__u32		norm;

};

Members

- decomp_enable: whether to enable the luma frequency decomposition module for LTM.

- coef: the coefficients for LTM luma frequency decomposition.

- norm: the LTM luma frequency decomposition normalization parameter.

You can use the bo_helper API drm_vs_get_ltm_norm to obtain the value of norm, where bo_helper is a user-space tool library.

drm_vs_1d_lut

Definition

#define VS_MAX_1D_LUT_ENTRY_CNT	129

struct drm_vs_1d_lut {

bool		enable;

__u32		entry_cnt;

__u32		data[VS_MAX_1D_LUT_ENTRY_CNT];

};

Members

- enable: whether to enable the LTM luma adjustment module.

- entry_cnt: the number of valid entries in the data array.

- data: the coefficients for LTM luma adjustment.

drm_vs_ltm_grid

Definition

struct drm_vs_ltm_grid {

bool		enable;

__u16		width;

__u16		height;

__u16		depth;

__u16		hist_depth;

};

Members

- enable: whether to enable the configuration of the LTM affine grid size.

- width: the LTM affine grid width. Set this member to a value ranging from 1 to 16.

- height: the LTM affine grid height. Set this member to a value ranging from 1 to 16.

- depth: the LTM affine grid depth. Set this member to a value ranging from 4 to 8.

- hist_depth: the LTM Histogram bin numbers.  Set this member to a fixed value 64. Used in LTM2.0 and GTM CABC.

drm_vs_ltm_af_filter

Definition

#define VS_LTM_AFFINE_LUT_NUM		1152

struct drm_vs_ltm_af_filter {

bool		enable;

u16		weight;

u32		slope[VS_LTM_AFFINE_LUT_NUM];

u32		bias[VS_LTM_AFFINE_LUT_NUM];

};

Members

- enable: whether to enable the LTM affine grid filtering module.

- weight: the LTM affine grid filtering weight. Set this member to a value ranging from 0 to 1024.

- slope: the LTM affine slope coefficients.

- bias: the LTM affine bias coefficients.

drm_vs_ltm_af_slice

Definition

#define VS_LTM_AFFINE_SLICE_NUM	2

struct drm_vs_ltm_af_slice {

bool		enable;

__u16		start_pos[VS_LTM_AFFINE_SLICE_NUM];

__u16		scale[VS_LTM_AFFINE_SLICE_NUM];

__u16		scale_half[VS_LTM_AFFINE_SLICE_NUM];

__u8		non_roi_mapping_way;

__u8		block_ratio_hr;

};

Members

- enable: whether to enable the LTM affine grid slicing module.

- start_pos: the start coordinates.

- start_pos[0]: the start X-coordinate

- start_pos[1]: the start Y-coordinate

- scale: the LTM grid scale.

- scale[0]: the X scale

- scale[1]: the Y scale

- scale_half: the LTM grid half scale.

- non_roi_mapping_way: whether to use the nearest block for mapping. Used in LTM2.0 and GTM CABC.

- block_ratio_hr: the norm coefficient of original resolution. Only used in LTM2.0.

drm_vs_ltm_af_trans

Definition

#define VS_LTM_AFFINE_OUT_SCALE_SIZE		17

struct drm_vs_ltm_af_trans {

bool		enable;

__u8		slope_bit;

__u8		bias_bit;

__u16		scale[VS_LTM_AFFINE_OUT_SCALE_SIZE];

};

Members

- enable: whether to enable the LTM local affine transformation module.

- slope_bit: the number of fraction bits for the affine slope.

- bias_bit: the adjusted bias bits.

- scale: the coefficients for scaling the input luma to calculate the minimum output.

drm_vs_ltm_tone_adj

Definition

#define VS_MAX_1D_LUT_ENTRY_CNT	129

struct drm_vs_ltm_tone_adj {

bool		enable;

bool		luma_from;

__u32		entry_cnt;

__u32		data[VS_MAX_1D_LUT_ENTRY_CNT];

};

Members

- enable: whether to enable the LTM global tone adjustment module.

- luma_from: whether to use the local affine transformation output as the luma input.

Set this member to one of the following values:

- True: uses the local affine transformation output.

- False: uses the luma calculation output. In this case, LTM is bypassed.

- entry_cnt: the number of valid entries in the data array.

- data: the LTM global tone adjustment coefficients.

drm_vs_ltm_color

Definition

#define VS_LTM_ALPHA_GAIN_SIZE	121

#define VS_LTM_ALPHA_LUMA_SIZE	129

#define VS_LTM_SATU_CTRL_SIZE		257

struct drm_vs_ltm_color {

bool		enable;

bool		satu_ctrl;

__u16		luma_thresh;

__u16		gain[VS_LTM_ALPHA_GAIN_SIZE];

__u16		luma[VS_LTM_ALPHA_LUMA_SIZE];

__u16		satu[VS_LTM_SATU_CTRL_SIZE];

};

Members

- enable: whether to enable the LTM color processing module.

- satu_ctrl: whether to enable saturation control.

- luma_thresh: the input luma threshold.

- gain: the LTM alpha gain LUT.

- luma: the LTM alpha luma LUT.

- satu: the LTM saturation control LUT.

drm_vs_ltm_dither

Definition

#define VS_LTM_DITHER_COEF_NUM	3

struct drm_vs_ltm_dither {

bool		enable;

__u32		table_low[VS_LTM_DITHER_COEF_NUM];

__u32		table_high[VS_LTM_DITHER_COEF_NUM];

};

Members

- enable: whether to enable the LTM dither module.

- table_low: the dither table low values for LTM.

- table_high: the dither table high values for LTM.

drm_vs_ltm_cd_set

Definition

#define VS_LTM_CD_COEF_NUM		4

#define VS_LTM_CD_THRESH_NUM		4

#define VS_LTM_CD_SLOPE_NUM		2

struct drm_vs_ltm_cd_set {

bool		enable;

bool		overlap;

__u32		min_wgt;

__u32		filt_norm;

__u32		coef[VS_LTM_CD_COEF_NUM];

__u32		thresh[VS_LTM_CD_THRESH_NUM];

__u32		slope[VS_LTM_CD_SLOPE_NUM];

};

Members

- enable: whether to enable the LTM content detection module.

- overlap: whether to enable the overlapping function of the content detection histogram.

- min_wgt: the minimum weight for content detection. Set this member to a value ranging from 0 to 16.

- filt_norm: the filter normalization value for content detection.

- coef: the content detection coefficients.

- thresh: the thresholds for the content detection histogram.

- slope: the slopes for the content detection histogram.

You can use the bo_helper API drm_vs_get_ltm_cd_params to obtain the values of members in the drm_vs_ltm_cd_set structure, where bo_helper is a user-space tool library.

drm_vs_ltm_hist_set

Definition

#define VS_LTM_HIST_POS_NUM		2

#define VS_LTM_HIST_SCALE_NUM		2

struct drm_vs_ltm_hist_set {

bool		enable;

bool		overlap;

__u32		grid_depth;

__u32		start_pos[VS_LTM_HIST_POS_NUM];

__u32		grid_scale[VS_LTM_HIST_SCALE_NUM];

__u32		hist_overlap_ratio;

__u32		block_ration_lr;

};

Members

- enable: whether to enable the LTM local histogram module.

- overlap: whether to enable the overlapping function of the local histogram.

- grid_depth: the grid depth for histogram. Set this member to a value ranging from 1 to 64.

- start_pos: the start coordinates for histogram.

- grid_scale: the grid scaling ratio for histogram.

- hist_overlap_ratio: the overlap ratio during histogram statistics. Only used in LTM2.0.

- block_ration_lr: the norm coefficient of downscale resolution. Used in LTM2.0 and GTM CABC.

drm_vs_ltm_ds

Definition

struct drm_vs_ltm_ds {

bool			enable;

__u32			h_norm;

__u32			v_norm;

__u32			crop_l;

__u32			crop_r;

__u32			crop_t;

__u32			crop_b;

struct drm_vs_rect	output;

__u32			roi_x_start;

__u32			roi_y_start;

__u32			roi_x_end;

__u32			roi_y_end;

__u32			roi_x_len;

__u32			roi_y_len;

};

Members

- enable: whether to enable the LTM downscaling module.

- h_norm: the normalization factor applied to the box filter for horizontal downscaling.

- v_norm: the normalization factor applied to the box filter for vertical downscaling.

- crop_l: the downscaling cropping offset from the left.

- crop_r: the downscaling cropping offset from the right.

- crop_t: the downscaling cropping offset from the top.

- crop_b: the downscaling cropping offset from the bottom.

- output: the output size for downscaling. For details, see Section 3.2.1, drm_vs_rect.

- roi_x_start:  the downscaling ROI x coordinate of start point. Used in LTM2.0 and GTM CABC.

- roi_y_start:  the downscaling ROI y coordinate of start point. Used in LTM2.0 and GTM CABC.

- roi_x_end: the downscaling ROI x coordinate of end point. Used in LTM2.0 and GTM CABC.

- roi_y_end: the downscaling ROI y coordinate of end point. Used in LTM2.0 and GTM CABC.

- roi_x_len:  the width of downscaling ROI. Only used in LTM2.0.

- roi_y_len: the height of downscaling ROI. Only used in LTM2.0.

You can use the bo_helper API drm_vs_get_ltm_ds_params to obtain the values of members in the drm_vs_ltm_ds structure, where bo_helper is a user-space tool library.

drm_vs_ltm_hist_get

Definition

#define VS_LTM_HIST_RESULT_NUM	9216

struct drm_vs_ltm_hist_get {

bool		enable;

__u32		hist_bo_handle;

__u32		fd;

__u32		result[VS_LTM_HIST_RESULT_NUM];

};

Members

- enable: indicates whether the LTM local histogram module is enabled.

- hist_bo_handle: the buffer object handle for writing back the LTM local histogram through the WDMA.

- fd: the file descriptor of the drm device.

- result: the LTM local histogram read from the related registers and returned through the APB interface.

drm_vs_ltm_af_lut

Definition

struct drm_vs_ltm_af_lut {

bool			enable;

__u16			entry_cnt;

__u16			ltm_af_lut[1088];

};

Members

- enable: whether to enable the affine lookup table configuration. Used in LTM2.0 and GTM CABC.

- entry_cnt: the number of entries in the lookup table. Used in LTM2.0 and GTM CABC.

- ltm_af_lut: the affine lookup table for LTM. Used in LTM2.0 and GTM CABC.

drm_vs_ltm_sre_wopr

Definition

struct drm_vs_ltm_sre_wopr {

bool			blending_enable;

__u8			wopr_thr;

};

Members

- blending_enable: whether to enable the LTM wopr module.  Used in LTM2.0 and GTM CABC.

- wopr_thr: the threshold of luma.  Used in LTM2.0 and GTM CABC.

drm_vs_ltm_sd

Definition

struct drm_vs_ltm_sd {

bool			blending_enable;

__u8			r_thr;

__u8			g_thr;

__u8			b_thr;

__u8			delta_minmax_thr;

__u8			delta_rg_thr;

};

Members

- blending_enable: whether to enable the LTM scan detection module. Used in LTM2.0 and GTM CABC.

- r_thr: the r channel threshold of scan detection. Used in LTM2.0 and GTM CABC.

- g_thr: the g channel threshold of scan detection. Used in LTM2.0 and GTM CABC.

- b_thr: the b channel threshold of scan detection. Used in LTM2.0 and GTM CABC.

- delta_minmax_thr: the absolute value threshold between the r channel and the g channel of the scan detection module. Used in LTM2.0 and GTM CABC.

- delta_rg_thr: the delta rgb threshold of scan detection, where delta_rgb=max(r,g,b)-min(r,g,b). Used in LTM2.0 and GTM CABC.

drm_vs_ltm_mode

Specifies the algorithm mode of LTM2.0. Set this member to a value listed in the following table.

Table 19. drm_vs_csc_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_LTM_LTM | 0 | Use the LTM algorithm mode |
| VS_LTM_SRE | 1 | Use the SRE algorithm mode |
| VS_LTM_FOSS | 2 | Use the FOSS algorithm mode |

LTM_LUMA_AVE_GET

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the LTM luma average configurations provided through the drm_vs_ltm_luma_ave structure |
| Description | The ID of the blob object set. The default value is 0. When LTM_LUMA_AVE_GET is used to query the LTM luma average result, the luma average level (ave) is read from the related register. The corresponding blob object is then returned to the user mode and accessible by the application. |

LTM_HIST_CD_GET

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the LTM content detection histogram provided through the drm_vs_ltm_cd_get structure |
| Description | The ID of the blob object set. The default value is 0. When LTM_HIST_CD_GET is used to query LTM content detection histogram, the value of result is read from the related register. The corresponding blob object is then returned to the user mode and accessible by the application. |

drm_vs_ltm_cd_get

Definition

#define VS_LTM_CD_RESULT_NUM	256

struct drm_vs_ltm_cd_get {

__u32		result[VS_LTM_CD_RESULT_NUM];

};

Members

- result: the returned LTM content detection histogram.

LTM_LOCAL_HIST_GET

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the LTM local histogram configurations provided through the drm_vs_ltm_hist_get structure |
| Description | The ID of the blob object set. The default value is 0. When LTM_LOCAL_HIST_GET is used to query LTM local histogram, the value of result is read from the related register. The corresponding blob object is then returned to the user mode and accessible by the application. |

drm_vs_ltm_hist_get

Definition

#define VS_LTM_HIST_RESULT_NUM	9216

struct drm_vs_ltm_hist_get {

bool		enable;

__u32		fd;

__u32		hist_bo_handle;

__u32		result[VS_LTM_HIST_RESULT_NUM];

};

Members

- enable: indicates whether the LTM local histogram module is enabled.

- fd: the file descriptor of the drm device.

- hist_bo_handle: the buffer object handle for writing back LTM local histogram through the WDMA.

- result: the LTM local histogram read from the related register and returned through the APB interface.

LTM_HW_READOUT

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the LTM hardware readout configurations provided through the drm_vs_ltm_hw_readout structure |
| Description | The ID of the blob object set. The default value is 0. When LTM_HW_READOUT is used to query LTM readout data, the value of result is read from the related register. The corresponding blob object is then returned to the user mode and accessible by the application. Only used in LTM2.0. |

drm_vs_ltm_hw_readout

Definition

#define VS_LTM2_MAX_W_GRID	8

#define VS_LTM2_MAX_H_GRID	8

#define VS_LTM2_MAX_GRID_NUM	(VS_LTM2_MAX_W_GRID * VS_LTM2_MAX_H_GRID)

#define VS_GTM_HIST_BIN_NUM   64

struct drm_vs_ltm_hw_readout {

__u32		sum_txt;

__u8		avg_wopr[VS_LTM2_MAX_GRID_NUM];

__u8		avg_sd[VS_LTM2_MAX_GRID_NUM];

__u8		avg_luma[VS_LTM2_MAX_GRID_NUM];

__u32 		histogram[VS_GTM_HIST_BIN_NUM];

__u32 		wopr_sum;

__u32 		sd_sum;

bool		read_done;

};

Members

- sum_txt: indicates the number of text point. Only used in LTM2.0.

- avg_wopr: the white point ratio of each block. Only used in LTM2.0.

- avg_sd:  block scan detection ratio. Only used in LTM2.0.

- avg_luma: the average luma of each block. Used in LTM2.0 and GTM CABC.

- read_done:  indicates whether the data has been read completely. Used in LTM2.0 and GTM CABC.

- histogram: the GTM CABC histogram bins read back by APB. Only used in GTM CABC mode.

- wopr_sum: the sum of the white point detection result . Only used in GTM CABC mode.

- sd_sum: the sum of the skin detection result . Only used in GTM CABC mode.

Global Tone Mapping

GTM

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the global tone mapping (GTM) configurations provided through the drm_vs_ltm_xgamma structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_gtm

Definition

struct drm_vs_gtm {

struct drm_vs_ltm_xgamma 		gtm_degamma;

struct drm_vs_ltm_xgamma 		gtm_gamma;

struct drm_vs_ltm_luma 		gtm_luma;

struct drm_vs_ltm_tone_adj 		tone_adj;

struct drm_vs_ltm_color 		gtm_color;

struct drm_vs_ltm_dither 		gtm_dither;

struct drm_vs_ltm_luma_ave 		gtm_luma_set;

struct drm_vs_ltm_ds 		gtm_ds;

struct drm_vs_ltm_af_luts  		gtm_af_luts;

struct drm_vs_ltm_grid 		grid_size;

struct drm_vs_ltm_hist_set 		gtm_hist_set;

struct drm_vs_ltm_af_slice 		af_slice;

struct drm_vs_ltm_sre_wopr 		sre_wopr;

struct drm_vs_ltm_sd 		gtm_sd;

bool color_protect_			enable;

enum drm_vs_gtm_mode 		gtm_mode;

};

Members

- gtm_degamma: the GTM de-gamma configurations. For details, see Section 4.3.2.1.2, drm_vs_ltm_xgamma.

- gtm_gamma: the GTM gamma configurations. For details, see Section 4.3.2.1.2, drm_vs_ltm_xgamma.

- gtm_luma: the GTM luma calculation configurations. For details, see Section 4.3.2.1.3, drm_vs_ltm_luma.

- tone_adj: the GTM global tone adjustment configurations. For details, see Section 4.3.2.1.10, drm_vs_ltm_tone_adj.

- gtm_color: the GTM color processing configurations. For details, see Section 4.3.2.1.11, drm_vs_ltm_color.

- gtm_dither: the GTM dither configurations. For details, see Section 4.3.2.1.12, drm_vs_ltm_dither.

- gtm_luma_set: the GTM luma average configurations. For details, see Section 3.2.9, drm_vs_ltm_luma_ave.

- gtm_ds: the GTM downscaling configurations. For details, see Section 4.3.2.1.15, drm_vs_ltm_ds.

- gtm_af_luts: the GTM CABC affine LUT configurations. The entry_cnt must be 17. Only used when gtm_mode is VS_GTM_CABC.

- grid_size: the GTM CABC grid width/height and the histogram grid depth. For details, see drm_vs_ltm_grid.

- gtm_hist_set: the GTM CABC histogram configurations, including the histogram grid scale and the block ratio lr. For details, see drm_vs_ltm_hist_set.

- af_slice: the slice affine grid enable and the non-ROI mapping way. For details, see drm_vs_ltm_af_slice.

- sre_wopr: the white point detection blending and threshold. For details, see drm_vs_ltm_sre_wopr.

- gtm_sd: the skin detection blending and thresholds. For details, see drm_vs_ltm_sd.

- color_protect_enable: whether to enable color protect in GTM CABC mode.

- gtm_mode: the GTM mode. Only VS_GTM_CABC is supported by the cid with the gtm cabc capability, and only VS_GTM is supported by the cid without it. For details, see drm_vs_gtm_mode.

drm_vs_gtm_mode

Definition

enum drm_vs_gtm_mode {

VS_GTM = 0,

VS_GTM_CABC = 1,

};

Members

- VS_GTM: the legacy GTM mode, in which the register configuration uses the LTM v1 modules. Only supported by the cid without the gtm cabc capability.

- VS_GTM_CABC: the GTM CABC (content adaptive backlight control) mode, in which the register configuration reuses the LTM2.0 modules and the histogram is read back by APB. Only supported by the cid with the gtm cabc capability (gtm_cabc = 1 in drm_vs_dc_info).

drm_vs_gtm_cabc_out

Definition

struct drm_vs_gtm_cabc_out {

double cabc_duty;

double backlight_reduce_coff;

};

Members

- cabc_duty: the CABC backlight duty cycle calculated by the algorithm. It is used for backlight control and is not configured to the DPU registers.

- backlight_reduce_coff: the backlight reduction coefficient calculated by the algorithm. It is used for backlight control and is not configured to the DPU registers.

Sharpness

SHARPENSS

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the sharpness switch and ink mode configurations provided through the drm_vs_sharpness structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_sharpness

Definition

struct drm_vs_sharpness {

bool					enable;

enum drm_vs_sharpness_ink_mode	ink_mode;

};

Members

- enable: whether to enable the sharpness module.

- ink_mode: the ink mode. Set this member to a value listed in the following table.

Table 19. drm_vs_sharpness_ink_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_SHARPNESS_INK_DEFAULT | 0x0 | The default mode, in which the ink mode is disabled. |
| VS_SHARPNESS_INK_G0 | 0x1 | Shows Gaussian layer 0 at output. |
| VS_SHARPNESS_INK_G1 | 0x2 | Shows Gaussian layer 1 at output. |
| VS_SHARPNESS_INK_G2 | 0x3 | Shows Gaussian layer 2 at output. |
| VS_SHARPNESS_INK_L0 | 0x4 | Shows Laplacian layer 0 at output. |
| VS_SHARPNESS_INK_L1 | 0x5 | Shows Laplacian layer 1 at output. |
| VS_SHARPNESS_INK_L2 | 0x6 | Shows Laplacian layer 2 at output. |
| VS_SHARPNESS_INK_ADAPT | 0x7 | Shows the color adaptive region. |
| VS_SHARPNESS_INK_V0 | 0x8 | Shows variance of layer 0 at output. |
| VS_SHARPNESS_INK_V1 | 0x9 | Shows variance of layer 1 at output. |
| VS_SHARPNESS_INK_V2 | 0xA | Shows variance of layer 2 at output. |
| VS_SHARPNESS_INK_LUMA | 0xB | Shows luma gains. |

SHARPENSS_CSC

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that performs color space conversion (CSC) for sharpness, with coefficients provided through the drm_vs_sharpness_csc structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_sharpness_csc

Definition

#define VS_SHARPNESS_CSC_COEF_NUM	9

#define VS_SHARPNESS_CSC_OFFSET_NUM	3

struct drm_vs_sharpness_csc {

int		y2r_coef[VS_SHARPNESS_CSC_COEF_NUM];

int		r2y_coef[VS_SHARPNESS_CSC_COEF_NUM];

int		y2r_offset[VS_SHARPNESS_CSC_OFFSET_NUM];

int		r2y_offset[VS_SHARPNESS_CSC_OFFSET_NUM];

};

Members

- y2r_coef: the YUV-to-RGB conversion coefficients.

- r2y_coef: the RGB-to-YUV conversion coefficients.

- y2r_offset: the YUV-to-RGB conversion offsets.

- r2y_offset: the RGB-to-YUV conversion offsets.

SHARPNESS_LUMA_GAIN

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that maps input coefficients to output coefficients under control of luma gains, provided through the drm_vs_sharpness_luma_gain structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_sharpness_luma_gain

Definition

#define VS_SHARPNESS_LUMA_GAIN_LUT_ENTRY_NUM	9

struct drm_vs_sharpness_luma_gain {

int		lut[VS_SHARPNESS_LUMA_GAIN_LUT_ENTRY_NUM];

};

Members

lut: the luma gain LUT.

SHARPNESS_LPF

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that performs cascaded vertical low-pass filtering (VLPF) and horizontal low-pass filtering (HLPF), with the configurations provided through the drm_vs_sharpness_lpf structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_sharpness_lpf

Definition

#define VS_SHARPNESS_LPF_COEF_NUM	4

struct drm_vs_sharpness_lpf {

__u32		lpf0_coef[VS_SHARPNESS_LPF_COEF_NUM];

__u32		lpf1_coef[VS_SHARPNESS_LPF_COEF_NUM];

__u32		lpf2_coef[VS_SHARPNESS_LPF_COEF_NUM];

__u32		lpf0_norm;

__u32		lpf1_norm;

__u32		lpf2_norm;

};

Members

- lpf0_coef: the coefficients of low-pass filter 1.

- lpf1_coef: the coefficients of low-pass filter 2.

- lpf2_coef: the coefficients of low-pass filter 3.

- lpf0_norm: the normalization gain of low-pass filter 1.

- lpf1_norm: the normalization gain of low-pass filter 2.

- lpf2_norm: the normalization gain of low-pass filter 3.

SHARPNESS_LPF_NOISE

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that maps input coefficients to output coefficients under adaptive control of the noise model, which is estimated by Y input, with the configurations provided through the drm_vs_sharpness_lpf_noise structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_sharpness_lpf_noise

Definition

#define VS_SHARPNESS_LPF_NOISE_LUT_NUM		9

struct drm_vs_sharpness_lpf_noise {

__u32		lut0[VS_SHARPNESS_LPF_NOISE_LUT_NUM];

__u32		lut1[VS_SHARPNESS_LPF_NOISE_LUT_NUM];

__u32		lut2[VS_SHARPNESS_LPF_NOISE_LUT_NUM];

__u32		luma_strength0;

__u32		luma_strength1;

__u32		luma_strength2;

};

Members

- lut0: the 1D noise LUT of filter 1.

- lut1: the 1D noise LUT of filter 2.

- lut2: the 1D noise LUT of filter 3.

- luma_strength0: the strength entry of filter 1.

- luma_strength1: the strength entry of filter 2.

- luma_strength2: the strength entry of filter 3.

SHARPNESS_LPF_CURVE

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that maps input coefficients to output coefficients with 1D LUTs provided through the drm_vs_sharpness_lpf_curve structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_sharpness_lpf_curve

Definition

#define VS_SHARPNESS_LPF_CURVE_LUT_NUM		13

struct drm_vs_sharpness_lpf_curve {

__u32		lut0[VS_SHARPNESS_LPF_CURVE_LUT_NUM];

__u32		lut1[VS_SHARPNESS_LPF_CURVE_LUT_NUM];

__u32		lut2[VS_SHARPNESS_LPF_CURVE_LUT_NUM];

__u32		master_gain;

};

Members

- lut0: the 1D curve generation LUT of filter 1, including six control nodes and seven slopes.

- lut1: the 1D curve generation LUT of filter 2, including six control nodes and seven slopes.

- lut2: the 1D curve generation LUT of filter 3, including six control nodes and seven slopes.

- master_gain: the sharpness master gain.

SHARPNESS_COLOR_ADAPTIVE

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that performs color confidence detection by using input U/V data, with the color adaptive mode provided through the drm_vs_sharpness_color_adaptive structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_sharpness_color_adaptive

Definition

#define VS_SHARPNESS_CA_MODE_NUM		3

struct drm_vs_sharpness_color_adaptive {

struct drm_vs_sharpness_color_adaptive_mode	mode[VS_SHARPNESS_CA_MODE_NUM];

};

Members

mode: the color adaptive mode. For details, see Section 4.3.4.7.2, drm_vs_sharpness_color_adaptive_mode.

drm_vs_sharpness_color_adaptive_mode

Definition

struct drm_vs_sharpness_color_adaptive_mode {

bool		enable;

__u32		gain;

__u32		theta_center;

__u32		theta_range;

__u32		theta_slope;

__u32		radius_center;

__u32		radius_range;

__u32		radius_slope;

};

Members

- enable: whether to enable the color adaptive mode.

- gain: the gain coefficients.

- theta_center: the Theta center.

- theta_range: the Theta range.

- theta_slope: the Theta slope.

- radius_center: the Radius center.

- radius_range: the Radius range.

- radius_slope: the Radius slope.

SHARPNESS_COLOR_BOOST

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that performs sharpness color boost for U/V components, with configurations provided through the drm_vs_sharpness_color_boost structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_sharpness_color_boost

Definition

struct drm_vs_sharpness_color_boost {

__u32		pos_gain;

__u32		neg_gain;

__u32		y_offset;

};

Members

- pos_gain: the positive gain for sharped Y and original Y.

- neg_gain: the negative gain for sharped Y and original Y.

- y_offset: the offset to adjust the results of sharped Y and original Y.

SHARPNESS_SOFT_CLIP

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that performs soft clip, with configurations provided through the drm_vs_sharpness_soft_clip structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_sharpness_soft_clip

Definition

struct drm_vs_sharpness_soft_clip {

__u32		pos_offset;

__u32		neg_offset;

__u32		pos_wet;

__u32		neg_wet;

};

Members

- pos_offset: the positive offset for soft clipping.

- neg_offset: the negative offset for soft clipping.

- pos_wet: the positive weight for soft clipping.

- neg_wet: the negative weight for soft clipping.

SHARPNESS_DITHER

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that performs bit-depth reduction from RGB U12 to RGB U10 by using 2x2 ordered dither, with the dither table configurations provided through the drm_vs_sharpness_dither structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_sharpness_dither

Definition

struct drm_vs_sharpness_dither {

__u32		table_low[3];

__u32		table_high[3];

};

Members

- table_low: the dither table low RGB values.

- table_high: the dither table high RGB values.

SR2000

SR2000

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9000-SR |
| Data Type | Blob object that contains the RGB-to-YUV conversion, scaler, noise reduction, super resolution, and YUV-to-RGB conversion configurations provided through the drm_vs_sr2000 structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_sr2000

Definition

struct drm_vs_sr2000 {

bool					r2y_enable;

bool					scale_enable;

bool					nr_enable;

bool					ssr3_enable;

bool					y2r_enable;

struct drm_vs_r2y_config		r2y;

struct drm_vs_scale_config		scale;

struct drm_vs_nr_config		nr;

struct drm_vs_ssr3_config		ssr3;

struct drm_vs_y2r_config		y2r;

};

Members

- r2y_enable: whether to enable the RGB-to-YUV conversion module.

- scale_enable: whether to enable the scaler module.

- nr_enable: whether to enable the noise reduction module.

- ssr3_enable: whether to enable the super resolution module.

- y2r_enable: whether to enable the YUV-to-RGB conversion module.

- r2y: performs RGB-to-YUV conversion in SR2000, with coefficients provided through the drm_vs_r2y_config structure.

- scale: performs scaling in SR2000, with coefficients provided through the drm_vs_scale_config structure.

- nr: performs noise reduction in SR2000, with coefficients provided through the drm_vs_nr_config structure.

- ssr3: performs super resolution in SR2000, with coefficients provided through the drm_vs_ssr3_config structure.

- y2r: performs YUV-to-RGB conversion in SR2000, with coefficients provided through the drm_vs_y2r_config structure.

drm_vs_r2y_config

Definition

#define VS_MAX_R2Y_COEF_NUM		15

struct drm_vs_r2y_config {

enum drm_vs_csc_mode		mode;

enum drm_vs_csc_gamut	gamut;

__s32				coef[VS_MAX_R2Y_COEF_NUM];

__u32 				output_bus_format;

};

Members

- mode: the color space conversion (CSC) mode. For details, see Section 3.1.1, drm_vs_csc_mode.

- gamut: the color gamut. For details, see Section 3.1.2, drm_vs_csc_gamut.

- coef[VS_MAX_R2Y_COEF_NUM]: the user-defined coefficients.

- coef[0] to coef[8]: coefficients 00 to 22 in the S7.12 format (1 signed bit, 6 integer bits, and 12 fraction bits).

- coef[9] to coef[11]: pre-offsets D0 to D2. Each offset has 13 bits (1 signed bit and 12 integer bits).

- coef[12] to coef[14]: post-offsets D0-D2. Each offset has 13 bits (1 signed bit and 12 integer bits).

- output_bus_format: the output format of debugging write-back data.

The output format is usually determined by the encoder, but the default output format for the DPU DRM driver is MEDIA_BUS_FMT_RGB888_1X24.

drm_vs_nr_config

Definition

#define VS_NR_COEF_NUM	25

struct drm_vs_nr_config {

__u32		coef[VS_NR_COEF_NUM];

};

Members

coef: the noise reduction coefficient corresponding to two noise reduction modes.

The built-in noise reduction modes NR_weak and NR_strong are available for adjusting the denoising strength on images.

drm_vs_ssr3_config

Definition

#define VS_SSR3_COEF_NUM	268

struct drm_vs_ssr3_config {

__u32		DepHWid;

__u32		DepVWid;

int		HWidRatio;

int		VWidRatio;

int		HRatioSft;

int		VRatioSft;

__u32		coef[VS_SSR3_COEF_NUM];

};

Members

- DepHWid: The value is fixed to 0x30.

- DepVWid: The value is fixed to 0x22.

- HWidRatio: the ratio calculated by placing the DepHWid and out_width values in the [ssr3_get_ratio_and_shift] formula.

- VWidRatio: the ratio calculated by placing the DepVWid and out_height values in the [ssr3_get_ratio_and_shift] formula.

- HRatioSft: the ratio calculated by placing the DepHWid and out_width values in the [ssr3_get_ratio_and_shift] formula.

- VRatioSft: the ratio calculated by placing the DepVWid and out_height values in the [ssr3_get_ratio_and_shift] formula.

- coef: the super resolution coefficients corresponding to seven super resolution modes.

Seven built-in modes are available for image sharpening in different scenarios: strongUps, TV1, TV2, TV3, DESK, GAME, and NATURE.

drm_vs_y2r_config

Definition

#define VS_MAX_Y2R_COEF_NUM		15

struct drm_vs_y2r_config {

enum drm_vs_csc_mode		mode;

enum drm_vs_csc_gamut	gamut;

__s32				coef[VS_MAX_Y2R_COEF_NUM];

};

Members

- mode: the color space conversion (CSC) mode. For details, see Section 3.1.1, drm_vs_csc_mode.

- gamut: the color gamut. For details, see Section 3.1.2, drm_vs_csc_gamut.

- coef: the YUV-to-RGB conversion coefficients.

  - coef[0] to coef[8]: coefficients 00 to 22, in the S7.12 format (1 signed bit, 6 integer bits, 12 fraction bits).

  - coef[9] to coef[11]: pre-offsets D0 to D2. Each offset has 13 bits (1 signed bit and 12 integer bits).

  - coef[12] to coef[14]: post-offsets D0 to D2. Each offset has 13 bits (1 signed bit and 12 integer bits).

Color Conversion Matrix Before De-gamma

CCM_NON_LNR

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the matrix configurations for color conversion before de-gamma, provided through the drm_vs_ccm structure |
| Description | The ID of the blob object set. The default value is 0. |

De-Gamma

DEGAMMA

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the de-gamma LUT configurations provided through the drm_vs_xstep_lut structure |
| Description | The ID of the blob object set. The default value is 0. |

Brightness Compensation

BRIGHTNESS

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the brightness configurations provided through the drm_vs_brightness structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_brightness

Definition

struct drm_vs_brightness {

__u16					target;

__u16					threshold;

__u16					luma_coef[3];

struct drm_vs_brightness_roi	roi0;

struct drm_vs_brightness_roi	roi1;

struct drm_vs_brightness_mask	mask;

};

Members

- target: the target gain, in the U1.10 format.

- threshold: the protection threshold, in the U14 format.

- luma_coef: the coefficients used to calculate the luma value when mode is set to VS_DATA_CALC_WEIGHT.

- luma_coef[0]: coef_R, the coefficient of the red value.

- luma_coef[1]: coef_G, the coefficient of the green value.

- luma_coef[2]: coef_B, the coefficient of the blue value.

Make sure that the sum of the three coefficients is 1024.

- roi0: the brightness ROI 0. For details, see Section 4.3.8.1.2, drm_vs_brightness_roi.

- roi1: the brightness ROI 1. For details, see Section 4.3.8.1.2, drm_vs_brightness_roi.

- mask:The ID of the frame buffer object that stores brightness mask data.

drm_vs_brightness_roi

Definition

struct drm_vs_brightness_roi {

bool 				roi_enable;

struct drm_vs_rect		rect;

};

Members

- roi_enable: whether to enable brightness ROI.

- rect: the brightness ROI. For details, see Section 3.2.1, drm_vs_rect.

drm_vs_brightness_mask

Definition

struct drm_vs_brightness_mask {

enmu drm_vs_brightness_cal_mode		mode;

__u32						mask_fb_id;

};

Members

- mode: the luma calculation method. Set this member to a value listed in the following table.

Table 20. drm_vs_brightness_cal_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_BRIGHTNESS_CAL_MODE_WEIGHT | 0 | Derives from the weighted sum of the red, green, and blue values by using the following formula: (R×coef_R+G×coef_G+B×coef_B+512)>>10 |
| VS_BRIGHTNESS_CAL_MODE_ MAX | 1 | Derives from the maximum of the red, green, and blue values. |
| VS_BRIGHTNESS_CAL_MODE_COUNT | 2 | The number of brightness luma modes. |

- mask_fb_id: the ID of the frame buffer that stores brightness mask data.

Color Conversion Matrix After De-gamma

CCM_LNR

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the matrix configurations for color conversion after de-gamma, provided through the drm_vs_ccm structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_ccm

Definition

struct drm_vs_ccm {

__s32		coef[9];

__s32		offset[3];

};

Members

- coef: coefficients 00 to 22, in the S2.11 format (1 signed bit, 1 integer bit, 11 fraction bits).

- offset: offsets 0 to 2, in the S15.0 format (bit 14 is a signed bit and bits 13:0 are integer bits).

Color Conversion Matrix Extension

CCM_LNR_EX

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the configurations for color conversion matrix extension, provided through the drm_vs_ccm structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_ccm

Definition

struct drm_vs_ccm {

__s32		coef[9];

__s32		offset[3];

};

Members

- coef: coefficients 00 to 22, in the S2.11 format (1 signed bit, 1 integer bit, and 11 fraction bits).

- offset: offsets 0 to 2, in the S15.0 format (bit 14 is a signed bit and bits 13:0 are integer bits).

Color Calibration Path Mode

COLOR_CALIBRATION_PATH_MODE0

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Enum object that defines the data path mode for color calibration, provided through the drm_vs_color_calibration_path_mode enumeration |
| Description | The ID of the enum object. The default value is 0. |

drm_vs_color_calibration_path_mode

Table 21. drm_vs_color_calibration_path_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| ADP_COLOR_MODE_MODE0 | 0 | Mode 0 in which ROI data is transferred to the ROI-based 3D LUT module, and other data is transferred to the color conversion matrix extension module. |
| ADP_COLOR_MODE_MODE1 | 1 | Mode 1 in which all data is transferred to the ROI-based 3D LUT module. |

3D LUT

PRIOR_3DLUT

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series and DC8x00 series |
| Data Type | An array of blob objects each of which contains a prior 3D LUT entry provided through the drm_vs_color structure. The number of blob objects in the array is 4913, as specified by the VS_MAX_PRIOR_3DLUT_SIZE macro. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_color

Definition

struct drm_vs_color {

__u32		a;

__u32		r;

__u32		g;

__u32		b;

};

Members

- a: reserved for 3D LUT.

- r: an LUT value of the red channel. Each value is U14 valid at LSB.

- g: an LUT value of the green channel. Each value is U14 valid at LSB.

- b: an LUT value of the blue channel. Each value is U14 valid at LSB.

ROI0_3DLUT

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the ROI 3D LUT configurations provided through the drm_vs_roi_lut_config structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_roi_lut_config

Definition

#define VS_MAX_ROI_3DLUT_SIZE		729

struct drm_vs_roi_lut_config {

struct drm_vs_rect		rect;

struct drm_vs_color		data[VS_MAX_ROI_3DLUT_SIZE];

};

Members

- rect: the ROI. For details, see Section 3.2.1, drm_vs_rect.

- data: the entries in the 3D LUT of the ROI.

The number of entries is 9 × 9 × 9 = 729, as specified by the VS_MAX_ROI_3DLUT_SIZE macro.

Each entry is specified using the drm_vs_color structure. For structure details, see Section 4.3.12.2.2, drm_vs_color.

drm_vs_color

Definition

struct drm_vs_color {

__u32		a;

__u32		r;

__u32		g;

__u32		b;

};

Members

- a: reserved for 3D LUT.

- r: an LUT value of the red channel. Each value is U14 valid at LSB.

- g: an LUT value of the green channel. Each value is U14 valid at LSB.

- b: an LUT value of the blue channel. Each value is U14 valid at LSB.

Data Conversion

DATA_EXTEND

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the bit dept extension mode of RGB or YUV components through the drm_vs_data_extend structure This bit depth extension feature extends pixel components by padding extra bits as lowest bits. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_data_extend

Definition

struct drm_vs_data_extend {

enum drm_vs_data_extend_mode	data_extend_mode;

struct drm_vs_alpha_data_extend	alpha_data_extend;

};

Members

- data_extend_mode: the bit depth extension mode. For details, see Section 3.1.7, drm_vs_data_extend_mode.

- alpha_data_extend: reserved.

DATA_TRUNC

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the data truncation configurations provided through the drm_vs_data_trunc structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_data_trunc

Definition

struct drm_vs_data_trunc{

enum drm_vs_data_trunc_mode		gamma_data_trunc;

enum drm_vs_data_trunc_mode		panel_data_trunc;

enum drm_vs_data_trunc_mode		blend_data_trunc;

enum drm_vs_data_trunc_mode		wb_data_trunc;

};

Members

Set each of the following members to a value listed in Table 22.

- gamma_data_trunc: the mode of data truncation after the gamma dither module.

This member is valid only when the gamma dither module is disabled.

- panel_data_trunc: the mode of data truncation after the panel dither module.

This member is valid only when the panel dither module is disabled.

- blend_data_trunc: the mode of data truncation after the blend dither module.

This member is valid only when the blend dither module is disabled.

- wb_data_trunc:  the mode of data truncation after the write-back module.

This member is valid only when the write-back dither module is disabled.

Table 22. drm_vs_data_trunc_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_DATA_TRUNCATE | 0 | Reduces bits by truncating the lower bits. |
| VS_DATA_ROUNDING | 1 | Reduces bits by rounding. |

Dither

GAMMA_DITHER, PANEL_DITHER, and DBI_DITHER

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the gamma dither, panel dither, and DBI dither configurations provided through the drm_vs_dither structure |
| Description | The ID of the blob object set. The default value is 0. |

LLV_DITHER

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the low-level dither configurations provided through the drm_vs_llv_dither structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_llv_dither

Definition

struct drm_vs_llv_dither {

enum drm_vs_dth_frm_idx			idx_type;

__u8						sw_index;

__u16						start_x;

__u16						start_y;

__u16						mask;

__u16						threshold;

__u16						linear_threshold;

struct drm_vs_random_dither_seed		seed;

};

Members

- idx_type: the frame index type. For details, see Section 3.1.3, drm_vs_dth_frm_idx.

- sw_index: the frame index.

This member is valid only if idx_type is set to VS_DTH_FRM_IDX_SW.

- start_x: the start X-coordinate.

- start_y: the start Y-coordinate.

- mask: the mask used to XOR with the value generated by PRNG.

- threshold: the low-level dither threshold.

- linear_threshold: the linear threshold for low-level dither.

- seed: the random seed configurations for blend dither. For details, see Section 4.3.14.2.2, drm_vs_random_dither_seed.

drm_vs_random_dither_seed

Definition

#define VS_RANDOM_DITHER_SEED_NUM	8

struct drm_vs_random_dither_seed {

bool		hash_seed_x_enable;

bool		hash_seed_y_enable;

bool		permut_seed1_enable;

bool		permut_seed2_enable;

__u32		hash_seed_x[VS_RANDOM_DITHER_SEED_NUM];

__u32		hash_seed_y[VS_RANDOM_DITHER_SEED_NUM];

__u32		permut_seed1[VS_RANDOM_DITHER_SEED_NUM];

__u32		permut_seed2[VS_RANDOM_DITHER_SEED_NUM];

};

Members

- hash_seed_x_enable: indicates whether the hash seed X-coordinates are valid.

If the hash seed X-coordinates are valid, hash_seed_x is parsed and you need to configure the related registers.

- hash_seed_y_enable: indicates whether the hash seed Y-coordinates are valid.

If the hash seed Y-coordinates are valid, hash_seed_y is parsed and you need to configure the related registers.

- permut_seed1_enable: indicates whether permutation seed array 1 is valid.

If permutation seed array 1 is valid, permut_seed1 is parsed and you need to configure the related registers.

- permut_seed2_enable: indicates whether permutation seed array 2 is valid.

If permutation seed array 2 is valid, permut_seed2 is parsed and you need to configure the related registers.

- hash_seed_x: the array of hash seed X-coordinates.

The array requires eight entries. The default value is [45031, 47417, 37483, 43973, 37517, 45319, 41203, 49261].

This member is valid only if hash_seed_x_en is set to 1.

- hash_seed_y: the array of hash seed Y-coordinates.

The array requires eight entries. The default value is [14463, 56533, 24659, 16843, 20789, 22447, 18371, 32503].

This member is valid only if hash_seed_y_en is set to 1.

- permut_seed1: the permutation seed array 1.

The array requires eight entries. The default value is [30340, 60458, 19992, 32054, 45828, 55594, 31668, 17290].

This member is valid only if permut_seed1_en is set to 1.

- permut_seed2: the permutation seed array 2.

The array requires eight entries. The default value is [27779, 27179, 30013, 29179, 29363, 30529, 27253, 23599].

This member is valid only if permut_seed2_en is set to 1.

DITHER_ENABLED

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC8x00 series |
| Data Type | Boolean |
| Description | Enables or disables panel dithering. The default value is False. |

Gamma

PRIOR_GAMMA

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the prior gamma LUT configurations provided through the drm_vs_gamma_lut structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_gamma_lut

Definition

#define VS_MAX_LUT_SEG_CNT 10

#define VS_MAX_GAMMA_ENTRY_CNT 257

struct drm_vs_gamma_lut {

struct drm_vs_color		seg_cnt;

struct drm_vs_color		seg_point[VS_MAX_LUT_SEG_CNT - 1];

struct drm_vs_color		seg_step[VS_MAX_LUT_SEG_CNT];

struct drm_vs_color		entry_cnt;

struct drm_vs_color 		data[VS_MAX_GAMMA_ENTRY_CNT];

};

Members

- seg_cnt: the number of segments.

The maximum segment number is 10 for DC9400 and 8 for DC9500.

- seg_point: the arrays of segment points except for the first and last points.

The first point is fixed at 0 and the last point is fixed at the maximum, which is not configurable.

The difference between two neighboring segment points (seg_point[N+1] – seg_point[N]) must be a power of 2.

Each point is U14 valid at LSB for DC9400 and U16 valid at LSB for DC9500.

- seg_step: the per-segment X step array.

The X step for each segment must be a power of 2.

Each X step is U14 valid at LSB for DC9400 and U16 valid at LSB for DC9500.

- entry_cnt: the number of entries, which is 257 for output panels 0 to 3 of DC9400 and 129 for output panels 0 to 3 of DC9500.

- data: the per-entry LUT value array.

The number of entries is 257, as specified by the VS_MAX_GAMMA_ENTRY_CNT macro.

Each entry is specified using the drm_vs_color structure. For structure details, see Section 4.3.15.1.2, drm_vs_color.

drm_vs_color

Definition

struct drm_vs_color {

__u32		a;

__u32		r;

__u32		g;

__u32		b;

};

Members

- a: reserved.

- r: an LUT value of the red channel. Each value is U14 valid at LSB.

- g: an LUT value of the green channel. Each value is U14 valid at LSB.

- b: an LUT value of the blue channel. Each value is U14 valid at LSB.

Blur Compensation

BLUR

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the blur configurations provided through the drm_vs_blur structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_blur

Definition

#define VS_BLUR_COEF_NUM 4

struct drm_vs_blur {

struct drm_vs_rect 	roi;

__u8 			coef[3][VS_BLUR_COEF_NUM];

__u32 			norm[3];

__u8			coef_num;

__u32			mask_fb_id;

};

Members

- roi: the blur ROI. For details, see Section 3.2.1, drm_vs_rect.

- coef: the coefficients of red, green, and blue channels in horizontal direction for the blur module.

- coef_h[0][0] to coef_h[0][3]: the coefficients of the red channel

- coef_h[1][0] to coef_h[1][3]: the coefficients of the green channel

- coef_h[2][0] to coef_h[2][3]: the coefficients of the blue channel

- norm: the weights of red, green, and blue channels for the horizontal filter of the blur module in the case with normalization.

- norm[0]: the weight of the red channel

- norm[1]: the weight of the green channel

- norm[2]: the weight of the blue channel

- coef_num: the number of entries in the coef array.

Entries coef[3][0] to coef[3][coef_num-1] are valid.

- mask_fb_id: the ID of the frame buffer that stores blur mask data.

Rounded Corner Display

RCD

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the rounded corner display (RCD) configurations provided through the drm_vs_rcd structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_rcd

Definition

struct drm_vs_rcd {

struct	 drm_vs_rcd_mask		mask;

struct drm_vs_rcd_roi		roi;

struct drm_vs_rcd_bg 		background;

};

Members

- mask: the RCD mask blending type and the ID of the frame buffer that stores the RCD mask data.

- roi: the RCD ROI.

- background: the background color of the RCD ROI.

drm_vs_rcd_mask

Definition

struct drm_vs_rcd_mask {

enum drm_vs_mask_blend_type		type;

__u32					mask_fb_id;

};

Members

- type: the blending type of the RCD mask. Set this member to a value listed in the following table.

Table 23. drm_vs_mask_blend_type Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_MASK_BLD_NORM | 0 | Does not inverse the RCD mask. |
| VS_MASK_BLD_INV | 1 | Inverses the RCD mask. |
| VS_MASK_BLD_COUNT | 2 | Indicates the number of RCD mask types. |

- mask_fb_id: the ID of the frame buffer that stores the RCD mask.

drm_vs_rcd_bg

Definition

struct drm_vs_rcd_bg {

__u32 			pnl_color;

bool			roi_enable;

__u32			roi_color;

struct drm_vs_rect	roi_rect;

};

Members

- pnl_color: the RCD background color of the panel.

- roi_enable: whether to enable the RCD background ROI.

- roi_color: the color of the RCD background ROI.

- roi_rect: the RCD background ROI. For details, see Section 3.2.1, drm_vs_rect.

drm_vs_rcd_roi

Definition

struct drm_vs_rcd_roi {

bool 				top_enable;

bool				btm_enable;

struct drm_vs_rect		top_roi;

struct drm_vs_rect		btm_roi;

};

Members

- top_enable: whether to enable the top RCD ROI.

- btm_enable: whether to enable the bottom RCD ROI.

- top_roi: the top RCD ROI. For details, see Section 3.2.1, drm_vs_rect.

- btm_roi: the bottom RCD ROI. For details, see Section 3.2.1, drm_vs_rect.

On Pixel Sum

OPS

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the on pixel sum calculation (OPS) configurations provided through the drm_vs_ops structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_ops

Definition

struct drm_vs_ops {

__u16 lut[256];

struct drm_vs_ops_roi0		roi0;

struct drm_vs_ops_roi1		roi1;

};

Members

- lut: lookup table (LUT) for calculating the R, G, and B channel values. This table contains 256 entries. Each entry is a 16-bit unsigned integer, but only bits [0:9] are valid.

- roi0: direct operation ROI.

- roi1: weighted operation ROI.

drm_vs_ops_roi0

Definition

struct drm_vs_ops_roi0 {

bool			roi_enable;

bool			roi_linear;

struct	 drm_vs_rect	rect;

};

Members

- roi_enable: whether to enable direct operation ROI.

- roi_linear: whether to use LUT table for the ROI.

- drm_vs_rect: ROI coordinates and dimensions. For details, see Section 3.2.1, drm_vs_rect.

drm_vs_ops_roi1

Definition

#define VS_OPS_ROI_WEIGHT_CNT 2500

struct drm_vs_ops_roi1 {

bool		roi_enable;

__u16 		x;

__u16		y;

__u16		index;

__u32 		weight[VS_OPS_ROI_WEIGHT_CNT];

};

Members

- roi_enable: whether to enable weighted operation ROI.

- x: the X-coordinate of the top-left corner of the ROI

- y: the Y-coorindate of the top-left corner of the ROI.

- index: the index of the weighted operation ROI.

- weight: the weight coefficients for ROI regions.

OPS_RESULT

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the on pixel sum (OPS) calculation results provided through the drm_vs_ops_result structure. |
| Description | The ID of the blob object. The blob object is created in the driver initialization phase and an immutable blob ID can be obtained. |

drm_vs_ops_result

Definition

struct drm_vs_ops_result {

struct drm_vs_ops_roi_result		roi0_ops_res;

struct drm_vs_ops_roi_result		roi1_ops_res;

};

Members

- roi0_ops_res: the calculation result of the direct operation ROI. For details, see Section 4.3.18.2.2, drm_vs_ops_roi_result.

- roi1_ops_res: the calculation result of the weighted operation ROI. For details, see Section 4.3.18.2.2, drm_vs_ops_roi_result.

drm_vs_ops_roi_result

Definition

struct drm_vs_ops_roi_result {

__u64 ops_red_sum;

__u64 ops_green_sum;

__u64 ops_blue_sum;

};

Members

- ops_red_sum: the calculatoin result of the red channel.

- ops_green_sum: the calculation result of the green channel.

- ops_blue_sum: the calculation result of the blue channel.

Note: When OPS is enabled, OPS_RESULT is automatically updated with the OPS calculation results at the end of each frame.

Burn-in Compensation

DBI

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the DBI configurations provided through structures drm_vs_dbi, drm_vs_dbi_main_roi, drm_vs_dbi_udc, drm_vs_position, and drm_vs_dbi_dither. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_dbi

Definition

struct drm_vs_dbi {

struct drm_vs_buffer_object main_lut_bo;

struct drm_vs_buffer_object udc0_lut_bo;

struct drm_vs_buffer_object udc1_lut_bo;

struct drm_vs_buffer_object collect_bo;

struct drm_vs_dbi_main_roi main_roi0;

struct drm_vs_dbi_main_roi main_roi1;

struct drm_vs_dbi_main_roi main_roi2;

struct drm_vs_dbi_udc udc0;

struct drm_vs_dbi_udc udc1;

struct drm_vs_dbi_dither dither;

};

Members

- main_lut_bo: the buffer object that stores the lookup table of the main ROIs.

- udc0_lut_bo: the buffer object that stores the lookup table of UDC 0.

- udc1_lut_bo: the buffer object that stores the lookup table of UDC 1.

- collect_bo: the buffer object that stores the result of DBI.

- main_roi0: controls the main ROI 0.

- main_roi1: controls the main ROI 1.

- main_roi2: controls the main ROI 2.

For details about main_roi{n} (n = 0 to 2), see Section 4.3.19.1.2, drm_vs_dbi_main_roi.

- udc0: controls the UDC 0.

- udc1: controls the UDC 1.

For details about udc0 and udc1, see Section 4.3.19.1.3, drm_vs_dbi_udc.

- dither: controls the DBI dither.

For details, see Section 4.3.19.1.5, drm_vs_dbi_dither.

drm_vs_dbi_main_roi

Definition

struct drm_vs_dbi_main_roi {

bool enable;

struct drm_vs_rect roi_rect;

};

Members

- enable: whether to enable the main ROI.

- roi_rect: the coordinates, width, and height of the main ROI.

For details, see Section 3.2.1, drm_vs_rect.

drm_vs_dbi_udc

Definition

struct drm_vs_dbi_udc {

bool enable;

struct drm_vs_position pos;

};

Members

- enable: whether to enable the UDC.

- pos: the UDC coordinates.

For details, see Section 4.3.19.1.4, drm_vs_position.

drm_vs_position

Definition

struct drm_vs_position {

__u16 x;

__u16 y;

};

Members

- x: the coordinate on the horizontal axis.

- y: the coordinate on the vertical axis.

drm_vs_dbi_dither

Definition

struct drm_vs_dbi_dither {

bool enable;

struct drm_vs_dither dither;

};

Members

- enable: whether to enable dither.

- dither: dither configurations. For details, see Section 3.2.6, drm_vs_dither.

DBI_CORRECTION_COEF

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the DBI correction coeffient configurations provided through structures drm_vs_dbi_correction_coefs and drm_vs_dbi_correction_coef. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_dbi_correction_coefs

Definition

struct drm_vs_dbi_correction_coefs {

struct drm_vs_dbi_correction_coef main_roi_coef;

struct drm_vs_dbi_correction_coef udc0_coef;

struct drm_vs_dbi_correction_coef udc1_coef;

};

Members

- main_roi_coef: color compensation coeffients of the main ROI.

- udc0_coef: color compensation coeffients of UDC 0.

- udc1_coef: color compensation coeffients of UDC 1.

For details about the coefficients, see Section 4.3.19.2.2, drm_vs_dbi_correction_coef.

drm_vs_dbi_correction_coef

Definition

#define VS_DBI_MAX_CORRECTION_AREA_CNT	8

struct drm_vs_dbi_correction_coef {

int slope_m_r[VS_DBI_MAX_CORRECTION_AREA_CNT];

int slope_a_r[VS_DBI_MAX_CORRECTION_AREA_CNT];

int intercept_m_r[VS_DBI_MAX_CORRECTION_AREA_CNT];

int intercept_a_r[VS_DBI_MAX_CORRECTION_AREA_CNT];

__U32 area_threshold_r[VS_DBI_MAX_CORRECTION_AREA_CNT];

int slope_m_g[VS_DBI_MAX_CORRECTION_AREA_CNT];

int slope_a_g[VS_DBI_MAX_CORRECTION_AREA_CNT];

int intercept_m_g[VS_DBI_MAX_CORRECTION_AREA_CNT];

int intercept_a_g[VS_DBI_MAX_CORRECTION_AREA_CNT];

__U32 area_threshold_g[VS_DBI_MAX_CORRECTION_AREA_CNT];

int slope_m_b[VS_DBI_MAX_CORRECTION_AREA_CNT];

int slope_a_b[VS_DBI_MAX_CORRECTION_AREA_CNT];

int intercept_m_b[VS_DBI_MAX_CORRECTION_AREA_CNT];

int intercept_a_b[VS_DBI_MAX_CORRECTION_AREA_CNT];

__U32 area_threshold_b[VS_DBI_MAX_CORRECTION_AREA_CNT];

};

Members

- slope_m_r/g/b: the slope multiplier of the red/green/blue channel.

- slope_a_r/g/b: the slope addend of the red/green/blue channel.

- intercept_m_r/g/b: the intercept multiplier of the red/green/blue channel.

- intercept_a_r/g/b: the intercept addend of the red/green/blue channel.

- area_threshold_r/g/b: the brightness interval of the red/green/blue channel.

- Macro VS_DBI_MAX_CORRECTION_AREA_CNT: the number of areas with different correction factors.

Letter Box Appendant

LBOX

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the letter box appendant configurations provided through structures drm_vs_lbox, drm_vs_color, and drm_vs_rect. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_lbox

Definition

struct drm_vs_lbox {

struct drm_vs_color bg_color;

struct drm_vs_rect box_rect;

};

Members

- bg_color: the background color values of the letter box.

For details, see Section 4.3.20.1.2, drm_vs_color.

- box_rect: the rectangular area as the letter box.

For details, see Section 3.2.1, drm_vs_rect.

drm_vs_color

Definition

struct drm_vs_color {

__u32		a;

__u32		r;

__u32		g;

__u32		b;

};

Members

- a: This member is invalid for setting the background color of the letter box.

- r[9:0]: 10-bit red or Y channel value.

- g[9:0]: 10-bit green or U channel value.

- b[9:0]: 10-bit blue or V channel value.

Sub-Pixel Rendering

SPR

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the sub-pixel rendering (SPR) configurations provided through the drm_vs_spr structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_spr

Definition

struct drm_vs_spr {

bool					pq_out_ofi_enable;

enum drm_vs_spr_out_mode		spr_out_mode;

};

Members

- pq_out_ofi_enable: whether the pipeline is open for integration.

- spr_out_mode: the SPR output format. Set this member to a value listed in the following table.

Table 24. drm_vs_spr_out_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_DC_SPR_OUT_MODE422 | 0 | The output format is RGB422. |
| VS_DC_SPR_OUT_MODE444 | 1 | The output format is RGB444. |

Histogram/RGB Histogram V2

HISTOGRAM_0/HISTOGRAM_1/HISTOGRAM_2/HISTOGRAM_3

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the histogram module 0, histogram module 1, histogram module 2, and histogram module 3 configurations provided through structures drm_vs_histogram, drm_vs_hist_bin, drm_vs_hist_out, and drm_vs_rect. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_histogram

Definition

struct drm_vs_histogram {

enum 		drm_vs_hist_pos 	pos;

enum		drm_vs_hist_mode	hist_mode

bool 					read_confirm;

struct 	drm_vs_hist_bin 	bin;

struct 	drm_vs_hist_out 	out;

struct		drm_vs_rect		roi;

struct		drm_vs_rect		block_roi;

};

Members

- pos: the histogram input position. For details, see Section  3.1.5, drm_vs_hist_pos.

- hist_mode: the histogram calculation mode. The available modes are defined as follows:

enum drm_vs_hist_mode {

VS_HIST_NORMAL;

VS_HIST_WEIGHT;

VS_HIST_COUNT;

};

- read_confirm: whether to enable histogram read protection.

- bin: the histogram bin calculation method.

For details, see Section 4.3.22.1.2, drm_vs_hist_bin.

- out: the histogram result output settings.

For details, see Section 3.2.7, drm_vs_hist_out.

- roi: the ROI for the histogram.

For details, see Section 3.2.1, drm_vs_rect.

- block_roi: the blocking region for the histogram inside the ROI.

For details, see Section 3.2.1, drm_vs_rect.

drm_vs_hist_bin

Definition

struct drm_vs_hist_bin {

enum drm_vs_hist_bin_mode 		bin_mode;

__u32					coef[3];

};

Members

- bin_mode: the bin calculation mode. For details, see Section 3.1.6, drm_vs_hist_bin_mode.

- coef: the coefficients used to calculate the binned value when bin_mode is set to VS_DATA_CALC_WEIGHT.

  - coef[0]: REG_HIST_BIN_COEF_R, which is the coefficient of the red value.

  - coef[1]: REG_HIST_BIN_COEF_G, which is the coefficient of the green value.

  - coef[2]: REG_HIST_BIN_COEF_B, which is the coefficient of the blue value.

- Make sure that the sum of the three coefficients is 1024.

HISTOGRAM_RGB

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the RGB histogram configurations provided through the drm_vs_histogram_rgb structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_histogram_rgb

Definition

struct drm_vs_histogram_rgb {

bool 				read_confirm;

struct drm_vs_hist_out 	out;

};

Members

- read_confirm: whether to enable histogram read protection.

- out: the histogram result output settings.

For details, see Section 3.2.7, drm_vs_hist_out.

HISTOGRAM_RESULT

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the all histograms result provided through the drm_vs_histogram_result structure. |
| Description | The ID of the blob object. The blob object is created in the driver initialization phase and an immutable blob ID can be obtained. |

drm_vs_histogram_result

Definition

#define VS_HIST_RESUKT_BIN_CNT 256

struct drm_vs_histogram_result {

__u32 histogram_0_result[VS_HIST_RESULT_BIN_CNT];

__u32 histogram_1_result[VS_HIST_RESULT_BIN_CNT];

__u32 histogram_2_result[VS_HIST_RESULT_BIN_CNT];

__u32 histogram_3_result[VS_HIST_RESULT_BIN_CNT];

__u32 histogram_rgb_result[VS_HIST_RESULT_BIN_CNT * 3];

};

Members

- histogram_0_result: the result of histgoram module 0. This member is valid only when out_mode is set to VS_HIST_BY_APB for histgogram module 0.

- histogram_1_result: the result of histgoram module 1. This member is valid only when out_mode is set to VS_HIST_BY_APB for histgogram module 1.

- histogram_2_result: the result of histgoram module 2. This member is valid only when out_mode is set to VS_HIST_BY_APB for histgogram module 2.

- histogram_3_result: the result of histgoram module 3. This member is valid only when out_mode is set to VS_HIST_BY_APB for histgogram module 3.

- histogram_rgb_result: the result of RGB histgoram. This member is valid only when out_mode is set to VS_HIST_BY_APB for RGB histgogram.

Note: When the specified histogram is enabled and its out_mode is set to VS_HIST_BY_APB, the HISTOGRAM_RESULT blob will be automatically updated with the result of the specified histogram at the end of each frame.

RGB-to-YUV Conversion

R2Y_CONFIG

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series and DC8x00 series |
| Data Type | Blob object that contains the RGB-to-YUV conversion configurations provided through the drm_vs_r2y_config structure. |
| Description | The ID of the blob object set. The default value is 0, which indicates that the DRM driver uses the RGB-to-YUV coefficients defined by ITU-R BT.2020 with a limited YUV color range. |

drm_vs_r2y_config

Definition

#define VS_MAX_R2Y_COEF_NUM    15

struct drm_vs_r2y_config {

enum drm_vs_csc_mode		mode;

enum drm_vs_csc_gamut	gamut;

__s32				coef[VS_MAX_R2Y_COEF_NUM];

__u32				output_bus_format;

};

Members

- mode: the color space conversion (CSC) mode. For details, see Section 3.1.1, drm_vs_csc_mode.

- Note: DC8x00 series only support the VS_CSC_CM_F2L mode.

- gamut: the color gamut. For details, see Section 3.1.2, drm_vs_csc_gamut.

Note: DC8x00 series only support gamuts VS_CSC_CG_601, VS_CSC_CG_709, and VS_CSC_CG_2020.

- coef[VS_MAX_R2Y_COEF_NUM]: the user-defined coefficients.

- For DC9x00 series:

- coef[0] to coef[8]: coefficients 00 to 22 in the S7.12 format (1 signed bit, 6 integer bits, and 12 fraction bits).

- coef[9] to coef[11]: pre-offsets D0 to D2. Each offset has 13 bits (1 signed bit and 12 integer bits).

- coef[12] to coef[14]: post-offsets D0-D2. Each offset has 13 bits (1 signed bit and 12 integer bits).

- For DC8x00 series:

- coef[0] to coef[2]: pre-offsets D0 to D2. Each offset has 9 bits (1 signed bit and 8 integer bits).

- coef[3] to coef[11]: coefficients 00 to 22 in the S7.12 format (1 signed bit, 6 integer bits, and 12 fraction bits).

- coef[12] to coef[14]: post-offsets D0-D2. Each offset has 9 bits (1 signed bit and 8 integer bits).

- output_bus_format: the output format of debugging write-back data.

The output bus format is usually determined by the encoder. The default output bus format is MEDIA_BUS_FMT_RGB888_1X24.

UV Downsampling

DOWN_SAMPLE

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the UV downsampling configurations provided through the drm_vs_ds_config structure |
| Description | The ID of the blob object set. The default value is 0. |

VRR Display Mode

VRR_REFRESH

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC8x00 series |
| Data Type | Blob object that contains the variable refresh rate (VRR) provided through u32. |
| Description | The ID of the blob object set. The default value is 0. |

VRR Compensation

VRR_ENABLED

| Category | Standard CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Boolean |
| Description | Enables or disables variable refresh rate (VRR). The default value is False. If this property is set to True to enable VRR, you need to set vsync_start, vsync_end, and vsync_total in the drmModeModeInfo structure through MODE_ID according to the desired vertical front porch (VFP) of the current frame and the next frame in order to achieve an optimal VRR. |

MODE_ID

| Category | Standard CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the display mode information provided through the drmModeModeInfo structure. |
| Description | The ID of the blob object set. The default value is 0. |

drmModeModeInfo

Definition

#define DRM_DISPLAY_MODE_LEN 32

typedef struct _drmModeModeInfo {

uint32_t	clock;

uint16_t	hdisplay, hsync_start, hsync_end, htotal, hskew;

uint16_t	vdisplay, vsync_start, vsync_end, vtotal, vscan;

uint32_t	flags;

uint32_t	type;

char		name[DRM_DISPLAY_MODE_LEN];

} drmModeModeInfo, *drmModeModeInfoPtr;

Members

- clock: the pixel clock frequency in kHz.

- hdisplay: the horizontal visible area in pixels.

- hsync_start: the horizontal synchronization start.

- hsync_end: the horizontal synchronization end.

- htotal: the horizontal total area in pixels.

- hskew: reserved.

- vdisplay: the vertical visible area in pixels.

- vsync_start: the vertical synchronization start.

- vsync_end: the vertical synchronization end.

- vtotal: the vertical total area in pixels.

- vscan: reserved.

- flags: reserved.

- type: reserved.

- name: the name of the display mode.

Panel Output Cropping

PANEL_CROP

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the cropping size information provided through the drm_vs_panel_crop structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_panel_crop

Definition

struct drm_vs_panel_crop {

struct drm_vs_rect		crop_rect;

__u16 				panel_src_width;

__u16				panel_src_height;

};

Members

- crop_rect: the rectangular area of cropping. For details, see Section 3.2.1, drm_vs_rect.

- panel_src_width: the width of the source image before cropping.

- panel_src_height: the height of the source image before cropping.

Panel Output Splitting

SPLITER

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the output panel vertical splitting configurations provided through the drm_vs_spliter structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_spliter

Definition

struct drm_vs_spliter {

__u32		left_x;

__u32		left_w;

__u32		right_x;

__u32		right_w;

__u32		src_w;

};

Members

- left_x: the left edge of the left sub-image.

- left_w: the width of the left sub-image.

- right_x: the left edge of the right sub-image.

- right_w: the width of the right sub-image.

- src_w: the width of the source image.

RAM-less

RAMLESS_FB

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Null |
| Description | Whether to enable the RAM-less feature. |

OFIFO Splicing

SPLICE_MODE

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the OFIFO splicing mode configurations provided through the drm_vs_splice_mode structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_splice_mode

Definition

struct drm_vs_splice_mode {

bool		splice0_enable;

__u8		splice0_crtc_mask;

__u8		splice0_output_intf;

bool		splice1_enable;

__u8		splice1_crtc_mask;

__u8		splice1_output_intf;

__u16		src_panel_width0;

__u16		src_panel_width1;

};

Members

- splice0_enable: whether to enable splicer 0.

- splice0_crtc_mask: whether to enable each CRTC for splicer 0.

- Bit 0: whether to enable CRTC0 for splicer 0.

- Bit 1: whether to enable CRTC1 for splicer 0.

- Bit 2: whether to enable CRTC2 for splicer 0.

- Bit 3: whether to enable CRTC3 for splicer 0.

Setting a bit to 1 enables the CRTC and setting a bit to 0 disables the CRTC.

Enable two CRTCs for splicer 0.

- splice0_output_intf: the output interface ID for splicer 0.

- splice1_enable: whether to enable splicer 1.

- splice1_crtc_mask: whether to enable each CRTC for splicer 1.

- Bit 0: whether to enable CRTC0 for splicer 1.

- Bit 1: whether to enable CRTC1 for splicer 1.

- Bit 2: whether to enable CRTC2 for splicer 1.

- Bit 3: whether to enable CRTC3 for splicer 1.

- Setting a bit to 1 enables the CRTC and setting a bit to 0 disables the CRTC.

- Enable two CRTCs for splicer 1.

- splice1_output_intf: the output interface ID for splicer 1.

- src_panel_width0: the width of the source image output by panel 0 before splicing.

- src_panel_width1: the width of the source image output by panel 1 before splicing.

Free Synchronization

FREE_SYNC

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the free synchronization configurations provided through the drm_vs_free_sync structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_free_sync

Definition

struct drm_vs_free_sync {

enum   drm_vs_free_sync_type	type;

struct drm_vs_free_sync_mode	mode;

};

Members

- type: indicates whether the members specified in the mode parameter are valid. Set this member to one of the values listed in Table 25.

Table 25. drm_vs_free_sync_type Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_FREE_SYNC_CONFIG | 0 | Only the free_sync_max_delay member is valid. |
| VS_FREE_SYNC_FINISH | 1 | Only the free_sync_finish member is valid. |
| VS_FREE_SYNC_CONFIG_FINISH | 2 | Both of the members are valid. |

- mode: the free synchronization information. For details, see Section 4.3.31.1.2, drm_vs_free_sync_mode.

drm_vs_free_sync_mode

Definition

struct drm_vs_free_sync_mode {

__u16		free_sync_max_delay;

bool		free_sync_finish;

};

Members

- free_sync_max_delay: the timeout period within which the DPU keeps waiting for the GPU to finish rendering a frame. If the GPU finishes rendering a frame within this timeout period, the DPU displays the frame rendered by GPU. If the rendering times out, the DPU still displays the current frame.

- free_sync_finish: indicates whether the GPU finishes rendering a frame.

DP Synchronization

DP_SYNC

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the DP synchronization configurations provided through the drm_vs_dp_sync structure. The DP synchronization can synchronize the clock domain for multiple selected DP interfaces to display images synchronously. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_dp_sync

Definition

struct drm_vs_dp_sync {

__u8		dp_sync_crtc_mask;

bool		duplicate;

};

Members

- dp_sync_crtc_mask: whether to enable the synchronization for each DP interface.

- Bit 0: whether to enable the synchronization for DP interface 0.

- Bit 1: whether to enable the synchronization for DP interface 1.

- Bit 2: whether to enable the synchronization for DP interface 2.

- Bit 3: whether to enable the synchronization for DP interface 3.

Setting a bit to 1 enables the synchronization and setting a bit to 0 disables the synchronization. Enable synchronization for at least two DP interfaces.

- duplicate: whether to enable output duplication.

- 0: no duplication. Each interface displays different content.

- 1: duplication mode. Both interfaces display identical content.

PVRIC Decompressor

PVRIC_FILTER

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Boolean |
| Description | Enables or disables PVRIC filtering. The default value is False. |

PVRIC_CONST

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the PVRIC constant color value provided through the drm_vs_pvric_const structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_pvric_const

Definition

struct drm_vs_pvric_const {

struct drm_vs_color 		color[2];

};

Members

color: the constant color values. For structure details, see Section 4.3.33.2.2, drm_vs_color.

- color[0]: constant value 0.

It specifies the constant color to be assigned to the following signals when the compressor tile header indicates that the tile consists of only a constant color:

  - Non-YUV format: fbc_fbdc_cr_ch0123_val0

  - YUV format: fbc_fbdc_cr_y_val0 and fbc_fbdc_cr_uv_val0

- color[1]: constant value 1.

It specifies the constant color to be assigned to the following signals when the compressor tile header indicates that the tile consists of only a constant color:

  - Non-YUV format: fbc_fbdc_cr_ch0123_val1

  - YUV format: fbc_fbdc_cr_y_val1 and fbc_fbdc_cr_uv_val1

drm_vs_color

Definition

struct drm_vs_color {

__u32		a;

__u32		r;

__u32		g;

__u32		b;

};

Members

For non-YUV formats:

- a[7:0]: color channel 3 or bits 31:24 of each pixel

- r[7:0]: color channel 2 or bits 23:16 of each pixel

- g[7:0]: color channel 1 or bits 15:8 of each pixel

- b[7:0]: color channel 0 or bits 7:0 of each pixel

For YUV formats:

- a[9:0]: 10-bit Y value

- r[9:0]: 10-bit UV value

Members g and b are invalid.

PVRIC_CLEAR

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the PVRIC clear color values provided through the drm_vs_pvric_clear structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_pvric_clear

Definition

struct drm_vs_pvric_clear {

__u64 		color[3];

};

Members

color: the plane clear color.

- color[0]: the clear color of plane 0

- color[1]: the clear color of plane 1

- color[2]: the clear color of plane 2

IOCTL for GEM Object Query

Compatible Hardware

This IOCTL is available only for DC9x00 series.

Definition

#define DRM_IOCTL_VS_GEM_QUERY \

DRM_IOWR(DRM_COMMAND_BASE + DRM_VS_GEM_QUERY, struct drm_vs_gem_query_info)

DRM_IOCTL_DEF_DRV(VS_GEM_QUERY, vs_gem_query_ioctl, DRM_MASTER)

For information about the private structure, see Section 4.3.33.4.1, drm_vs_gem_query_info.

drm_vs_gem_query_info

Definition

struct drm_vs_gem_query_info {

enum drm_vs_gem_query_type	type

__u32				handle;

__u64				data;

};

Members

- type: the type of GEM information to be queried. Set this member to VS_GEM_QUERY_HANDLE, which indicates a GEM handle is queried.

- handle: the handle of a GEM object.

- data: the returned data, which is the physical address of the buffer to which the handle points.

Write-Back Paths

Write-Back Point

WB_POINT

| Category | Private connector property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Enumeration |
| Values | See Table 26 |
| Description | The position of the write-back connector. For the supported write-back points, see the Write-Back Paths or Write-Back Features section in the provided Vivante display processing IP hardware architecture and features document. |

Table 26. drm_vs_wb_point Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_WB_DISP_OUT | 0 | (Tapping point 3) Writes back data from the output of the rounded corner display (RCD) module. |
| VS_WB_DISP_IN | 1 | (Tapping point 1) Writes back data from the blender before the post-processing pipeline. |
| VS_WB_DISP_CC | 2 | (Tapping point 2) Writes back data from the output of the post-processing color calibration module. |
| VS_WB_DISP_SPR | 3 | (Tapping point 4) Writes back data from the output of the SPR module. |
| VS_WB_DISP_DSC_OUT | 4 | (Tapping point 5) Writes back data from the output of the post-processing Display Stream Compression (DSC) module. |
| VS_WB_OFIFO_IN | 5 | Writes back data from the input of OFIFO. |
| VS_WB_OFIFO_OUT | 6 | Writes back data from the output of OFIFO. |
| VS_WB_POS_CNT | 7 | The number of write back points. |

Write-Back Splitter

SPLITER

| Category | Private connector property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the write-back splitter configurations provided through the drm_vs_wb_spliter structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_wb_spliter

Definition

struct drm_vs_wb_spliter {

struct drm_vs_rect		split_rect0;

struct drm_vs_rect		split_rect1;

};

Members

- split_rect0: the rectangle for the splitter in write-back path 0.

For details, see Section 3.2.1, drm_vs_rect.

- split_rect1: the rectangle for the splitter in write-back path 1.

For details, see Section 3.2.1, drm_vs_rect.

Write-Back Frame Difference

WB_FRM_DIFF

| Category | Private connector property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the write-back frame difference configurations provided through the drm_vs_frm_diff structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_frm_diff

Definition

struct drm_vs_frm_diff {

enum drm_vs_frm_diff_mode			mode;

enum drm_vs_frm_diff_hit_reset_mode		hit_reset_mode;

__u8 						cur_ratio;

__u8 						acc_threshold;

__u32 						fixed_threshold;

__u32 						fb_id;

};

Members

- mode: the work mode of the frame difference histogram module. Set this member to a value listed in the following table.

Table 27. drm_vs_frm_diff_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_HIST_MODE | 0 | The histogram mode. |
| VS_HIT_MODE | 1 | The hit mode. |

- hit_reset_mode: whether to reset the information of the previous frame when the hit mode is used. Set this member to a value listed in the following table.

Table 28. drm_vs_frm_diff_hit_reset_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_HIT_FLOP | 0 | Do not reset the information of the previous frame. |
| VS_HIT_RESET | 1 | Reset the information of the previous frame. |

- cur_ratio: the dynamic threshold ratio for frame difference detection.

- acc_threshold: the accumulation count threshold for frame difference detection.

- fixed_threshold: the fixed threshold of the frame-difference hisgrogram module.

- fb_id: the framebuffer ID of the previous fame.

WB_FRM_DIFF_RESULT

| Category | Private connector property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the write-back frame difference result provided through the drm_vs_frm_diff_result structure. |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_frm_diff_result

Definition

#define VS_FRM_DIFF_RESULT_BIN_COUNT 32

struct drm_vs_frm_diff_result {

__u32 			hist_res[VS_FRM_DIFF_RESULT_BIN_COUNT];

__u32 			hit_res;

};

Members

- hist_res: the result of write-back frame difference when the histogram mode is used.

- hit_res: the result of write-back frame difference when the hit mode is used.

Write-Back Scaler

SCALER

| Category | Private connector property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the write-back scaler configurations provided through the drm_vs_scale_config structure |
| Description | The ID of the blob object set. The default value is 0. |

Write-Back Dither

WB_DITHER

| Category | Private connector property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the write-back dither configurations provided through the drm_vs_dither structure |
| Description | The ID of the blob object set. The default value is 0. |

Write-Back RGB-to-YUV Conversion

R2Y

| Category | Private connector property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the write-back RGB-to-YUV configurations provided through the drm_vs_r2y_config structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_r2y_config

Definition

#define VS_MAX_R2Y_COEF_NUM    15

struct drm_vs_r2y_config {

enum drm_vs_csc_mode		mode;

enum drm_vs_csc_gamut	gamut;

__s32				coef[VS_MAX_R2Y_COEF_NUM];

__u32				output_bus_format;

};

Members

- mode: the color space conversion (CSC) mode. For details, see Section 3.1.1, drm_vs_csc_mode.

- gamut: the color gamut. For details, see Section 3.1.2, drm_vs_csc_gamut.

- coef[VS_MAX_R2Y_COEF_NUM]: the user-defined coefficients.

- coef[0] to coef[8]: coefficients 00 to 22, in the S7.12 format (1 signed bit, 6 integer bits, 12 fraction bits)

- coef[9] to coef[11]: pre-offsets D0 to D2. Each offset is 13-bit (1 signed bit, 12 integer bits)

- coef[12] to coef[14]: post-offsets D0-D2. Each offset is 13-bit (1 signed bit, 12 integer bits)

- output_bus_format: the color format of the output write-back data used for debug only. The values can be:

- MEDIA_BUS_FMT_RGB666_1X18

- MEDIA_BUS_FMT_RGB666_1X24_CPADHI

- MEDIA_BUS_FMT_RGB101010_1X30

- MEDIA_BUS_FMT_UYVY8_1X16

- MEDIA_BUS_FMT_UYVY10_1X20

- MEDIA_BUS_FMT_YUV8_1X24

- MEDIA_BUS_FMT_YUV10_1X30

- MEDIA_BUS_FMT_RGB888_1X24 (default)

Write-Back Rotation/Flip

WB_ROTATION

| Category | Private connector property |
| --- | --- |
| Compatible Hardware | DC9500 |
| Data Type | Bitmask |
| Values | ROT_0 = 0x01, ROT_90 = 0x02, ROT_180 = 0x04, ROT_270 = 0x08, FLIP_X = 0x11, FLIP_Y = 0x21, FLIPX_90 = 0x12, FLIPY_90 = 0x22 |
| Description | The rotation and flip modes. The default value is ROT_0 (0x01). |

Write-Back Cropping

WB_CROP

| Category | Private connector property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the cropping configurations provided through the drm_vs_panel_crop structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_panel_crop

Definition

struct drm_vs_panel_crop {

struct drm_vs_rect		crop_rect;

__u16 				panel_src_width;

__u16				panel_src_height;

};

Members

- crop_rect: the rectangular area output after cropping. For details, see Section 3.2.1, drm_vs_rect.

- panel_src_width: the width of the source image before cropping.

- panel_src_height: the height of the source image before cropping.

UV Downsampling

DOWN_SAMPLE

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the UV downsampling configurations provided through the drm_vs_ds_config structure |
| Description | The ID of the blob object set. The default value is 0. |

Write-back Stripe

WB_STRIPE_POS

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the postion of the write-back stripe, which is an offset to the write-back address, through the drm_vs_postion structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_postion

Definition

struct drm_vs_postion {

__u16		x;

__u16		y;

};

Members

- x: the X-coordinate of the write-back stripe.

- y: the Y-coordinate of the write-back stripe.

Miscellaneous

Side-by-Side Buffer

SBS

| Category | Private plane property |
| --- | --- |
| Compatible Hardware | DC9500 |
| Data Type | Blob object that contains the side-by-side buffer configurations provided through the drm_vs_sbs structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_sbs

Definition

struct drm_vs_sbs {

enum drm_vs_sbs_mode		mode;

__u16				left_w;

__u16				right_x;

__u16				right_w;

};

Members

- mode: the work mode of the side-by-side buffer. Set this member to a value listed in the following table.

Table 29. drm_vs_sbs_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_SBS_LEFT | 0 | The side-by-side buffer outputs the layer image to the left-port compositor. |
| VS_SBS_RIGHT | 1 | The side-by-side buffer outputs the layer image to the right-port compositor. |
| VS_SBS_SPLIT | 2 | The side-by-side buffer vertically splits the layer image to two parts, and outputs the left part to the left-port compositor and the right part to the right-port compositor. If this value is selected, set members left_w, right_x, and right_w to configure the split left and right parts. The union of the left and right parts must be the same as the layer image. This value is unavailable if the MODE member of the drm_vs_dma structure for the DMA_CONFIG property is set to VS_DMA_TWO_ROI or VS_DMA_SKIP_ROI. |
| VS_SBS_RESERVED | 3 | Reserved. |

- left_w: the width of the left part after layer image splitting, with the start X-coordinate being 0.

This member is valid only if mode is set to VS_SBS_SPLIT.

- right_x: the start X-coordinate of the right part after layer image splitting.

This member is valid only if mode is set to VS_SBS_SPLIT.

- right_w: the width of the right part after layer image splitting.

This member is valid only if mode is set to VS_SBS_SPLIT.

Display Stream Compression

DSC

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the VESA Display Stream Compression (DSC) v1.2a configurations provided through the drm_vs_dsc structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_dsc

Definition

struct drm_vs_dsc {

__u8		slices_per_line

__u8		ss_num;

__u16		slice_height;

__u16		picture_width;

__u16		picture_height;

bool		split_panel_enable;

bool		multiplex_mode_enable;

int		multiplex_out_sel;

bool		de_raster_enable;

bool		multiplex_eoc_enable;

bool		video_mode;

};

Members

- slices_per_line: the number of slices in each horizontal line.

- ss_num: the number of active soft slices.

- slice_height: the height of each slice.

- picture_width: the width of the source picture before DSC.

- picture_height: the height of the source picture before DSC.

- split_panel_enable: whether to split the video stream of the output panel.

- multiplex_mode_enable: whether to enable multiplexing.

- multiplex_out_sel: the output to which the multiplexed stream is sent.

This member is required when split_panel_enable and multiplex_mode_enable are set to 1.

- de_raster_enable: whether to enable the de-rasterization buffer.

- multiplex_eoc_enable: whether to insert zeros on partial words at each chunk end.

This member is required when split_panel_enable and multiplex_mode_enable are set to 1.

- video_mode: whether to enable the video mode.

VDCM

| Category | Private CRTC property |
| --- | --- |
| Compatible Hardware | DC9x00 series |
| Data Type | Blob object that contains the VESA Display Compression-M (VDC-M) v1.2 configurations provided through the drm_vs_vdc structure |
| Description | The ID of the blob object set. The default value is 0. |

drm_vs_vdc

Definition

struct drm_vs_vdc {

__u8		slices_per_line

__u8		ss_num;

__u16		slice_height;

bool		split_panel_enable;

bool		multiplex_mode_enable;

int		multiplex_out_sel;

bool		multiplex_eoc_enable;

bool		hs_split_input_enable;

bool		video_mode;

};

Members

- slices_per_line: the number of slices in each horizontal line.

- ss_num: the number of active soft slices.

- slice_height: the height of each slice.

- split_panel_enable: whether to split the video stream of the output panel.

- multiplex_mode_enable: whether to enable multiplexing.

- multiplex_out_sel: the output to which the multiplexed stream is sent.

- multiplex_eoc_enable: whether to align the start of each chunk with the most significant byte of the output interface.

- hs_split_input_enable: whether to split the input into hard slices.

- video_mode: whether to enable the video mode.

Display Frame Done Query

IOCTL for Display Frame Done Count Query

Definition

#define DRM_IOCTL_VS_GET_DISPLAY_FRAME_DONE \

DRM_IOWR(DRM_COMMAND_BASE+DRM_VS_GET_DISPLAY_FRAME_DONE, struct drm_vs_display_frm_done)

DRM_IOCTL_DEF_DRV(VS_GEM_QUERY, vs_get_display_frm_done_ioctl, DRM_MASTER)

drm_vs_display_frm_done

Definition

struct drm_vs_display_frm_done {

__u8 id;

__u8 frm_done_count;

};

Members

- id: Panel ID.

- frm_done_count: The count of panel frame done signal.

CRC

Compatible Hardware

The CRC feature for plane objects is available for DC9x00 series and DC8x00 series.

The CRC feature for CRTC objects is available only for DC8x00 series.

CRC Configuration

The cyclic redundancy check (CRC) feature can be configured through the debugfs node for plane or CRTC objects.

DC9x00 CRTC CRC Configuration

An example command for configuring DC9x00 CRTC CRC is provided as follows:

/sys/kernel/debug/dri/1/crtc-0# echo "enable:1 pos:1 a-seed:0x32 r-seed:0x23 
g-seed:0x56 b-seed:0x12" > CRC

The parameters used in this command are described in Table 30.

When the current frame is completed, you can query the CRC value by using the Linux command cat.

Table 30. DC9x00 CRTC CRC Parameters

| Parameter | Description |
| --- | --- |
| enable | Indicates whether to enable the CRC feature. Set this parameter to one of the following values: 0: disable 1: enable |
| pos | The position of CRC. Set this parameter to one of the following values: 0: BLD 1: POST_PROC 2: OFIFO_IN 3: OFIFO_OUT 4: WB |
| a-seed | The seed for the alpha channel. |
| r-seed | The seed for the red channel. |
| g-seed | The seed for the green channel. |
| b-seed | The seed for the blue channel. |

DRM Standard Compliant DC9x00 CRTC CRC Configuration

An example command for configuring DC9x00 CRTC CRC in compliance with DRM standards is provided as follows:

/sys/kernel/debug/dri/1/crtc-0/crc/# echo displayOut > control

In this command, displayOut indicates the CRC position after OFIFO. All CRC position values are listed in Table 31.

When the current frame is completed, the CRC value can be queried by using the Linux command cat. For example:

/sys/kernel/debug/dri/1/crtc-0/crc/# cat data

Table 31. DC9x00 CRTC CRC Position Values

| Value | Description |
| --- | --- |
| none | - The CRC value is not calculated. |
| displayOut | The CRC position is located after OFIFO. |
| ramlessOut | The CRC value is calculated when RAM-less is enabled. In this case, the CRC position is located after the write-back pipeline. |
| auto | The default CRC postion is used. This value has the same effect as displayOut. |

DC8x00 CRTC CRC Configuration

An example command for configuring DC8x00 CRTC CRC is provided as follows:

/sys/kernel/debug/dri/1/crtc-0# echo "enable:1 pos:2 initMode:0" > CRC

If you want to calculate the CRC values of 16 regions of interest (ROIs), you must specify additional parameters including index, roiX, roiY, roiW, and roiH. The example command is provided as follows:

/sys/kernel/debug/dri/1/crtc-0# echo "enable:1 pos:0 initMode:0 index:0 roiX:0 roiY:0 roiW:40 roiH:480" > CRC

The parameters used in the preceding commands are described in Table 32.

Table 32. DC8x00 CRTC CRC Parameters

| Parameter | Description |
| --- | --- |
| enable | Indicates whether to enable the CRC feature. Set this parameter to one of the following values: 0: disable 1: enable |
| pos | The position of CRC. Set this parameter to one of the following values: 0: VS_DISP_CRC_BLD 2: VS_DISP_CRC_OFIFO_IN 4: VS_DISP_CRC_WB |
| initMode | Indicates how to determine the initial value of CRC. If the pos parameter is set to 0, set initMode to one of the following values: 0: The initial value is fixed to 0. 1: The initial value is fixed to 0xFFFFFFFF. If the pos parameter is set to 2 or 4, set initMode to one of the following values: 0: The initial value is configurable. 1: The initial value is the result of the previous calculation. |
| initValue | (Optional) The custom initial value of CRC. This parameter is valid only when the pos parameter is set to 2 or 4 and the initMode parameter is set to 0. |
| xorValue | (Optional) The XOR value of CRC. This parameter is valid only when the pos parameter is set to 4. |
| index | (Optional) The index of the ROI whose CRC value you want to calculate. Set this parameter to a value in range from 0 to 15. This parameter is valid only when the pos parameter is set to 0. |
| roiX | (Optional) The X offset of the upper-left corner of the ROI. This parameter is valid only when the pos parameter is set to 0. |
| roiY | (Optional) The Y offset of the upper-left corner of the ROI. This parameter is valid only when the pos parameter is set to 0. |
| roiW | (Optional) The width of the ROI. This parameter is valid only when the pos parameter is set to 0. |
| roiH | (Optional) The height of the ROI. This parameter is valid only when the pos parameter is set to 0. |

DC9x00 Plane CRC Configuration

An example command for configuring DC9x00 plane CRC is provided as follows:

/sys/kernel/debug/dri/1/plane-0# echo "enable:1 pos:1 a-seed:0x32 r-seed:0x23 
g-seed:0x56 b-seed:0x12" > CRC

The parameters used in this command are described in Table 33.

Table 33. DC9x00 Plane CRC Parameters

| Parameter | Description |
| --- | --- |
| enable | Indicates whether to enable the CRC feature. Set this parameter to one of the following values: 0: disable 1: enable |
| pos | The position of CRC. Set this parameter to one of the following values: 0: DFC 1: HDR |
| a-seed | The seed for the alpha channel. |
| r-seed | The seed for the red channel. |
| g-seed | The seed for the green channel. |
| b-seed | The seed for the blue channel. |

DC8x00 Plane CRC Configuration

An example command for configuring DC8x00 plane CRC is provided as follows:

/sys/kernel/debug/dri/1/plane-0# echo "enable:1 initMode:0 initVAlue: 0x0 xorValue: 0x0 golden0:0x5f90b049 golden1:0x0 golden2:0x0 > CRC

The parameters used in this command are described in Table 34.

Table 34. DC8x00 Plane CRC Parameters

| Parameter | Description |
| --- | --- |
| enable | Indicates whether to enable the CRC feature. Set this parameter to one of the following values: 0: disable 1: enable |
| initMode | Indicates how to determine the initial value of CRC. 0: The initial value is configurable. 1: The initial value is the result of the previous calculation. |
| initValue | The initial value of CRC. This parameter is valid only when the initMode parameter is set to 0. |
| xorValue | The XOR value of CRC. |
| golden0 | The golden value of CRC results for the red channel. |
| golden1 | The golden value of CRC results for the green channel. |
| golden2 | The golden value of CRC results for the blue channel. |

CRC Results

The CRC results are stored in the following files of the debugfs directory of DRM. You can obtain the CRC results in the user space.

- plane_crc: stores the CRC results of each available plane object, including:

- The CRC result after the DMA module

- The CRC result before the blending pipeline

- crtc_crc: stores the CRC results of each available CRTC object, including:

- The CRC result after the blending module

- The CRC result after the low-level dither module and RCD module

- The CRC result after the RGB-to YUV module and before the panel dither module

- The CRC result after the DSC module

- The CRC result for the splitter1 output path after the DSC/splitter module

Note: The available CRTC objects vary among different product versions. For details, see the Cyclic Redundancy Check section in the Vivante display processing IP hardware architecture and features document.

- wb_crc: stores the CRC results of each available write-back path.

IOCTL for Display CRC Result Query

Definition

#define DRM_IOCTL_VS_GET_DISPLAY_CRC \

DRM_IOWR(DRM_COMMAND_BASE + DRM_VS_GET_DISPLAY_CRC, struct drm_vs_display_crc_result)

DRM_IOCTL_DEF_DRV(VS_GEM_QUERY, vs_get_display_crc_ioctl, DRM_MASTER)

drm_vs_display_crc_result

Definition

#define VS_MAX_DC_DISPLAY_NUM 5

struct drm_vs_display_crc_result {

struct drm_vs_crc result[VS_MAX_DC_DISPLAY_NUM];

};

Members

- VS_MAX_DC_DISPLAY_NUM: The maximum number of panels supported by the display processing IP.

- drm_vs_crc: CRC result for each panel.

For information about the the drm_vs_crc structure, see Section 4.5.4.3.2, drm_vs_crc.

drm_vs_crc

Definition

#define VS_MAX_CRC_CORE_NUM 2

struct drm_vs_display_crc {

struct drm_vs_color crc[VS_MAX_CRC_CORE_NUM];

};

Members

- VS_MAX_CRC_CORE_NUM: The maximum number of CRC cores at OFIFO output.

- crc: The CRC result of each panel.

Test Pattern

Compatible Hardware

- The test pattern feature for plane objects is available only for DC9x00 series.

- The text pattern feature for CRTC objects is available for DC9x00 series and DC8x00 series.

Test Pattern Configuration

The display processing IP supports built-in self-test (BIST) patterns at the major boundaries to examine the screen color.

DC9x00 CRTC and Plane Test Pattern Configuration

To change the configurations of the test patterns, use the private debugfs_fops.write operation with the following options:

- enable: whether to enable the test pattern feature.

- position: the test pattern position. Set this option to a value listed in Table 35.

- For plane objects, only the VS_PATRN_DMA value is available.

- For CRTC objects, the VS_PATRN_BLD and VS_PATRN_DITHER values are available.

Table 35. position Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_PATRN_DMA | 0 | After the DMA module. This value is available only for plane objects. |
| VS_PATRN_BLD | 1 | After the blending module. This value is available only for CRTC objects. |
| VS_PATRN_DITHER | 2 | After the RCD module for output panels 0 and 1. After the panel dither module for output panels 2 and 3. |

- mode: the test pattern mode. Set this option of a value listed in Table 36.

Table 36. mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_PURE_COLOR | 0 | The pure color mode. Whole frame/layer in a programmable color. |
| VS_COLOR_BAR_H | 1 | The horizontal color bar mode. White, yellow, cyan, green, magenta, red, blue, and black horizontal color bars with equal and programmable bar height. If the bar height is too small for the eight color bars to cover the entire frame/layer, the color bars are repeated from white to black. |
| VS_COLOR_BAR_V | 2 | The vertical color bar mode. White, yellow, cyan, green, magenta, red, blue, and black vertical color bars with equal and programmable bar width. If the bar width is too small for the eight color bars to cover the entire frame/layer, the color bars are repeated from white to black. |
| VS_RMAP_H | 3 | The horizontal Rmap mode. R, G, B, and W ramp color bars in horizontal direction with equal and programmable bar height. If the bar height is too small for the four color bars to cover the entire frame/layer, the ramp color bars are repeated from red to white. |
| VS_RMAP_V | 4 | The vertical Rmap mode. R, G, B, W ramp color bar in the direction of vertical with equal and programmable bar width. If the bar width is too small for the four color bars to cover the entire frame/layer, the ramp color bars are repeated from red to white. |
| VS_BLACK_WHITE_H | 5 | The black-white horizontal bar pattern mode. Horizontal black-white bar patterns with programmable bar height. The bar height ranges from 1 to 256. |
| VS_BLACK_WHITE_V | 6 | The black-white vertical bar pattern mode. Vertical black-white bar patterns with programmable bar width. The bar width ranges from 1 to 256. |
| VS_BLACK_WHITE_SQR | 7 | The black-white square pattern mode. Chess board pattern with programmable square size. The square side length ranges from 1 to 256. |
| VS_BORDER_PATRN | 8 | The border mode. One-pixel-wide border in a programmable color around the layer/frame. |
| VS_CURSOR_PATRN | 9 | The cursor mode. One-pixel-wide cross shown in a programmable color at a programmable coordinate. Note: The value of the specified pixel can be read back through the APB interface and stored in the corresponding debugfs file. |

- size: the size information. This option is invalid when mode is set to VS_PURE_COLOR or VS_BORDER_PATRN.

- Use size.x and size.y to configure the coordinates when mode is set to VS_CURSOR_PATRN.

- Use size.h to specify the height when mode is set to VS_COLOR_BAR_H, VS_RMAP_H, VS_BLACK_WHITE_H, or VS_BLACK_WHITE_SQR.

- Use size.w to specify the width when mode is set to VS_COLOR_BAR_V, VS_RMAP_V, or VS_BLACK_WHITE_V.

- color: a color. The value is of the U64 type with 16 bits per A/R/G/B channel.

This option is valid only when mode is set to VS_PURE_COLOR, VS_BORDER_PATRN, or VS_CURSOR_PATRN.

DC8x00 CRTC Test Pattern Configuration

To change the configurations of the test patterns, use the private debugfs_fops.write operation with the following options:

- index: indicates the ID of the area for which you want to enable the test pattern feature.

Value range: [0, 15].

- enable: whether to enable the test pattern feature.

- size.x: indicates the X-coordinate of the upper-left corner of the area where you want to enable the test pattern feature.

- size.y: indicates the Y-coordinate of the upper-left corner of the area where you want to enable the test pattern feature.

- size.w: indicates the width of the area where you want to enable the test pattern feature.

- size.h: indicates the height of the area where you want to enable the test pattern feature.

- color: indicates the color that the test pattern applies to the area.

Test Pattern Results

The test data is stored in the plane_pattern and crtc_pattern files in the debugfs directory of DRM. You can obtain the test data in the user space.

QoS Management

The quality of service (QoS) management feature can be configured through the debugfs node for plane, CRTC, or write-back connector objects.

Compatible Hardware

The QoS management feature is available only for DC9x00 series.

Plane QoS Level Configuration

Example commands for configuring plane QoS levels are provided as follows:

/sys/kernel/debug/dri/1/plane-0# echo "fe0QosValue0:15 fe1QosValue0:15 qosValue0:15" > QOS

/sys/kernel/debug/dri/1/plane-0# echo "fe0QosValue1:11 fe1QosValue1:11 qosValue1:11" > QOS

/sys/kernel/debug/dri/1/plane-0# echo "fe0QosValue2:7 fe1QosValue2:15 qosValue2:7" > QOS

/sys/kernel/debug/dri/1/plane-0# echo "fe0QosValue3:3 fe1QosValue3:15 qosValue3:3" > QOS

The parameters used for configuring the plane QoS levels are described in Table 37.

Table 37. Plane QoS Level Parameters

| Parameter | Description |
| --- | --- |
| fe0QosValue0 | The value of QoS level 0 of frontend 0 (FE0). Valid values: [0, 15] |
| fe0QosValue1 | The value of QoS level 1 of FE0. Valid values: [0, 15] |
| fe0QosValue2 | The value of QoS level 2 of FE0. Valid values: [0, 15] |
| fe0QosValue3 | The value of QoS level 3 of FE0. Valid values: [0, 15] |
| fe1QosValue0 | The value of QoS level 0 of frontend 1 (FE1). Valid values: [0, 15] |
| fe1QosValue1 | The value of QoS level 1 of FE1. Valid values: [0, 15] |
| fe1QosValue2 | The value of QoS level 2 of FE1. Valid values: [0, 15] |
| fe1QosValue3 | The value of QoS level 3 of FE1. Valid values: [0, 15] |
| qosValue0 | The value of QoS level 0 of the specified plane. Valid values: [0, 15] |
| qosValue1 | The value of QoS level 1 of the specified plane. Valid values: [0, 15] |
| qosValue2 | The value of QoS level 2 of the specified plane. Valid values: [0, 15] |
| qosValue3 | The value of QoS level 3 of the specified plane. Valid values: [0, 15] |

CRTC QoS Level Configuration

Example commands for configuring the CRTC QoS levels are provided as follows:

/sys/kernel/debug/dri/1/crtc-0# echo "qosTH0:9600 urgentTH0:5000 qosValue0:15" > QOS

/sys/kernel/debug/dri/1/crtc-0# echo "qosTH1:6400 urgentTH1:3500 qosValue1:11" > QOS

/sys/kernel/debug/dri/1/crtc-0# echo "qosTH2:3200 urgentTH2:1600 qosValue2:7" > QOS

/sys/kernel/debug/dri/1/crtc-0# echo "urgentHigh:6400 urgentLow:3200 qosValue3:3" > QOS

/sys/kernel/debug/dri/1/crtc-0# echo "histQosValue:3" > QOS

The parameters used for configuring CRTC QoS levels are described in Table 38.

Table 38. CRTC QoS Level Parameters

| Parameter | Description |
| --- | --- |
| qosTH0 | The QoS threshold 0 of the output FIFO (OFIFO) bound to the CRTC. |
| qosTH1 | The QoS threshold 1 of the OFIFO bound to the CRTC. |
| qosTH2 | The QoS threshold 2 of the OFIFO bound to the CRTC. |
| urgentHigh | The urgent high value of the OFIFO bound to the CRTC. |
| urgentLow | The urgent low value of the output FIFO bound to the CRTC. |
| qosValue0 | The value of QoS level 0 of the OFIFO bound to the CRTC. Valid values: [0, 15] |
| qosValue1 | The value of QoS level 1 of the OFIFO bound to the CRTC. Valid values: [0, 15] |
| qosValue2 | The value of QoS level 2 of the OFIFO bound to the CRTC. Valid values: [0, 15] |
| qosValue3 | The value of QoS level 3 of the OFIFO bound to the CRTC. Valid values: [0, 15] |
| histQosValue | The value of the QoS level of the histogram write-back. Valid values: [0, 15] This parameter is valid only when histogram is supported on CRTC. |

Write-back Connector QoS Level Configuration

An example command for configuring the write-back connector QoS level is provided as follows:

/sys/kernel/debug/dri/1/Writeback-1# echo "qosValue0:3" > QOS

The parameter used for configuring the write-back connector QoS level is described in Table 39.

Table 39. Write-Back Connector QoS Level Parameters

| Parameter | Description |
| --- | --- |
| qosValue | The value of the QoS level of the write-back connector. Valid values: [0, 15] |

Palette

Compatible Hardware

The palette feature for plane objects is available for DC9x00 series.

Palette Configuration

Default Value

The palette LUT is initialized upon driver installation. By default, the palette LUT contains 256 ARGB8 entries, forming an arithmetic sequence from 0x00000000 to 0xFFFFFFFF.

User Configuration

The default palette LUT can be reconfigured during runtime through the debugfs node for plane objects.

An example command for configuring DC9x00 plane palette is provided as follows:

/sys/kernel/debug/dri/1/plane-2# cat <drm-test path>/resource/lut/palette_lut.txt > palette

The palette LUT file used in this command is a resource file containing 256 lines. Each line is a 32-bit hexadecimal ARGB8 color value starting with “0x”.

Table 40 shows an example of the palette LUT file.

Table 40. palette_lut.txt Example

| Line | Value |
| --- | --- |
| 1 | 0xFF0000FF |
| 2 | 0xFF1100FF |
| 3 | 0xFF2200FF |
| … | … |
| 256 | 0xFFFFFFFF |

Active Palette LUT

The active palette LUT is accessible through the plane_palette files in the debugfs directory of DRM. Reading this file returns either the default gradient or the user configuration currently in use.

Capability Query

IOCTL for Feature Support Query

Compatible Hardware

This IOCTL is available for DC9x00 series and DC8x00 series.

Definition

#define DRM_IOCTL_VS_GET_FEATURE_CAP \

DRM_IOWR(DRM_COMMAND_BASE + DRM_VS_GET_FEATURE_CAP, struct drm_vs_query_feature_cap)

DRM_IOCTL_DEF_DRV(DRM_VS_GET_FEATURE_CAP, vs_get_feature_cap_ioctl, DRM_MASTER)

For information about the private structure, see Section 4.5.8.1.1, drm_vs_query_feature_cap.

drm_vs_query_feature_cap

Definition

struct drm_vs_query_feature_cap {

enum drm_vs_feature_cap_type	type;

__u32					value;

__u32					cap;

};

Members

- type: the feature to be queried. Set this member to a value listed in Table 41.

Table 41. drm_vs_feature_cap_type Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_FEATURE_CAP_ CURSOR_WIDTH | 0 | The supported maximum cursor width. |
| VS_FEATURE_CAP_ CURSOR_HEIGHT | 1 | The supported maximum cursor height. |
| VS_FEATURE_CAP_USE_VCMD | 2 | - Whether the VCMD mode is used. If this feature is queried, possible return values include: 0: yes 1: no |
| VS_FEATURE_CAP_VCMD_CMDBUF_REMAINING | 3 | - Whether any CMD buffer is still available in the SRAM pool. If this feature is queried, possible return values include: 0: yes 1: no |

- value: user data used to query.

- cap: the return value of the query.

IOCTL for Plane Capability Query

Compatible Hardware

This IOCTL is available for DC9x00 series, DCNano series, and DC8x00 series.

Definition

#define DRM_IOCTL_VS_QUERY_PLANE_CAP \

DRM_IOWR(DRM_COMMAND_BASE + DRM_VS_QUERY_PLANE_CAP, struct drm_vs_query_plane_cap)

DRM_IOCTL_DEF_DRV(VS_QUERY_PLANE_CAP, vs_query_plane_cap_ioctl, DRM_MASTER)

drm_vs_query_plane_cap

Definition

struct drm_vs_query_plane_cap {

__u32					plane_id;

__u32					min_width;

__u32					min_height;

__u32					max_width;

__u32					max_height;

__s32					min_scale;

__s32					max_scale;

__u64					reserved[4];

};

Members

- plane_id: [in] the ID of the DRM plane object to be queried.

- min_width: [out] the minimum source buffer width.

- min_height: [out] the minimum source buffer height.

- max_width: [out] the maximum source buffer width.

- max_height: [out] the maximum source buffer height.

- min_scale: [out] the minimum scaling ratio, which is defined as (src << 16)/dst.

- max_scale: [out] the maximum scaling ratio, which is defined as src << 16)/dst.

- reserved: [out] reserved.

Reset

IOCTL for Software and Hardware Reset

Compatible Hardware

This IOCTL is available for DC9x00 series and DC8x00 series.

Definition

#define DRM_IOCTL_VS_SW_RESET \

DRM_IOWR(DRM_COMMAND_BASE + DRM_VS_SW_RESET, struct drm_vs_reset)

DRM_IOCTL_DEF_DRV(DRM_VS_SW_RESET, vs_sw_reset_ioctl, DRM_MASTER)

For more information about the private structure, see Section 4.5.9.1.1, drm_vs_reset.

drm_vs_reset

Definition

struct drm_vs_reset {

enum drm_vs_reset_mode	mode;

};

Members

- mode: the reset mode. Set this member to a value listed in Table 42.

Table 42. drm_vs_reset_mode Values

| Enumeration Value | Numerical Value | Description |
| --- | --- | --- |
| VS_RESET | 0 | - Resets the status of all hardware and software. |
| VS_FE0_RESET 1 | 1 | Resets the status of the DPU_FE0 hardware and software status. |
| VS_FE1_RESET 1 | 2 | Resets the status of the DPU_FE1 hardware and software. |
| VS_BE_RESET 1 | 3 | Resets the status of the DPU_BE hardware and software. |

Note 1: When you set the mode member to VS_FE0_RESET, VS_FE1_RESET, or VS_BE_RESET for DC8x00 series, all hardware and software are reset.

Write-Back Status Query

IOCTL for Querying the Reception Status of the Write-Back Frame Done Interrupt

Compatible Hardware

This IOCTL is available for DC9x00 series and DC8x00 series.

Definition

#define DRM_IOCTL_VS_GET_WB_FRM_DONE \

DRM_IOWR(DRM_COMMAND_BASE + DRM_VS_GET_WB_FRM_DONE, struct drm_vs_wb_frm_done)

DRM_IOCTL_DEF_DRV(VS_GET_WB_FRM_DONE, vs_get_wb_frm_done_ioctl, DRM_MASTER)

For more information about the private structure, see Section 4.5.10.1.1, drm_vs_wb_frm_done.

drm_vs_wb_frm_done

Definition

struct drm_vs_wb_frm_done {

__u8 	wb_id;

bool	wb_frm_done;

};

Members

- wb_id: indicates the identifier of the write-back connector to be queried.

- wb_frm_done: indicates whether the write-back of the frame is finished.

Blending Write-Back Information Query

IOCTL for Querying the Blending Write-Back Information

Compatible Hardware

This IOCTL is available for DC9x00 series.

Definition

#define DRM_IOCTL_VS_GET_BLD_WB_INFO \

DRM_IOWR(DRM_COMMAND_BASE + DRM_VS_GET_BLD_WB_INFO, struct drm_vs_query_bld_wb_info)

DRM_IOCTL_DEF_DRV(VS_GET_BLD_WB_INFO, vs_get_bld_wb_info_ioctl, DRM_MASTER)

For more information about the private structure, see Section 4.5.11.1.1, drm_vs_query_bld_wb_info.

drm_vs_query_bld_wb_info

Definition

#define VS_MAX_BLD_WB_SIZE	2

struct drm_vs_query_bld_wb_info {

bool 					bld_wb_support;

struct drm_vs_blend_wb_info 		bld_wb_info[VS_MAX_BLD_WB_SIZE];

__u8 					bld_wb_num;

};

Members

- bld_wb_support: specifies whether the current display architecture supports blending write-back.

- bld_wb_info: specifies the connector ID and CRTC ID for each blending write-back pipeline.

- bld_wb_num: specifies the number of simultaneous blending write-back pipelines supported by the current hardware architecture.

drm_vs_blend_wb_info

Definition

struct drm_vs_blend_wb_info {

__u8 	bld_wb_connector_id;

__u8 	bld_wb_crtc_id;

};

Members

- bld_wb_connector_id: indicates the connector ID for the blending write-back pipeline.

- bld_wb_crtc_id: indicates the CRTC ID for the blending write-back pipeline.

A Document Revision History

This chapter describes the differences between different reversions of this document.

Note: This document is not necessarily updated for each patch or minor revision.

| Document Revision | Date | Compatible Software | Change History |
| --- | --- | --- | --- |
| 1.14 | 2026-10-21 | Display processing DRM drivers v2.x.x and v3.x.x | Added the following sections about GTM CABC: Section 4.3.3.2, drm_vs_gtm_mode Section 4.3.3.3, drm_vs_gtm_cabc_out Updated the definition and member descriptions of the following structures: Section 4.3.3.1, drm_vs_gtm (added the GTM CABC members and the gtm_mode member) Section 4.3.2.5, drm_vs_ltm_hw_readout (added histogram, wopr_sum and sd_sum) Updated the member descriptions of the LTM structures shared with GTM CABC: "Only used in LTM2.0." was changed to "Used in LTM2.0 and GTM CABC." (drm_vs_ltm, drm_vs_ltm_luma, drm_vs_ltm_grid, drm_vs_ltm_af_slice, drm_vs_ltm_hist_set, drm_vs_ltm_ds, drm_vs_ltm_af_luts, drm_vs_ltm_sre_wopr and drm_vs_ltm_sd). |
| 1.13 | 2026-08-31 | Display processing DRM drivers v2.x.x and v3.x.x | Added the following sections about LTM2.0: Section4.3.2.1.17, drm_vs_ltm_af_lut Section4.3.2.1.18, drm_vs_ltm_sre_wopr Section4.3.2.1.19, drm_vs_ltm_sd Section4.3.2.5, LTM_HW_READOUT Updated the definition and member descriptions of the following structures: Section4.3.2.1.1, drm_vs_ltm Section4.3.2.1.3, drm_vs_ltm_luma Section4.3.2.1.6, drm_vs_ltm_grid Section4.3.2.1.8, drm_vs_ltm_af_slice Section4.3.2.1.14, drm_vs_ltm_hist_set Section4.3.2.1.15, drm_vs_ltm_ds |
| 1.12 | 2026-08-11 | Display processing DRM drivers v2.x.x and v3.x.x | Added Section 4.1.5, Gamma. Added Section 4.5.8.2, IOCTL for Plane Capability Query. Made miscellaneous refinements. |
| 1.11 | 2026-06-30 | Display processing DRM drivers v2.x.x and v3.x.x | Added the following sections: Section 4.5.3, Display Frame Done Query Section 4.5.4.3, IOCTL for Display CRC Result Query Made miscellaneous refinements. |
| 1.10 | 2026-03-05 | Display processing DRM drivers v2.x.x and v3.x.x | Section 4.3.18, On Pixel Sum Updated the drm_vs_ops structure to include drm_vs_ops_roi0 and drm_vs_ops_roi1. Updated the drm_vs_ops_result structure. Added the drm_vs_ops_roi_result structure. Section 4.3.21, Sub-Pixel Rendering - Added the drm_vs_spr structure to include the spr out mode member and added the corresponding member descriptions. Section 4.4.3, Write-Back Frame Difference - Added a new section. |
| 1.09 | 2026-02-05 | Display processing DRM drivers v2.x.x and v3.x.x | Section 3.1.10, drm_vs_hdr_mode - Added a new section. Section 4.1.7.1.1, drm_vs_hdr - Updated the drm_vs_hdr structure to include the hdr_mode member and added the corresponding member descriptions. |
| 1.08 | 2026-01-12 | Display processing DRM drivers v2.x.x and v3.x.x | Section 4.5.7, Palette Added palette LUT support to plane objects for the DC9x00 series. |
| 1.07 | 2025-12-23 | Display processing DRM drivers v2.x.x and v3.x.x | Updated Figure 1. Display Processing DRM Driver Architecture. Updated the framework of the document by moving enumerations and structures used across APIs to Section 3, Universal Enumerations and Structures. Removed the section on Histogram/RGB Histogram V1. Section 4.1.11.3, Fast Clear Modifier Updated the modifier layout for Bit [14:14] and Bit [15:15]. Section 4.1.12.1, DEC400A Modifier Updated the modifier layout for Bit [12:12]. Section 4.1.13.1, Custom Format Modifier Updated the modifier layout for Bit [13:13]. Updated the definition and member descriptions of the following structures: Section 4.1.1.3.1, drm_vs_ex_layer Section 4.1.3.1.1, drm_vs_y2r_config Section 4.1.7.1.1, drm_vs_hdr Section 4.1.7.5.4, drm_vs_lut Section 4.1.9.1.2, drm_vs_alpha_data_extend Section 4.2.4.1.1, drm_vs_blender_dither Section 4.2.4.1.2, drm_vs_random_dither_seed Section 4.3.2.1.1, drm_vs_ltm Section 4.3.2.1.4, drm_vs_ltm_freq_decomp Section 4.3.2.1.12, drm_vs_ltm_dither Section 4.3.2.1.16, drm_vs_ltm_hist_get Section 4.3.2.3.1, drm_vs_ltm_cd_get Section 4.3.2.4.1, drm_vs_ltm_hist_get Section 4.3.3.1.1, drm_vs_gtm Section 4.3.4.2.1, drm_vs_sharpness_csc Section 4.3.5.1.1, drm_vs_sr2000 Section 4.3.5.1.3, drm_vs_nr_config Section 4.3.5.1.4, drm_vs_ssr3_config Section 4.3.5.1.5, drm_vs_y2r_config Section 4.3.8.1.1, drm_vs_brightness Section 4.3.8.1.2, drm_vs_brightness_roi Section 4.3.8.1.3, drm_vs_brightness_mask Section 4.3.13.2.1, drm_vs_data_trunc Section 4.3.14.2.2, drm_vs_random_dither_seed Section 4.3.19.2.2, drm_vs_dbi_correction_coef Section 4.3.22.1.2, drm_vs_hist_bin Section 4.3.22.2.1, drm_vs_histogram_rgb Section 4.3.22.3.1, drm_vs_histogram_result Section 4.3.23.1.1, drm_vs_r2y_config Section 4.3.32.1.1, drm_vs_dp_sync Section 4.3.33.4, IOCTL for GEM Object Query Section 4.4.1.1, WB_POINT Section 4.4.10.1.1, drm_vs_postion Section 4.5.8.1.1, drm_vs_query_feature_cap Section 4.5.11.1.1, drm_vs_query_bld_wb_info Made miscellaneous refinements. |
| 1.06 | 2025-11-14 | Display processing DRM drivers v2.x.x and v3.x.x | Section 4.3.14.1, GAMMA_DITHER, PANEL_DITHER, and DBI_DITHER Refined the description of the table_low[3] and table_high[3] members of the drm_vs_wb_dither structure. Section 4.4.5.1, WB_DITHER Updated the structure name from drm_vs_wb_dither to drm_vs_dither. Refined the description of the table_low[3] and table_high[3] members. Section 4.5.11, Blending Write-Back Information Query Added a new section. |
| 1.05 | 2025-09-29 | Display processing DRM drivers v2.x.x and v3.x.x | Updated the name of the document on the front page and in the header. Removed the section on scaling. Section 4.1.4.1.1, drm_vs_colorkey Added the rect member to the drm_vs_colorkey to specify a rectangular area for applying color keying. Updated the name of the colorkey member to colorkey_low. Updated the data type of colorkey_low and colorkey_high from u32 to struct drm_vs_color. Section 4.1.11.2.1, dc_dec400_fc Added the stride[3] member to the dc_dec400_fc structure. Section 4.3.15, Gamma Removed members of the drm_vs_gamma_lut structure that are specific to DC9000-SR. Updated the data type from u32 to struct drm_vs_color for the following members of the drm_vs_gamma_lut structure: - seg_cnt - seg_point - set_step - entry_cnt Added a data member to the drm_vs_gamma_lut structure. Removed the section on the ROI0_GAMMA and ROI1_GAMMA structure. Section 4.3.22.1.1, drm_vs_histogram Added the hist_mode member to the drm_vs_histogram structure. Added description about the drm_vs_hist_mode enumeration. |
| 1.04 | 2025-03-20 | DC9x00 and DC8x00 series | Added DBI dither in Section 4.3.14.1, GAMMA_DITHER, PANEL_DITHER, and DBI_DITHER. Added the VS_DTH_FRM6 and VS_DTH_FRM10 enumeration values of the drm_vs_dth_frm_mode member. |
| 1.03 | 2025-01-22 | DC9x00 and DC8x00 series | Added Section 4.3.19, Burn-in Compensation. Added Section 4.5.4.1.2, DRM Standard Compliant DC9x00 CRTC CRC Configuration. |
| 1.02 | 2024-12-23 | DC9x00 and DC8x00 series | Replaced the drm_vs_gamut_map structure with the drm_vs_ccm structure and added corresponding descriptions in the following sections: 4.1.7, High Dynamic Range 4.2.2, Non-Linear Gamut Mapping 4.2.6, Gamut Mapping Updated the data type from u32 to u64 for members factor_x and factor_y of the drm_vs_scale_config structure in the following sections: 4.3.1, Scaling 4.4.4, Write-Back Scaler Section 4.3.8, Brightness Compensation Removed the mode member from the drm_vs_brightness structure. Added the roi0, roi1, and mask members to the drm_vs_brightness structure. Removed the BRIGHTNESS_ROI, BRIGHTNESS_MASK, and BRIGHTNESS_DEC properties. Added the following sections: 4.3.10, Color Conversion Matrix Exten 4.3.11, Color Calibration Path Mode 4.3.29, RAM-less 4.4.10, Write-back Stripe Section 4.3.16, Blur Compensation Added the mask_fb_id member to the drm_vs_blur structure. Removed the BLUR_MASK and BLUR_DEC properties. Section 4.3.17, Rounded Corner Display Removed the RCD_ENABLE, RCD_BG, RCD_ROI, RCD_MASK, and RCD_DEC properties. Added the drm_vs_rcd structure. Moved the drm_vs_rcd_bg, drm_vs_rect, and drm_vs_rcd_roi structures under the RCD property. Section 4.4.1, Write-Back Point Added the VS_WB_DISP_DSC_OUT value to the WB_POINT enumeration. Updated the numerical values of the WB_POINT enumeration. Section 4.5.8, Capability Query Removed the following values from the type member of the drm_vs_query_feature_cap structure: VS_FEATURE_CAP_FBC VS_FEATURE_CAP_MAX_BLEND_LAYER VS_FEATURE_CAP_LINEAR_YUV_ROTATION VS_FEATURE_CAP_ANY_RESOLUTION VS_FEATURE_CAP_MAX_WIDTH VS_FEATURE_CAP_MAX_HEIGHT VS_FEATURE_CAP_CONN_POSSIBLE_CRTC |
| 1.01 | 2024-12-04 | DC9x00 and DC8x00 series | Section Histogram/RGB Histogram V1 Renamed this section to distinguish the histogram configurations for the DP path and the histogram configurations for the x86 path described in Section 4.3.22, Histogram/RGB Histogram V2. Added constraints on the enumeration values of the pos member in the drm_vs_hist structure. Added the following sections: 4.3.18, On Pixel Sum 4.3.20, Letter Box Appendant 4.3.22, Histogram/RGB Histogram V2 |
| 1.00 | 2024-11-18 | DC9x00 and DC8x00 series | Section 4.5.8, Capability Query Added the member value to the drm_vs_query_feature_cap structure. Added the following enumeration values to the type member of the drm_vs_query_feature_cap structure: VS_FEATURE_CAP_ANY_RESOLUTION VS_FEATURE_CAP_MAX_WIDTH VS_FEATURE_CAP_MAX_HEIGHT VS_FEATURE_CAP_CONN_POSSIBLE_CRTC VS_FEATURE_CAP_USE_VCMD VS_FEATURE_CAP_VCMD_CMDBUF_REMAINING |
| 0.99 | 2024-11-01 | DC9x00 and DC8x00 series | Section 4.1.7.1, HDR_UNION Added the following members and their descriptions: eotf_enable gamut_map_enable tone_map_enable oetf_enable Added requirements for the following members: eotf_config gamut_map tone map oetf_config Section 4.1.7.4, Gamut Mapping Removed the mode member and related descriptions. Added Section 4.2.2, Non-Linear Gamut Mapping. Section 4.2.6.1, BLD_GAMUT_MAP Removed the mode member and related descriptions. |
| 0.98 | 2024-09-23 | DC9x00 and DC8x00 series | Changed the defined value of VS_SCALE_VERT_COEF_NUM from 43 to 77 in the following sections: 4.3.1, Scaling 4.3.5, SR2000 4.4.4, Write-Back Scaler Section 4.1.6, De-Gamma Correction Changed the category of DEGAMMA_CONFIG from CRTC property to plane property. Section 4.1.11.2, Fast Clear Updated the descriptions of fc_enable in the dc_dec400_fc structure. Added the following sections: 4.1.14, Color Conversion Matrix After De-gamma 4.1.15, Cropping 4.4.9, UV Downsampling |
| 0.97 | 2024-07-01 | DC9x00 and DC8x00 series | Section 4.1.3, YUV-to-RGB Conversion Modified the descriptions of the coef member in the drm_vs_y2r_config structure. Section 4.1.7, High Dynamic Range Moved the sections about the de-multiply, EOTF, gamut mapping, tone mapping, and OETF modules under this section. Added the drm_vs_hdr structure. Section 4.1.9, Data Extending Added the enable member to the drm_vs_alpha_data_extend structure. Section 4.1.11, DEC400 Decompressor Added the fast clear property and fast clear modifier. Section 4.1.12, DEC400A Decompressor Added Bits [12:12] for DEC400A RGB-to-YUV conversion configurations. Modified the code for modifier generation. Added Section 4.1.13, Custom Format and Section 4.3.5, SR2000. Section 4.3.2, Local Tone Mapping Replaced the result member with fd in the drm_vs_ltm_hist_get structure. Section 4.3.15, Gamma Added members specific to DC9000-SR. Miscellaneous refinements. |
| 0.96 | 2024-05-31 | DC9x00 and DC8x00 series | Section 4.1.1, DMA Added the MULTI_EXT_LAYER property. Added Section 4.1.12, DEC400A Decompressor. Section 4.3.2, Local Tone Mapping Replaced the following properties with the LTM property: LTM_ENABLE, LTM_DEGAMMA, LTM_GAMMA, LTM_LUMA, LTM_FREQ_DECOMP, LTM_LUMA_ADJ, LTM_GRID, LTM_AF_FILTER, LTM_AF_SLICE, LTM_AF_TRANS, LTM_TONE_ADJ, LTM_COLOR, LTM_DITHER, LTM_DOWN_SCALER, LTM_LUMA_AVE_SET, LTM_HIST_CD_SET, and LTM_LOCAL_HIST_SET. Added the enable member to all structures except drm_vs_rect and drm_vs_ltm_cd_get. Section 4.3.3, Global Tone Mapping Replaced the following properties with the GTM property: GTM_ENABLE, GTM_DEGAMMA, GTM_GAMMA, GTM_LUMA, GTM_TONE_ADJ, GTM_COLOR, GTM_DITHER, GTM_LUMA_AVE_SET, and GTM_DOWN_SCALER. Sections 4.4.2, 4.4.4, 4.4.5, 4.4.6, and 4.4.7 Changed the category from private CRTC property to private connector property. Added Section 4.4.8, Write-Back Cropping. |
| 0.95 | 2024-04-30 | DC9x00 and DC8x00 series | Section 4.1.11, DEC400 Decompressor Updated the definitions of the following macros that indicate the data alignment modes for DEC400 compression: - DRM_FORMAT_MOD_VS_DEC_ALIGN_32 - DRM_FORMAT_MOD_VS_DEC_ALIGN_64 Added the following macros to indicate the data alignment modes for DEC400 compression: DRM_FORMAT_MOD_VS_DEC_ALIGN_SHIFT DRM_FORMAT_MOD_VS_DEC_ALIGN_MASK Updated the macro for DEC400 modifier generation. Added Section 4.5.6, QoS Management. |
| 0.94 | 2024-04-15 | DC9x00 and DC8x00 series | Section 4.1.11, DEC400 Decompressor Replaced the macro #define DRM_FORMAT_MOD_VS_DEC_TILE_16X8_YUVSP8X8 0x38 with #define DRM_FORMAT_MOD_VS_DEC_TILE_32X4_YUVSP8X8 0x38 to indicate the DEC400 tile type. Added the following macros to indicate DEC400 tile types: - #define DRM_FORMAT_MOD_VS_DEC_TILE_16X8_YUVSP8X8 0x39 - #define DRM_FORMAT_MOD_VS_DEC_TILE_16X4_YUVSP8X8 0x3A Added Section 4.5.10, Write-Back Status Query. |
| 0.93 | 2024-03-29 | DC9x00 and DC8x00 series | Section 4.3.28, Panel Output Splitting Added the SPLITER property. Section 4.4.1, Write-Back Point Exchanged the numerical values of VS_WB_DISP_OUT and VS_WB_DISP_IN for the WB_POINT property. |
| 0.92 | 2024-03-18 | DC9x00 and DC8x00 series | Updated the DEC400 tile type in Section 4.1.11, DEC400 Decompressor. |
| 0.91 | 2024-03-07 | DC9x00 and DC8x00 series | Sections 4.1.9, 4.3.16, 4.3.27, and 4.3.32 Removed the private enable member from the following structures: drm_vs_alpha_data_extend, drm_vs_blur, drm_vs_panel_crop, and drm_vs_dp_sync. Section 4.3.27, Panel Output Cropping Added the panel_src_width and panel_src_height members to the drm_vs_panel_crop structure.4.3.27 Section 4.3.28, Panel Output Splitting Added the src_panel_width0 and src_panel_width1 members to the drm_vs_splice_mode structure. Section 4.3.31, Free Synchronization Changed the data type of the mode member in the drm_vs_free_sync structure from union drm_vs_free_sync_mode to struct drm_vs_free_sync_mode. Added the enumeration value VS_FREE_SYNC_CONFIG_FINISH to the type member in the drm_vs_free_sync structure. Section 4.5.2, Display Stream Compression Added the picture_width and picture_height members to the drm_vs_dsc structure. |
| 0.90 | 2024-02-06 | DC9x00 and DC8x00 series | Chapter 2, Standard Properties Added standard CRTC properties GAMMA_LUT, GAMMA_LUT_SIZE, and CTM. Section 4.1.2, UV Upsampling Removed support of the UP_SAMPLE property for DC9000Nano. Sections 4.1.7.4, 4.3.8, 4.3.16, 4.3.17, 4.4.2, 4.4.4 and 4.4.5 Removed the private enable member from the following structures: drm_vs_scale_config, drm_vs_gamut_map, drm_vs_brightness, drm_vs_dec_enable, drm_vs_scale_config, and drm_vs_wb_dither. Section 4.1.6, De-Gamma Correction Replaced the DEGAMMA_MODE property with DEGAMMA_CONFIG. Section 4.1.7.4, Gamut Mapping Added support of the GAMUT_MAP property for DC8x00 series. Section 4.3.12, 3D LUT Removed the ROI1_3DLUT property. Section 4.3.23, RGB-to-YUV Conversion Added descriptions about the coef member in the drm_vs_r2y_config structure to distinguish DC8x00 series and DC9x00 series. Section 4.5.4, CRC Added support for DC8x00 series and related descriptions about CRTC and Plane CRC configurations. Added descriptions about DC9x00 plane CRC configuration. Section 4.5.5, Test Pattern - Added support for DC8x00 series and related descriptions about test pattern configuration. Section 4.5.8, Capability Query Added the following values to the drm_vs_feature_cap_type enumeration: VS_FEATURE_CAP_CURSOR_WIDTH, VS_FEATURE_CAP_CURSOR_HEIGHT, and VS_FEATURE_CAP_LINEAR_YUV_ROTATION. Added Section 4.5.9, Reset. |
| 0.89 | 2023-11-30 | DC9x00 and DC8x00 series | Sections 4.1.7.3, 4.1.7.6, 4.2.2, and 4.2.5 Updated the structure definition for drm_vs_xstep_lut. Section 4.2.3, OETF Changed the name of OETF to BLD_OETF. Section 4.3.12, 3D LUT Removed the LUT_ROI0_PRIMARY property. Section 4.3.13, Data Conversion Changed the data type of the DATA_EXTEND property from enumeration to blob object and added the drm_vs_data_extend structure. Updated the drm_vs_data_trunc structure for the DATA_TRUNC property. |
| 0.88 | 2023-11-03 | DC9x00 and DC8x00 series | Section 1.1, DRM Driver Overview Updated the DRM driver architecture. Added the following sections: Section 4.1.9, Data Extending Section 4.1.11, DEC400 Decompressor Section 4.3.27, Panel Output Cropping Section 4.3.28, Panel Output Splitting Section 4.3.30, OFIFO Splicing Section 4.3.31, Free Synchronization Section 4.3.32, DP Synchronization |
| 0.87 | 2023-09-11 | DC9x00 and DC8x00 series | Section 4.3.1, Scaling, and Section 4.4.4, Write-Back Scaler: Added members initial_offsetx and initial_offsety for structure drm_vs_scale_config. |
| 0.86 | 2023-08-21 | DC9x00 and DC8x00 series | Added support for DC8x00 series. |
| 0.85 | 2023-08-04 | DC9x00 series | Added the following sections: Section 4.1.4, Color Keying Section 4.1.6, De-Gamma Correction Section 4.3.25, VRR Display Mode Section 4.3.14, Dither Added private property DITHER_ENABLED. Section 4.5.8.1.1, drm_vs_query_feature_cap Added an enumeration value VS_FEATURE_CAP_MAX_BLEND_LAYER for cap. |
| 0.84 | 2023-07-17 | DC9x00 series | Added the following sections: Section 4.1.8, Super Frame Line Padding Section 4.5.8, Capability Query |
| 0.83 | 2023-05-12 | DC9x00 series | Section 4.3.8, Brightness Compensation Added the BRIGHTNESS_ROI, BRIGHTNESS_MASK, and BRIGHTNESS_DEC properties. Updated the drm_vs_brightness structure for the BRIGHTNESS property as follows: - Added the enable member. - Updated the name of the rgb_coef member to luma_coef. Removed the IOCTL for brightness compensation mask configuration. Removed the BRIGHTNESS_ROI0 and BRIGHTNESS_ROI1 properties. Replaced the Blur section with Section 4.3.16, Blur Compensation. Section 4.3.17, Rounded Corner Display Added the RCD_ROI, RCD_MASK, and RCD_DEC properties. Updated the drm_vs_rcd_bg structure for the RCD_BG property. Removed the IOCTL for RCD ROI configuration. Section 4.3.33, PVRIC Decompressor Removed the BLUR_DECOMP, RCD_DECOMP, and BRIGHT_DECOMP properties. Added the PVRIC_CLEAR property. Section 4.4.6, Write-Back RGB-to-YUV Conversion - Added the output_bus_format parameter to the drm_vs_r2y_config structure for the R2Y property. |
| 0.82 | 2023-04-13 | DC9x00 series | Added the following sections: drm_vs_rec rm_vs_scale_confi Section 4.4.7,Write-Back Rotation/Flip Globally corrected the data type of members in drm_vs_color from 32 to u32. |
| 0.81 | 2023-03-06 | DC9x00 series | Added the following sections: Section 4.4.2, Write-Back Splitter Section 4.4.4, Write-Back Scaler Section 4.4.6, Write-Back RGB-to-YUV Conversion Section 4.5.2, Display Stream Compression: Added the VDCM property. |
| 0.80 | 2023-01-20 | DC9x00 series | Added the following sections: Section 4.4.5, Write-Back Dither Section 4.5.1, Side-by-Side Buffer Section 4.5.2, Display Stream Compression Section 4.1.1, DMA: Added the EXT_LAYER_FB property. Section on scaling: Removed the FILTER_COEF property. Added the SCALER property. Section 4.1.7.4, Gamut Mapping and Section 4.2.6, Gamut Mapping: Updated the drm_vs_gamut_map structure for the GAMUT_MAP and BLD_GAMUT_MAP properties as follows: Added the enable member. Added the following values to the drm_vs_gamut_mode enumeration: - VS_GAMUT_2020_TO_DCIP3 - VS_GAMUT_DCIP3_TO_2020 - VS_GAMUT_DCIP3_TO_SRGB - VS_GAMUT_SRGB_TO_DCIP3 Updated value names for the drm_vs_gamut_mode enumeration as follows: - From VS_GAMUT_709_To_2020 to VS_GAMUT_709_TO_2020 - From VS_GAMUT_2020_To_709 to VS_GAMUT_2020_TO_709 Section 4.2.4, Blend Dither: Updated the drm_vs_blender_dither structure for the BLD_DITHER property by removing the keyword DPU from the drm_vs_dth_frm_idx values. Section 4.2.8, Solid Color: Removed the COLOR_CONFIG property. Added the CLEAR property. Section 4.3.14, Dither: Updated the drm_vs_dither structure for properties GAMMA_DITHER and PANEL_DITHER as follows: - Changed the name of the idx_type member to index_type. - Removed the keyword DPU from the drm_vs_dth_frm_idx values. - Added the frm_mode member. Updated the drm_vs_llv_dither structure for the LLV_DITHER property by removing the keyword DPU from the drm_vs_dth_frm_idx values. Miscellaneous minor refinements. |
| 0.71 | 2022-12-30 | DC9x00 series | Added the following sections: Section 4.1.1, DMA Section 4.3.3, Global Tone Mapping Removed the RGB Histogram section. Section 4.3.1, Scaling: Removed properties BLD_SIZE and FILTER_COEF. Added the Scaling property. Section 4.3.2, Local Tone Mapping: Updated the drm_vs_ltm_af_filter structure for the LTM_AF_FILTER property. Histogram section: Started to use the same property for both the histogram and RGB histogram features. Replaced the drm_vs_histogram structure for the HISTOGRAM property with drm_vs_hist, drm_vs_hist_block, and drm_vs_hist_roi. Added properties HIST_GET and RGB_HIST_GET. Section 4.4.1, Write-Back Point: Changed the type of the WB_POINT property from blob object into enumeration. Section 4.5.4, CRC: Added the method of setting the seed for CRC calculation. |
| 0.70 | 2022-11-28 | DC9x00 series | Added the following sections: Section 4.1.7, High Dynamic Range Section 4.3.33, PVRIC Decompressor Section 4.1.7.3, EOTF, Section 4.1.7.6, OETF, Section 4.2.3, OETF, and Section 4.2.5, EOTF Replaced the drm_vs_data_block structure with drm_vs_xstep_lut. Section 4.1.7.5, Tone Mapping Updated the drm_vs_tone_map structure for the TONE_MAP property as follows: Changed the name of the drm_vs_tonemap_y structure to drm_vs_tone_map_y. Changed the type of the lut member from drm_vs_data_block to drm_vs_lut. Section 4.2.4, Blend Dither Replaced the RANDOM_DITHER property with BLD_DITHER. Section 4.3.1, Scaling: Changed the name of the COMPOSE_SIZE property to BLD_SIZE. Section 4.3.2, Local Tone Mapping Added properties for local tone mapping configuration and query. Section 4.3.4, Sharpness Added properties for sharpness configuration. Section 4.3.7, De-Gamma Replaced the drm_vs_data_block structure for the DEGAMMA property with drm_vs_xstep_lut. Section 4.3.8, Brightness Compensation Added an IOCTL for brightness compensation mask configuration. Added properties BRIGHTNESS, BRIGHTNESS_ROI0, and BRIGHTNESS_ROI1. Removed the IOCTL for brightness compensation configuration. Section 4.3.12, 3D LUT Added properties PRIOR_3DLUT, ROI0_3DLUT, and ROI1_3DLUT. Removed the 3DLUT property. Section 4.3.14, Dither Added properties GAMMA_DITHER, PANEL_DITHER, and LLV_DITHER. Removed the DITHER property. Section 4.3.14, Dither Added properties PRIOR_GAMMA, ROI0_GAMMA, and ROI1_GAMMA. Removed the GAMMA property. Section 4.3.16, Blur Added an IOCTL for blur configuration. Section 4.3.17, Rounded Corner Display Added property RCD_ENABLE. Added an IOCTL for RCD ROI configuration. Updated the drm_vs_rcd_bg structure for the RCD_BG property. Removed the IOCTL for RCD configuration. Section 4.3.23, RGB-to-YUV Conversion Updated the drm_vs_r2y_config structure for the R2Y_CONFIG property as follows: Added members mode and gamut. Changed the value of the VS_MAX_R2Y_COEF_NUM macro from 12 to 15. Section 4.3.24, UV Downsampling Updated the drm_vs_down_sample structure for the DOWN_SAMPLE property as follows: Changed the structure name to drm_vs_ds_config. Changed the type of the h_mode and v_mode members from drm_vs_down_sample_mode to drm_vs_ds_mode. |
| 0.60 | 2022-10-17 | DC9x00 series | Added Section 4.1.10, PVRIC Decompressor. |
| 0.50 | 2022-08-12 | DC9x00 series | Draft release. |

- B Conventions

The following conventions apply in VeriSilicon documents:

Main sections are numbered with digits, and appendices are identified with letters.

The word "assert" means to drive a signal true or active.

Signals that are active LOW end with an "n."

Hexadecimal numbers are indicated by prefix "0x" or "n'h", where n specifies the bit width.

- For example, both 0x000032CF and 32'h000032CF indicate a 32-bit hexadecimal value of 13007.

Binary numbers are indicated by prefix "0b" or "n'b", where n specifies the bit width.

- For example, both 0b101 and 3'b101 indicate a 3-bit binary value of 5.

Variables (for example, x and n) are represented in italics when they appear in prose contexts such as body text and document names.

An asterisk (*) matches any sequence of zero or more characters in a filename or path.

Replaceable terms in file paths, commands, and snippets are surrounded with angle brackets < >.

Code snippets and file paths are given in Consolas typeset.

Command line notations adhere to the following conventions:

  - Square brackets [ ] indicate that the enclosed command option is optional.

  - Curly brackets { } indicate that exactly one of the enclosed options (separated by commas) must be chosen.

Interval notation adheres to the following standard mathematical practice:

  - The leftmost value indicates the lower bound and the rightmost value indicates the upper bound.

  - A square bracket [ or ] indicates that the adjacent bound is inclusive.

  - A parenthesis ( or ) indicates that the adjacent bound is exclusive.

Glossary

This section lists terms used in VeriSilicon documentation, together with full names and definitions for those terms.

| Term | Description |
| --- | --- |
| AHB | Advanced High-performance Bus |
| APB | Advanced Peripheral Bus |
| AXI | Advanced eXtensible Interface |
| CRC | cyclic redundancy check |
| DBI | de burn-in |
| DUT | design under test |
| DDR | double data rate |
| DPI | direct programming interface |
| FSDB | fast signal database |
| OFIFO | output first-in first-out |
| QoS | Quality of service |
| RCD | rounded corner display |
| SV | SystemVerilog |
| UVM | Universal Verification Methodology |